| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT3V-A1102 |
| --- | --- |

| Title: | Description of Core Experiment 2: Illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Jaejoon Lee | Email: | jaejoon1.lee@samsung.com |
| Source: | CE2 Coordinator |  |  |

_____________________________

# Abstract

The goal of Core Experiment (CE) 2 is to test illumination compensation for inter-view coding which is applied to 3D video coding both to AVC compatible and HEVC compatible. JCT2-A0015(Samsung), JCT2-A0022(NTT) for AVC based and JCT2-A0086 (LG) for HEVC based were proposed in the 1st JCT-3V meeting. For further study, performance comparison to weighted prediction, CE2 will be continued until 2nd JCT-3V meeting. To find the best solution for illumination compensation, proposed tools will be evaluated in terms of coding efficiency and complexity in the next meeting.

# Participants

| Participant | Contact | Email | Proponent/ Crosschecker |
| --- | --- | --- | --- |
| Samsung | Jaejoon Lee (coordinator) Michael Mishourovsky Alexey Fartukov Igor Kovluga Konstantin Dubkov Ilsoon Lim Kwan-Jung Oh | jaejoon1.lee@samsung.com mike.mishky@samsung.com a.fartukov@samsung.com i.kovliga@samsung.com k.dubkov@samsung.com ilsoon.lim@samsung.com kwanjung.oh@samsung.com | P/C |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp | P/C |
| LG | Hongbin Liu Jiwook Jung Jaewon Sung Jie Jia Sehoon Yea | hongbin.liu@lge.com jiwook.jung@lge.com jw.sung@lge.com jie.jia@lge.com sehoon@lge.com | P/C |
| Qualcomm | Li Zhang Ying Chen | lizhang@qualcomm.com cheny@qualcomm.com | C |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Yu-Wen Huang Shawmin Lei | jl.lin@mediatek.com yiwen.chen@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com | C |

# Tools under Test

The following tools will eb evaluated in the CE

JCT2-A0015 [Samsung] Inter-view coding with adaptive luminance compensation

This technology deals with local estimation of probable luminance discrepancy existing between at least two object projections, captured by multiview system; this estimation is based on an analysis of pixel values of a reference block, so-called template area nearby reference block and finally corresponding template area nearby encoded block. For each unique pixel-value within a reference block an estimation of most probable correction is derived; pixels of a reference block are corrected based on estimations completed before. This method is realized as an auxiliary mode for all partitions starting from P16x16 through to sub-partitions 4x4 for texture P-slices only and successfully works with interview and view synthesis references. This auxiliary mode is signaled by an additional 1 bit-flag right after mb_type at the macroblock level. Adaptive luminance compensation is implemented in the view synthesis mode only.

JCT2-A0022 [NTT] Adaptive illumination compensation

The proposed method compensates illumination differences between prediction picture and coded picture in a localized manner. Although one compensation model is utilized for all blocks, their parameters are decided block by block. For example, a linear function like the weighted prediction is assumed as the compensation function, different scaling factor and offset can be used for each block.

These compensation parameters are derived by minimizing the differences between the prediction picture and decoded picture on the already decoded neighbouring blocks. The prediction picture is generated by using the prediction mode for the coding target block, not the neighbouring block. This minimization problem can be solved by the least square method. The same derivation process can be performed at the decoder side without any additional information, so that no compensation parameter is encoded.

Since the minimization is conducted at the neighbouring pixels, it is not guaranteed that the compensation can always reduce prediction errors on the coding block. This means that it is necessary to make a decision on whether the compensation is applied or not in order to achieve a better coding performance. One of the simple ways is to introduce a flag, but it brings an increase of bitrate and requires syntax changes at the MB level. The illumination compensation is turn on if the reliability of the derived compensation parameters is high. The reliability is expressed by calculating how much the compensation can reduce prediction errors on the neighbouring blocks. Compensation is applied when prediction errors are reduced to less than half of those without the compensation. Benefit of sub-MB skip/direct and of not transmitting MV info and building context are to be evaluated.

JCT2-A0086 [LG] Illumination compensation for inter-view prediction

- Linear IC model is employed to compensate illumination discrepancy between different views. Parameters in IC model are estimated for each PU (prediction unit) using available reconstructed neighbouring pixels and therefore are not need to be encoded. Applying IC or not is decided at CU (coding unit) level, and an IC flag is coded for CU in which inter-view prediction is used. A method is also designed to adaptively determine whether there is illumination discrepancy in a picture, and an IC flag is encoded for the first slice of each picture in dependent views.

### Core Experiment Conditions

#### 3.1 Software

The integrated 3D-AVC and 3D-HEVC-Test Model will be used in the experiment.

#### Test Sequences

Test sequences will be used according to common test conditions.

#### Coding Conditions

- Coding conditions will be used according to common test conditions.

- Performance evaluation compared to weighted prediction will be also conducted.
For this, weighted prediction will be turned on as the second test condition.

#### Evaluation Criteria

- Measure impact on bitrate/PSNR. PSNR shall be calculated for the decoded texture views relative to original texture views and for the synthesized views relative to uncompressed synthesized views. Use 4-point BD-rate according to common test conditions. The 3D-ATM anchor is used as reference for calculation of BD measurements.

- Encoding and decoding run time relative to those of the anchor shall also be reported.

### Timelines

2012/09/27	Make software and test data available for cross-check

2012/10/09	Upload contributions to JCT-3V #2

### References

- A. Fartukov, I. Kovliga, M. Mishurovskiy, K. Dubkov, J. Lee, “3D-CE1.a results on inter-view coding with adaptive luminance compensation by Samsung,” JCT2-A0015, JCT-3V Meeting, Stokholm, Sweden, July 2012

- S. Shimizu, S. Sugimoto, H. Kimata, “CE1.a results on adaptive illumination  compensation,” JCT2-A0022, JCT-3V Meeting, Stokholm, Sweden, July 2012

- H. Liu, J. Jung, J. Sung, J.Jia, S. Yea, “3D-CE1.h related: Illumination Compensation for Inter-View Prediction”, JCT2-A0086, JCT-3V Meeting, Stokholm, Sweden, July 2012