| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT3V-A1103 |
| --- | --- |

| Title: | Description of Core Experiment CE3 on Inter-Component Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 92130 ISSY LES MOULINEAUX France | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange.com |
| Source: | CE coordinators |  |  |

# Introduction

This Core Experiment relates to inter-component prediction for 3D-AVC and 3D-HEVC.

During the 1st JCT-3V meeting, 2 contributions have been proposed taking benefit of inter-component predictions. The goal of this CE is to evaluate them in the context of the 3D-ATM5.0, and 3D-HTM4.0 under the common test conditions.

# Participants List

| Table 1: participant list. Participant | Person, Email address |
| --- | --- |
| ETRI | G. Bang (gbang@etri.re.kr) K. Y. Kim (kimky@khu.ac.kr) |
| Nokia | D. Rusanovskyy (Dmytro.Rusanovskyy@nokia.com) |
| Orange Labs | J. Jung (joelb.jung@orange.com) K. Viswanathan (kartik.viswanathan@orange.com) |
| Qualcomm | Z. Li (Zhang, Li <lizhang@qualcomm.com>) |
| Samsung | K.-J. Oh (kwanjung.oh@samsung.com) J. Lee (jaejoon1.lee@samsung.com) D.-S. Park (dusikpark@samsung.com) J. Y. Lee (jinyoung79.lee@samsung.com) |

# Description of the related contributions

JCT3V-A0042 [Samsung] 3D-AVC results on improved inside view motion prediction

Modifications to the existing IVMP (Inside View Motion Prediction, M22583) tool are proposed to improve coding efficiency. The IVMP availability conditions and motion prediction method are changed. Other parts including syntax are unchanged.

Coding results for these modifications report 0.2% bit rate reduction without any impact on complexity.

The impact on the complexity and the bit rate reduction are to be assessed in 3D-ATM5.0.

More detailed description of the method is required.

JCT3V-A0044 [Orange Labs] Depth Quadtree Prediction for HTM

This contribution presents a modification of the depth quadtree decoding process. For a given CTU, the quadtree of the depth is linked to the collocated CTU quadtree in the texture, in a way that a given CU of the depth cannot be split more than its collocated CU in the texture. The quadtree of the depth is predictively encoded relatively to the quadtree of the texture. A 60% runtime is reported compared to the reference, while BDR gains of 0.5% and 1.0% are observed for synthetized only and coded + synthesized cases respectively, under common test conditions.

The impact on the complexity and the bit rate reduction are to be assessed in 3D-HTM4.0.

More detailed description of the method is required.

# Experiment Descriptions

## Section A (ATM)

To be finalized before final CE description upload (Timeline T1)

| Experiment | Proposed Method | Tester 1 | Tester 2 |
| --- | --- | --- | --- |
| Anchor | OFF |  |  |
| A1 | ON | Samsung | XXX |

Table 2: Summary of the experiments – Section A.

## Section H (HTM)

To be finalized before final CE description upload (Timeline T1)

| Experiment | Proposed Method |  | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- |
|  | Applied on Intra Slices | Applied on Inter Slices |  |  |
| Anchor | OFF | OFF |  |  |
| H1 | ON | ON | Orange Labs | XXX |
| H2 | OFF | ON | Orange Labs | XXX |

Table 3: Summary of the experiments – Section H.

# Test Sequences / Test Points / SW Configuration

Simulation results will be generated for sequences and coding conditions specified in the document JCT3V-A1100. Latest version of the 3D-ATM and 3D HTM, available on April 15th and April 15th respectively will be used for evaluation.

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-Rate).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration)

- Decoding time (compared to the default configuration)

# Time-line

| T1 | End of meeting + 3 weeks | Final CE description and description of proposals |
| --- | --- | --- |
| T2 | Availability of 3D-ATMxx Availability of 3D-HTMxx | Start of tool integration |
| T3 | Oct. 2nd | Completion of tool integration, test configuration announcement Start of testing |
| T4 | Oct. 9th | Testing complete Reporting of all results to CE3 participants complete |
| T5 | Official document submission deadline | Input documents uploaded |
| T6 | T5 + 3 days | CE3 summary report uploaded |

Table 4: time-line