| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT3V-A1103 |
| --- | --- |

| Title: | Description of Core Experiment 3 on Inter-Component Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 92130 ISSY LES MOULINEAUX France | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange.com |
| Source: | CE coordinators |  |  |

# Introduction

This Core Experiment relates to inter-component prediction for 3D-AVC and 3D-HEVC.

During the 1st JCT-3V meeting, two contributions have been proposed taking benefit of inter-component predictions. The goal of this CE is to evaluate them in the context of the 3D-ATM5.0, and 3D-HTM4.0 under the common test conditions.

# Participants List

| Participant | Person, Email address | Status |
| --- | --- | --- |
| ETRI | G. Bang (gbang@etri.re.kr) K. Y. Kim (kimky@khu.ac.kr) | - |
| Intel | W. Zhang (wenhao.zhang@intel.com) Y.-J. Chiu (yi-jen.chiu@intel.com) L. Xu (lidong.xu@intel.com) Z. Deng (zhipin.deng@intel.com) | C |
| LG | J. Heo, (jin78.heo@lge.com) S. Yea (sehoon.yea@lge.com) | C |
| Nokia | D. Rusanovskyy (Dmytro.Rusanovskyy@nokia.com) | - |
| Orange Labs | J. Jung (joelb.jung@orange.com) K. Viswanathan (kartik.viswanathan@orange.com) | P |
| Panasonic | S. Kamp (Steffen.Kamp@eu.panasonic.com) | C |
| Peking University | S. Wang (sqwang@jdl.ac.cn) | - |
| Qualcomm | L.Zhang (Zhang, Li <lizhang@qualcomm.com>) | - |
| Samsung | K.-J. Oh (kwanjung.oh@samsung.com) J. Lee (jaejoon1.lee@samsung.com) J. Y. Lee (jinyoung79.lee@samsung.com) | P |

Table 1: participant list.

# Description of the related contributions

JCT3V-A0042 [Samsung] 3D-AVC results on improved inside view motion prediction

Current design includes IVMP (Inside View Motion Prediction - m22583). In IVMP, the motion information, including mb_type, sub_mb_type, reference indices and motion vectors of the co-located MB in texture view component is inherited by the depth view component of the same view. IVMP is enabled when all of the corresponding four texture macroblocks are coded as skip or inter 16x16 or inter 16x8 or inter 8x16.

Modifications to the existing IVMP tool are proposed to improve coding efficiency. The IVMP availability conditions and motion prediction method are changed: it is proposed to enable IVMP when

at least one corresponding texture macroblock is coded as inter mode including P8x8 mode.
 In other words, IVMP is always enabled except when all corresponding four texture macroblocks are intra coded.

For the intra coded texture macroblock, motion information of inter coded neighboring macroblock is used. 
The macroblock which has more complex mb_type is selected when the multiple inter coded neighboring macroblocks are available. 
If the available neighboring macroblocks have same mb_type, the priority order is as follows: vertical, horizontal, diagonal.

The modified motion prediction uses 8x8 block size regardless of mb_type of corresponding texture. 
Instead, process that finds the reliable motion vector is added. 
The motion information of the 4x4 block that has motion vector of maximum magnitude is selected for motion prediction.

The impact on the complexity and the bit rate reduction are to be assessed in 3D-ATM5.0.

JCT3V-A0044 [Orange Labs] Depth Quadtree Prediction for HTM

This contribution presents a modification of the depth quadtree decoding process. For a given CTU, the quadtree of the depth is linked to the collocated CTU quadtree in the texture, in a way that a given CU of the depth cannot be split more than its collocated CU in the texture. The quadtree of the depth is predictively encoded relatively to the quadtree of the texture.

The proposed method applies the convention from Table 2 to predict depth partitioning from texture, and provide the residual depth partitioning. The basic idea is to send nothing if the CU of the texture is not further split, and send one bit to indicate if depth is split or not when the current partition of the texture is further split.

The proposed method applies the convention from Table 3 to do the inverse prediction from residual depth partitioning, and provide the depth partitioning. The basic idea is that when the texture is split, the residual depth is read to know if it is split too, or not.

| Texture QT; Depth QT; Residual Depth QT. 1; 1; 1. 0; 0; -. 1; 0; 0. Table 2: prediction of the depth. | Texture QT; Residual Depth QT; Depth QT. 1; 1; 1. 0; *; 0. 1; 0; 0. Table 3: inverse prediction of the residual depth. |
| --- | --- |

Following the rules established in Table 3, during the parsing of the depth, a ‘residual depth quadtree’ is read, and inverse predicted to provide the depth quadtree. When a ‘1’ is found in the texture, one bit is read in the residual depth QT to understand if the depth is split or not. When a ‘0’ is found in the texture, the depth is not parsed.

The impact on the complexity and the bit rate reduction are to be assessed in 3D-HTM4.0.

# Experiments Description

## Section A (ATM)

| Experiment | Proposed Method | Tester 1 | Tester 2 |
| --- | --- | --- | --- |
| Anchor | OFF |  |  |
| A1 | ON | Samsung | XXX |

Table 4: Summary of the experiments – Section A.

## Section H (HTM)

| Experiment | Proposed Method |  | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- |
|  | Applied on Intra Slices | Applied on Inter Slices |  |  |
| Anchor | OFF | OFF |  |  |
| H1 | ON | ON | Orange Labs | Intel |
| H2 | OFF | ON | Orange Labs | XXX |

Table 5: Summary of the experiments – Section H.

# Test Sequences / Test Points / SW Configuration

Simulation results will be generated for sequences and coding conditions specified in the document JCT3V-A1100. Latest version of the 3D-ATM and 3D HTM, available on 3 weeks after the 1st JCT-3V meeting respectively will be used for evaluation.

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-Rate).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration)

- Decoding time (compared to the default configuration)

# Time-line

| T1 | End of meeting + 3 weeks Aug. 10th | Final description of proposals sent to the coordinators |
| --- | --- | --- |
| T2 | Aug. 17th | Upload of Final CE description document |
| T3 | Availability of 3D-ATM5.0 Availability of 3D-HTM4.0 Aug. 10th | Start of tool integration |
| T4 | Sept. 28th | Completion of tool integration, test configuration announcement Start of testing |
| T5 | Oct. 8th | Testing complete Reporting of all results to CE3 participants complete |
| T6 | Official document submission deadline Oct. 9th | Input documents uploaded |
| T7 | T6 + 3 days Oct. 12th | CE3 summary report uploaded |

Table 6: Time-line.