| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0004 |
| --- | --- |

| Title: | AHG4: 3D-AVC Software Integration (AHG4) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy (Nokia), J.Y. Lee (Samsung), J.Lin (Mediatek), D. Tian (MERL), O.Stankiewicz (Poznan) | Email: | Dmytro.Rusanovskyy@nokia.com jinyoung79.lee@samsung.com jl.lin@mediatek.com tian@merl.com ostank@multimedia.edu.pl |

_____________________________

# Abstract

This document is a report on activities performed by AHG4 between 1st and 2nd JCT-3V meetings. Additionally, this document provides recommendation on further development of 3DV-ATM and integration procedure.

# Introduction

At the 1st JCT-3V meeting, the following mandates have been identified for AHG4:

- Coordinate development of the 3DV-ATM software and its distribution to JCT-3V members

- Produce documentation of software usage for distribution with the software

- Prepare and deliver 3DV-ATM v5.0 software version and the reference configuration encodings according to JCT3V-A1100 based on common conditions suitable for use in most core experiments (expected within 3 weeks after the meeting).

- Prepare and deliver 3DV-ATM v5.1 software version with remaining non-CTC adoptions (expected within 2 weeks after v5.0).

- Perform analysis and reconfirmation checks of the behavior of technical changes adopted into the draft design, and report the results of such analysis.

- Suggest configuration files for additional testing of tools.

- Coordinate with MVC+D / 3D-AVC Draft and Test Model editing (AHG2) to identify any mismatches between software and text.

- Prepare reference software related to JCT3V-A1001.

# Report

Mandates 1-4:

AhG4 would like to report that mandates 1-4 have been successfully accomplished.

The reference 3DV-ATM software version 5.0 has been made available according to plan within 3 weeks after the 1st JCT-3V meeting. The reference software version 5.1 was released with insignificant delay related to a bug fixing procedure and mismatch in cross-check results. It is understood, that resulting delay did not introduce major disturbances to the development process.

The most recent version of the 3DV-ATM software and related documentation is located in a public domain in the predefined location.

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/tags/3DV-ATMv5.1r2/

As a conclusion on mandates 1-4: AhG4 would like to recommend usage of the most recent versions (e.g. v5.1 instead of 5.0) of the reference software as a code base for any further development and results comparisons.

Mandates 5:

AhG4 has performed an analysis and reconfirmation check for tools adopted during the 1st JCT-3V meeting. Simulation results produced with 3DV-ATM v5.1r2 has been compared against the previous version 4.0. Summary of the results is given in Table 1.

Table 1. Comparative results of 3DV-ATM v5.1r2 in the EHP configuration

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | -10,94 | 0,37 | -0,84 | 0,05 | -10,23 | 0,35 | -8,55 | 0,29 |
| S02 | -1,82 | 0,05 | -0,55 | 0,03 | -1,75 | 0,05 | -3,52 | 0,11 |
| S03 | -3,94 | 0,14 | -0,31 | 0,02 | -3,75 | 0,13 | -21,83 | 0,77 |
| S04 | -2,43 | 0,09 | 1,38 | -0,08 | -2,31 | 0,08 | -9,97 | 0,37 |
| S05 | -10,46 | 0,53 | -0,38 | 0,02 | -9,27 | 0,44 | -7,89 | 0,35 |
| S06 | -13,98 | 0,73 | -0,58 | 0,03 | -12,75 | 0,65 | -10,59 | 0,50 |
| S08 | -5,90 | 0,25 | -0,47 | 0,02 | -5,14 | 0,22 | -5,56 | 0,20 |
| Average | -7,07 | 0,31 | -0,25 | 0,01 | -6,46 | 0,27 | -9,70 | 0,37 |

Additionally, as a result of collaboration activity with AhG9, comparative analysis of EHP vs HP coding configuration has been performed. Summary of results produced under modified CTC (QP settings with equal bitrate for EHP and HP) is given in Table 2.

Table 2. Summary of simulation results produced by HP with given QP of Table 1 compared against the EHP, 3DV-ATM v5.1r2.

|  | Texture Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | 38.45 | -1.40 | 42.35 | -1.47 | 41.45 | -1.44 |
| S02 | 12.83 | -0.40 | 16.10 | -0.49 | 15.36 | -0.49 |
| S03 | 22.55 | -0.73 | 20.68 | -0.70 | 23.68 | -0.71 |
| S04 | 29.57 | -1.06 | 31.57 | -1.13 | 27.49 | -0.96 |
| S05 | 27.35 | -1.41 | 40.15 | -1.77 | 37.76 | -1.60 |
| S06 | 34.12 | -1.56 | 40.77 | -1.74 | 36.46 | -1.52 |
| S08 | 15.23 | -0.64 | 20.39 | -0.80 | 21.21 | -0.73 |
| Average | 25.73 | -1.03 | 30.29 | -1.16 | 29.06 | -1.06 |

As a conclusion on mandates 5: AhG4 has identified the following problems with current version of 3DV-ATM and recommends their fastest resolving:

- 3DV-ATM v5.1 fails to encode MVD data with the full resolution depth map.

Full resolution depth map is not a part of CTC, but it is a part of specification and TM shall provide it. It is recommended to localize the problem and provide a bug-fix.

- Bugs in Weighted Prediction of 3DV-ATM have been reported.

WP is not a part of CTC, however it is related to an active topic that undergoes the development in CE3. It is recommended to localize the problem and provide a bug-fix.

- AhG4 recommends to start development of 3DV-ATM v6 with possible bug-fixing for problems mentioned above.

Mandates 6:

No changes to CTC configurations are currently required. AhG4 recommends learning the outcome of AhG9 results and adjusting CTC accordingly.

Mandates 7-8:

AhG4 would like inform that action points related to Mandates 7-8 has been handled in the working mode. AhG4 would like to recommend a decision on reference test model selection for MVC+D specification, as this would have a direct impact on the following integration plan.