| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0005 |
| --- | --- |

| Title: | JCT-3V AHG report: 3D-HEVC Software Integration (AHG5) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-3V |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Gerhard Tech (Fraunhofer HHI) Li Zhang (Qualcomm) YuLin Chang (Mediatek) Krzysztof Wegner (Poznan Univ. of Tech.) | Email: | gerhard.tech@hhi.fraunhofer.de lizhang@qualcomm.com yulin.chang@mediatek.com kwegner@multimedia.edu.pl |
| Source: | AHG |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AhG on 3D-HEVC Software Integration that have taken place between the 1st JCT-3V meeting in Stockholm (16–20 July 2012) and the 2nd JCT-3V meeting in Shanghai (13-19 October 2012). Activities focused on the integration of tools adopted at the 1st meeting into a common code base.

# Mandates

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| 3D-HEVC Software Integration (AHG5) (jct-3v@lists.rwth-aachen.de) - Coordinate development of the HTM software and its distribution to JCT-3V members - Produce documentation of software usage for distribution with the software - Prepare and deliver HTM 4.0 software version and the reference configuration encodings according to JCT3V-A1100 based on common conditions suitable for use in most core experiments (expected within 3 weeks after the meeting). - Prepare and deliver HTM 4.1 software that include additional items not integrated into the 4.0 version (expected within 2 weeks after the 4.0 software release). - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with MV-HEVC Draft and 3D-HEVC Test Model editing AhG to identify any mismatches between software and text. | G. Tech L. Zhang (co-chairs) Y. Chang K. Wegner (vice chairs) | N |

# Method of working

Development of the software was co-ordinated with the parties needing to integrate changes. A single track of development was pursued. The distribution of the software was announced on the JCT-3V email reflector and the software was made available through the SVN server:

# https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/

# Anchor bitstreams have been created and uploaded to:

ftp.hhi.fraunhofer.de; login: mpeg3dv_guest; path: /MPEG-3DV/HTM-Anchors/

# History of revisions

Multiple versions of the HTM software were produced and announced on the JCT-3V email reflector. The following sections give a brief summary of the changes made for each version and achieved coding gains.

## Version HTM-4.0

Normative Tools:

- (JCT3V-A0049) Inter-view motion prediction modification

- (JCT3V-A0126) Simplified disparity derivation

- (JCT3V-A0097) Disparity vector generation

- (JCT3V-A0070) Region boundary chain coding for depth maps

Non-Normative Tools:

- (JCT3V-A0033) VSO model based estimation

- (JCT3V-A0093) VSO early skip

- (JCT3V-A0087) RDO selection between Non-Zero Residual and All-Zero Residual Intra

Coding performance relevant bug fixes:

- (JCT3V-A0091) CABAC init flag fix (minor)

HTM-4.0 vs. HTM-3.1

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.6% | -2.0% | -0.7% | -3.1% | -2.8% | 76.4% | 98.2% | 99.0% |
| Kendo | 0.0% | -1.6% | -2.4% | -0.8% | -2.5% | -2.5% | 75.9% | 94.5% | 102.2% |
| Newspapercc | 0.0% | -0.7% | -0.7% | -0.3% | -4.6% | -4.0% | 75.4% | 103.8% | 100.9% |
| GhostTownFly | 0.1% | -0.1% | 4.9% | 0.7% | -3.4% | -2.6% | 77.5% | 95.4% | 100.5% |
| PoznanHall2 | 0.0% | -1.1% | 1.0% | 0.0% | -1.7% | -1.7% | 76.2% | 92.8% | 98.0% |
| PoznanStreet | 0.0% | -0.3% | 0.5% | 0.1% | -1.6% | -1.5% | 74.4% | 91.5% | 97.4% |
| UndoDancer | 0.0% | -1.1% | 1.2% | 0.1% | -4.6% | -3.3% | 76.0% | 94.7% | 98.0% |
| 1024x768 | 0.0% | -1.3% | -1.7% | -0.6% | -3.4% | -3.1% | 75.9% | 98.8% | 100.7% |
| 1920x1088 | 0.0% | -0.7% | 1.9% | 0.2% | -2.8% | -2.3% | 76.0% | 93.6% | 98.4% |
| average | 0.0% | -0.9% | 0.3% | -0.1% | -3.1% | -2.6% | 76.0% | 95.8% | 99.4% |

## Version HTM-4.0.1

Coding performance relevant bug fixes:

- Fix to Restricted Residual prediction (Minor)

- Fix to RDO mode selection regarding DMMs

- Fix signaling of deblocking filter disabling for depth

HTM-4.0.1 vs. HTM-4.0 (CTC, three view configuration)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | -0.1% | 100.2% | 98.4% | 102.8% |
| Kendo | 0.0% | 0.1% | 0.0% | 0.0% | 0.3% | 0.3% | 100.0% | 97.6% | 101.8% |
| Newspapercc | 0.0% | 0.1% | 0.0% | 0.0% | -1.4% | -0.8% | 100.1% | 100.7% | 101.2% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -0.6% | -0.4% | 99.6% | 94.1% | 98.5% |
| PoznanHall2 | 0.0% | -0.4% | 0.0% | -0.1% | -1.5% | -1.0% | 100.1% | 96.6% | 99.3% |
| PoznanStreet | 0.0% | 0.3% | 0.1% | 0.1% | -0.2% | -0.1% | 100.1% | 88.9% | 101.7% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -3.7% | -2.6% | 99.6% | 93.3% | 96.2% |
| 1024x768 | 0.0% | 0.1% | 0.0% | 0.0% | -0.4% | -0.2% | 100.1% | 98.9% | 102.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -1.5% | -1.0% | 99.8% | 93.2% | 98.9% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -1.1% | -0.7% | 99.9% | 95.6% | 100.2% |

## Version HTM-4.1

Non-Normative Tools (off by default):

- (JCT3V-A0119) VSO depth fidelity

- (JCT3V-A0044) Depth Quadtree restriction

HTM-4.1 vs. HTM-4.0.1 (CTC, three view configuration)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 100.5% | 100.1% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 103.0% | 100.4% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 99.8% | 101.6% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 103.4% | 102.3% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 104.2% | 101.6% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 110.3% | 102.8% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.4% | 108.3% | 104.5% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 101.1% | 100.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 106.5% | 102.8% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 104.1% | 101.9% |

Bitstreams are identical to HTM-4.0.1 bitstreams.

# Suggestions to improve the software integration process

For improvement of the software integration process this section lists recommendations that have not been regarded by all integrators when submitting software to the AHG. AHG chairs have partly fixed arising issues. For next integrations to the HTM software it is recommended that providers of software take especially care that

- Software works reasonably for non-CTC conditions (e.g. different number of views, different temporal/inter-view prediction structures)

- Software matches with suggested draft text also for non-CTC conditions.

- New tools are switchable by encoder configuration.

- Each new tool or bug fix is submitted by one commit to the SVN.

- For cross checks coded camera parameters and bit rates from file size are used.

- No tabs are included in the source code (Now disallowed by svn-system).

- Function naming is reasonable (e.g. not getDispVector2 )

# Open issues

- Since the MV-HEVC working draft [1]and the 3D-HEVC Test model Annex G [2] are based on HEVC text specification draft 8 [3] and HTM 4.2 is based on HM-6.1 there are mismatches between draft texts and HTM-software.

- With the integration of tools related to inter-view motion prediction (JCT3V-A0126, JCT3V-A0049, JCT3V-A0097) the issue arose that inter-view motion prediction only works properly under CTC. Moreover there are mismatches with the provided text.

# Related input documents

| JCT3V-B0046 | AHG5: MV-HEVC software for HTM | L. Zhang, Y. Chen, J. Kang (Qualcomm) |
| --- | --- | --- |
| JCT3V-B0179 | AHG5: Cross check of JCT3V-B0046 MV-HEVC software for HTM | G. Tech (HHI) |
| JCT3V-B0181 | AHG5: Cross check of MV-HEVC software for HTM | K. Wegner, O. Stankiewicz (Poznan Uni- versity of Technology) |

In JCT3V-B0046, a new software branch for MV-HEVC is proposed. It is reported that the new software is aligned with the MV-HEVC working draft and can be configured as part of the HTM software.

# Recommendations

The recommendations of the 3D-HEVC Software integration group are:

- Continue to develop reference software based on HTM version 4.1 and improve its quality

- Remove bug fix related macros introduced in previous HTM versions before starting integration towards HM 5.0.

- Continue to identify bugs and discrepancies with text, and address them.

- Discuss on how to address open issues.

- Discuss AHG5 related input documents.

# References

- JCT-3V, “MV-HEVC Working Draft 1”, JCT3V-A1004, JCT-3V Meeting, Stockholm, July 2012

- JCT-3V, “3D-HEVC Test Model 1”, JCT3V-A1005, JCT-3V Meeting, Stockholm, July 2012

- JCT-VC, “High Efficiency Video Coding (HEVC) text specification draft 8”, JCTVC-J1003, JCT-VC Meeting, Stockholm, July 2012