| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0009 |
| --- | --- |

| Title: | JCT-3V AHG Report: 3D Quality Assessment (AHG9) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Touradj Ebrahimi, EPFL Anthony Vetro, Mitsubishi Electric Vittorio Baroncini, FUB | Email: | touradj.ebrahimi@epfl.ch avetro@merl.com vittorio@fub.it |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document summarizes the AHG activity on 3D Quality Assessment.

# Mandates

- Study objective quality assessment metrics that would provide an accurate evaluation of synthesized views

- Provide software e.g. for computing such quality metrics that could be used for reporting on the evaluation of coding and rendering tools

- Recommend improvements to evaluation methodology

The email reflector for AHG9 is jct-3v@lists.rwth-aachen.de.

# Activity

Mandate 1: Study objective quality assessment metrics that would provide an accurate evaluation of synthesized views

The AHG started its work by focusing on the four objective quality assessment metrics that have the highest correlation with the subjective evaluation scores, as reported in JCT3V-A0150, namely VIFp, VQM, MS-SSIM, and SSIM.

Then, as the basis for our initial study, it was proposed that we perform simulations comparing the following anchor configurations:

- 3D-ATM v5.0 with inter-view prediction vs all tools enabled

- 3D-HTM v4.0 with inter-view prediction vs all tools enabled

Based on the decoded and synthesized videos, subjective/expert viewing will be conducted during the next JCT-3V meeting. From these viewing sessions, the subjective scores would then need to be correlated with the objective metrics to determine the correlation.

HHI, Mitsubishi, Nokia and Qualcomm agreed to bring the above viewing material to the 2nd JCT-3V meeting in Shanghai.

The following documents related to AHG have been registered:

- JCT3V-B0101: AHG9: AVC anchor material for quality assessment (Qualcomm/Mitsubishi)

- JCT3V-B0180: AHG9: Accuracy of 2D metrics for 3D video quality assessment (EPFL)

- JCT3V-B0140: Comments on 3D Video Quality Assessment in JCT-3V (LG/Yonsei)

- JCT3V-B0145: HTM 4.0 Coding Performance Evaluation (LG)

Mandate 2: Provide software e.g. for computing such quality metrics that could be used for reporting on the evaluation of coding and rendering tools

The following software packages were distributed to the email reflector on September 5, 2012:

- VQM can be obtained from the Institute for Telecommunication Sciences (ITS) website: http://vqm.its.bldrdoc.gov/. There are several versions of VQM. The Command Line Video Quality Metric (CVQM) software was suggested.

- VIFp, SSIM, and MS-SSIM can be computed using the Video Quality Measurement Tool made available on the EPFL web site: http://mmspg.epfl.ch/vqmt/

Additional information regarding VQM was also provided:

The CVQM software was developed using MATLAB and its associated toolboxes. It is therefore necessary to install the MATLAB Component Runtime (MCR) library before running CVQM.

VQM requires uncompressed AVI files in the UYVY color space. So, you need to convert your raw YUV 4:2:0 video, for example by using ffmpeg: ffmpeg -vcodec rawvideo -pix_fmt yuv420p -s wxh -i video.yuv -vcodec rawvideo -pix_fmt uyvy422 -r fps video.avi, where w, h, and fps are the width, height, and frame rate of the video.

Several models can be used. It was suggested to use the NTIA General Model. Example of usage: cvqm.exe 'original.avi' 'processed.avi' 'progressive' 'none' 'general'

Model values will be written to file 'processed.avi_model.txt' in this case, as shown in the example below:

0.749233 general

0.104844253244262 si_loss

0.382926021155058 hv_loss

0.242507602177597 hv_gain

0.030857768486401 color1

-0.016286893843846 si_gain

0.002156662476364 contati

0.002227752612876 color2

The first value (0.749233 in this case) corresponds to the NTIA General Model. A value of 0 means that the original and processed videos are identical (best quality), while a value of 1 means the worst quality.

Mandate 3: Recommend improvements to evaluation methodology

The AHG does not have any recommendations at this time on changes to the evaluation methodology, but would certainly consider this based on the experiments. However, it is encouraged that CE participants consider using these alternative metrics as part of their work.

# Recommendations

The AHG on 3D Quality Assessment recommends to:

- Conduct the viewing of anchor material for both AVC and HEVC to collect subjective scores

- Analyze the objective metrics to determine the correlation with the subjective scores

- Consider changes to the evaluation methodology in the common test conditions

- Re-establish the AHG for further study and experiments in this area