| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Shanghai, CN, 13–19 October 2012 | Document: JCT3V-B0010 |
| --- | --- |

| Title: | AHG 10 Report on Inter Component Dependencies |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | J. Jung J.-L. Lin (co-chairs) | Email: | joelb.jung@orange.com jl.lin@mediatek.com |
| Source: | Coordinators |  |  |

_____________________________

# Abstract

This report summarizes the activities of the Inter Component Dependencies ad hoc group (AHG10) between the 1st JCT-3V meeting and the 2nd JCT-3V meeting, and lists the related input documents submitted to this meeting according to their topics.

# Mandates

- Identify current inter component dependencies in the 3D-ATM and 3D-HTM softwares

- Evaluate the impact of these dependencies on the current design

# Reflector discussions

There were some discussions on the following topics:

- xxx

# Related contributions

The list of AHG10 related documents is as follows:

| JCT3V-B0023 | A+H | 3D-CE3: Summary report of Core Experiment 3 on Inter-Component Prediction | CE coordinator |
| --- | --- | --- | --- |
| JCT3V-B0057 | A | 3D-CE3.a related: On inside-view motion prediction for 3D-AVC | Qualcomm |
| JCT3V-B0065 | H | 3D-CE3.h related: Motion Parameter Inheritance Improvement | Intel |
| JCT3V-B0068 | H | 3D-CE3.h: Depth Quadtree Prediction for 3DHTM 4.1 | Orange Labs |
| JCT3V-B0083 | H | 3D-CE3.h related: Unconstrained motion parameter inheritance in 3D video coding | Qualcomm |
| JCT3V-B0084 | H | 3D-CE3.h related: Simplification on motion parameter inheritance | MediaTek |
| JCT3V-B0085 | H | 3D-CE3.h related: Removal of picture buffers for motion parameter inheritance | MediaTek |
| JCT3V-B0146 | A | 3D-CE3.a results on improved inside view motion prediction | Samsung |
| JCT3V-B0156 | H | 3D-CE3.h related results on improved motion parameter inheritance | Samsung |
| JCT3V-B0184 | A | 3D-CE3.a: Crosscheck of Samsung JCT3V-B0146 on improved inside view motion prediction | Xx |
| JCT3V-B0205 | H | 3D-CE3.h related: Cross check of JCT3V-B156 on Improved Motion Parameter Inheritance | Orange Labs |

Cross-checks:

| JCT3V-B0066 | H | 3D-CE3.h: Cross-check of JCT3V-B0068 on Depth Quadtree Prediction for 3DHTM 4.1 | Intel |
| --- | --- | --- | --- |
| JCT3V-B0143 | H | 3D-CE3.h: Cross-check of Depth Quadtree Prediction for 3DHTM 4.1 (JCT3V-B0068) | Panasonic |
| JCT3V-B0166 | H | 3D-CE3.h related: Cross check of JCT3V-B0083 on unconstrained motion parameter inheritance | Qualcomm |
| JCT3V-B0169 | A | 3D-CE3.a cross check on inside-view motion prediction of Qualcomm (JCT3V-B0057) | Samsung |
| JCT3V-B0170 | H | 3D-CE3.h cross check on depth quadtree prediction of Orange Labs (JCT3V-B0068) | Samsung |

# 3D-ATM Study

## Identify tools with inter component dependencies and impact on the design (such as additional buffers, delay, parsing changes, etc)

Tool 1: Inside View Motion Prediction – IMVP

Description:

In IVMP, the motion information, including mb_type, sub_mb_type, reference indices and motion vectors of the co-located MB in texture view component is inherited by the depth view component of the same view. IVMP is enabled when all of the corresponding four texture macroblocks are coded as skip or inter 16x16 or inter 16x8 or inter 8x16.

Impact:

xx

Tool 2: Direction-separate Motion Vector Prediction in Inter Mode

Description:

Direction-separate motion vector prediction is applied in Inter mode.

If the target reference picture is a temporal prediction picture, the motion vector prediction is derived as the median of the motion vectors of the adjacent blocks A, B, and C.

If the target reference picture is an inter-view prediction picture, the inter-view motion vectors of the neighboring blocks are employed for the inter-view prediction. If an inter-view motion vector is unavailable, a disparity vector which is derived from the maximum depth value of four corner depth samples within the associated depth block is used. The motion vector predictor is then derived as the median of the inter-view motion vector of the adjacent blocks A, B, and C.

Impact:

Xx

Tool 3: Inter-view Motion Vector Prediction in Skip/Direct Mode

Description:

In Skip and Direct mode, a MVP candidate is derived based on a predefined derivation order: inter-view candidate and the three spatial candidates derived from the neighboring blocks A, B, and C. The central point of the current block in the dependent view and its disparity vector are used to find the corresponding point in the base view. After that, the MV of the corresponding block is used as the inter-view candidate of the current block. The disparity vector (DV) can be derived from the neighboring blocks or the depth value of the central point. Specifically, if one of the neighboring blocks has a DV, the DV is used as the disparity. Otherwise, the depth-based disparity vector which is derived using the depth value of the central point and camera parameters is used.

Impact:

xx

## Total improvement provided by this package of tools

Not evaluated yet. Tool 2 cannot be disabled by a macro.

# 3D-HTM Study

## Identify tools with inter component dependencies and impact on the design (such as additional buffers, delay, parsing changes, etc)

Tool 1: Intra DMM Mode 3: Inter-component prediction of Wedgelet partitions

Description:

The basic principle of this mode is to predict the Wedgelet partition from a texture reference block, namely the co-located block of the associated video picture.

Impact:

xx

Tool 2: Intra DMM Mode 4: Inter-component prediction of Contour partitions

Description:

The basic principle of this mode is to predict a Contour partition from a texture reference block by inter-component prediction. Like for the inter-component prediction of a Wedgelet partition pattern, the reconstructed luminance signal of the co-located block of the associated video picture is used as a reference,

Impact:

xx

Tool 3: Motion Parameter Inheritance

Description:

This mode allows inheritance of the treeblock subdivision into CUs and PUs and their corresponding motion parameters from the video signal. If the MPI mode is indicated at a higher level of the depth map coding tree, corresponding to a CU size that is larger than the CU size that is used for the video signal, the CU/PU subdivision, together with the corresponding motion data, is inherited from the video signal.

Impact:

xx

Tool 4: QuadTree limitation

This non normative tool is in the 3D-HTM, yet disabled by default in the common test conditions.

Description:

3D-HTM splits a given CTB into smaller CUs, based on RD optimized decisions. A corresponding quadtree (QT) is obtained for the texture, and another one for the depth. This tool prevents the encoder from making full investigation of every possible QT configuration for the depth. The tool forces the encoder to limit the partitioning of the depth at the same level as the partitioning of the texture. For a given CTU, the quadtree of the depth is linked to the collocated CTB quadtree in the texture, so that a given CU of the depth cannot be split more than its collocated CU in the texture.

Impact:

xx

## Total improvement provided by this package of tools

Reference: 3D-HTM anchor

Tested: reference with Tool 1, Tool 2, Tool 3 disabled.

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 0,0% | -1,4% | -1,1% | 105,0% | 115,4% | 102,5% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,0% | -3,2% | -2,6% | 105,2% | 106,5% | 101,2% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 0,0% | -2,1% | -1,8% | 106,7% | 109,4% | 101,2% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,0% | -1,2% | -0,8% | 106,4% | 107,8% | 100,7% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,0% | -1,8% | -1,2% | 105,2% | 109,2% | 102,6% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,0% | -1,1% | -0,8% | 105,6% | 103,7% | 106,2% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,0% | -1,9% | -1,6% | 105,5% | 99,7% | 103,9% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,0% | -2,2% | -1,8% | 105,6% | 110,4% | 101,6% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,0% | -1,5% | -1,1% | 105,7% | 105,1% | 103,4% |
| Average | 0,0% | 0,0% | 0,0% | 0,0% | -1,8% | -1,4% | 105,7% | 107,3% | 102,6% |

Reference: 3D-HTM anchor + Tool 4

Tested: reference with Tool 1, Tool 2, Tool 3, Tool 4 disabled.

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons |  | x | x | x | x | x | x | x | x |
| Kendo |  | x | x | x | x | x | x | x | x |
| Newspapercc |  | x | x | x | x | x | x | x | x |
| GhostTownFly |  | x | x | x | x | x | x | x | x |
| PoznanHall2 |  | x | x | x | x | x | x | x | x |
| PoznanStreet |  | x | x | x | x | x | x | x | x |
| UndoDancer |  | x | x | x | x | x | x | x | x |
| 1024x768 |  | x | x | x | x | x | x | x | x |
| 1920x1088 |  | x | x | x | x | x | x | x | x |
| Average |  | x | x | x | x | x | x | x | x |

# Recommendations

The Inter Component Dependencies ad hoc group recommends:

- To review all related input contributions.

- To encourage further work for improving existing tools or propose new ones.