| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0022 |
| --- | --- |

| Title: | 3D-CE2 Summary Report on Illumination Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jaejoon Lee San 14-1 Nongseo-dong Giheung-gu Youngin-si Gyeonggi-do, 446-712, Korea | Tel: Email: | +82-31-280-1724 jaejoon1.lee@samsung.com |
| Source: | CE2 Coordinator |  |  |

_____________________________

# Abstract

This document is the summary report of Core Experiment 2: Illumination Compensation. CE2 was established in the 1st JCT-3V Meeting in Stockholm to test the tools for illumination compensation in 3D video coding. Two proposals were made in this meeting. One is AVC based, and the other one is HEVC based. Summary of proposed tools and cross check results are reported.

# Participants

Participants are as follows.

| Participant | Contact | Email | Proponent/ Crosschecker |
| --- | --- | --- | --- |
| Samsung | Jaejoon Lee (coordinator) Michael Mishourovsky Alexey Fartukov Igor Kovluga Konstantin Dubkov Ilsoon Lim Kwan-Jung Oh | jaejoon1.lee@samsung.com mike.mishky@samsung.com a.fartukov@samsung.com i.kovliga@samsung.com k.dubkov@samsung.com ilsoon.lim@samsung.com kwanjung.oh@samsung.com | P |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp | C |
| LG | Hongbin Liu Jiwook Jung Jaewon Sung Jie Jia Sehoon Yea | hongbin.liu@lge.com jiwook.jung@lge.com jw.sung@lge.com jie.jia@lge.com sehoon.yea@lge.com | P |
| Qualcomm | Li Zhang Ying Chen Jewon Kang | lizhang@qualcomm.com cheny@qualcomm.com jewonk@qualcomm.com | C |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Yu-Wen Huang Shawmin Lei | jl.lin@mediatek.com yiwen.chen@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com | C |
| Orange Labs | J. Jung | joelb.jung@orange.com | C |
| ETRI | Gun Bang | gbang@etri.re.kr | C |

# List of input documents

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| Samsung | JCT3V-B0031 | CE2.a results on inter-view coding with adaptive luminance compensation | Proposal |
| ETRI/KHU | JCT3V-B0071 | 3D-CE2.a: Cross check report of JCT2-B0031 on inter-view coding with adaptive luminance | Cross check (Missing) |
| NTT | JCT3V-B0106 | CE2.a: Cross check report of JCT3V-B0031 on inter-view coding with adaptive luminance compensation | Cross check |
| LG | JCT3V-B0045 | 3D-CE2.h : Results of Illumination Compensation for Inter-View Prediction | Proposal |
| ETRI/KHU | JCT3V-B0072 | 3D-CE2.h: Cross check report of JCT2-B0045 on Illumination Compensation for Inter-View Prediction | Cross check |
| Qualcomm | JCT3V-B0209 | 3D-CE2.h related: Cross check of JCT3V-B0045 Results of Illumination Compensation for Inter-View Prediction by LG | Cross check |

# Summary Proposals

- 3.1 JCT3V-B0031 [Samsung] Inter-view coding with adaptive luminance compensation 
   Cross-checked by ETRI/KHU (JCT3V-B0071) and NTT (JCT3V-B0106)

This technology deals with adaptive compensation of luminance discrepancy locally existing between adjacent views of a multiview video captured or rendered by a multiview system. An estimation process includes calculation of a luminance discrepancy between pixels of so-called template area nearby a reference block and pixels of template area nearby an encoding block. In order to enhance precision, outliers rejection technique is implemented through simple threshold detector. Single correction factor is estimated for all pixels of a reference block followed by correction that is realized using simple shift and multiplication operations solely. This method is realized as an auxiliary mode for macroblock modes like P16x16, P16x8, P8x16, P8x8 and P_Skip and is applied to P-slices of depended views using interview reference frame.

- 3.2 JCT3V-B0045 [LG] Illumination compensation for inter-view prediction 
Cross-checked by ETRI/KHU (JCT3V-B0072) and Qualcomm (JCT3V-B0209)

In the proposed method, linear IC model is employed to compensate illumination discrepancy between different views. Parameters in IC model are estimated for each PU (prediction unit) using available reconstructed neighbouring pixels and therefore are not need to be encoded. Applying IC or not is decided at CU (coding unit) level, and an IC flag is coded for CU in which inter-view prediction is used. A method is also designed to adaptively determine whether there is illumination discrepancy in a picture, and an IC flag is encoded for the first slice of each picture in dependent views.

# Testing Results

Test set of CE2 is as follows. Currently, weighted prediction is turned off in the ATM and HTM. To evaluate the performance of illumination compared to weighted prediction, test set is consisted as shown in Table 1..

Table 4.1 Test set of CE2.a

| Test Sets | Anchor | To be tested |  |
| --- | --- | --- | --- |
|  |  | Weighted Prediction | Proposed Tool |
| T1 | WP Off (CTC) | Off | On |
| T2 | WP On | On | On |
| T3 | WP Off | On | Off |

Table 4.2 Test results of JCT3V-B0031 (AVC- based)

| Sequences | BD-Rate (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | T1 WP Off vs. WP Off+ Proposed |  | T2 WP On vs. WP On + Proposed |  | T3 WP Off vs. WP On |  |
|  | Dec | Syn | Dec | Syn | Dec | Syn |
| Poznzn Hall2 | -2.52 | -1.68 | -2.83 | -2.01 | 0.61 | 0.82 |
| Poznan Street | -0.65 | 0.06 | -0.73 | 0.00 | 1.03 | 1.04 |
| Undo Dancer | -0.36 | -0.17 | -0.72 | -0.29 | 1.13 | 0.85 |
| GT Fly | -0.21 | -0.20 | -0.14 | -0.08 | 3.62 | 3.74 |
| Kendo | -3.03 | -2.13 | -3.48 | -2.55 | 2.90 | 3.35 |
| Balloons | -3.42 | -2.23 | -3.41 | -2.24 | 0.23 | 0.29 |
| Newspper | -2.80 | -1.61 | -3.19 | -1.92 | 4.22 | 4.42 |
| Average | -1.86 | -1.14 | -2.07 | -1.30 | 1.96 | 2.07 |
| Encoding Time (%) | 102.54 |  | 102.12 |  | 102.12 |  |
| Decoding Time (%) | 101.66 |  | 101.70 |  | 119.19 |  |

JCT3V-B0031 was cross checked by NTT and results are matched.

Table 4.3 Test results of JCT3V-B0045 (HEVC- based)

| Sequences | BD-Rate (%) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Set1 WP On |  |  | Set2 WP On for Luma |  |  | Set3 WP & IC On for Luma |  |  |
|  | Video only | Syn only | Coded & syn | Video only | Syn only | Coded & Syn | Video only | Syn only | Coded & syn |
| Balloons | 0.2 | 0.5 | 0.5 | -0.6 | -0.4 | -0.5 | -0.3 | 0.1 | 0.0 |
| Kendo | 0.1 | 1.0 | 1.0 | -1.4 | -1.0 | -1.1 | -0.8 | 0.3 | 0.2 |
| Newspaper | 0.2 | 0.4 | 0.4 | -0.3 | -0.2 | -0.2 | -0.1 | 0.2 | 0.2 |
| GT Fly | 0.4 | -0.2 | -0.1 | 0.0 | 0.0 | 0.0 | 0.4 | -0.1 | -0.1 |
| Poznan Hall2 | 0.3 | 1.0 | 0.9 | -0.2 | -0.1 | -0.1 | 0.1 | 0.9 | 0.8 |
| Poznan Street | 0.1 | 0.2 | 0.2 | 0.0 | 0.0 | 0.0 | 0.1 | 0.2 | 0.2 |
| Undo Dancer | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Average | 0.2 | 0.4 | 0.4 | -0.3 | -0.2 | -0.3 | -0.1 | 0.2 | 0.2 |
| Encoding Time (%) | 122.9 |  |  | 108.0 |  |  | 128.5 |  |  |
| Decoding Time (%) | 100.3 |  |  | 99.8 |  |  | 101.3 |  |  |

Table 4.3 Test results of JCT3V-B0045 (HEVC- based) continued.

| Sequences | BD-Rate (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Set4 WP & IC On for Luma & Depth |  |  | Set5 WP & IC On for Luma & Chroma |  |  |
|  | Video only | Syn only | Coded & syn | Video only | Syn only | Coded & Syn |
| Balloons | -0.6 | -0.6 | -0.6 | -0.6 | -0.6 | -0.6 |
| Kendo | -1.4 | -1.6 | -1.6 | -1.6 | -1.4 | -1.3 |
| Newspaper | -0.3 | -0.4 | -0.4 | -0.4 | -1.2 | -1.0 |
| GT Fly | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Poznan Hall2 | -0.2 | -0.1 | -0.1 | -0.1 | -0.2 | -0.2 |
| Poznan Street | 0.0 | -0.1 | -0.1 | -0.1 | -0.3 | -0.2 |
| Undo Dancer | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Average | -0.3 | -0.4 | -0.4 | -0.4 | -0.6 | -0.5 |
| Encoding Time (%) | 115.0 |  |  | 106.6 |  |  |
| Decoding Time (%) | 101.3 |  |  | 99.1 |  |  |

JCT3V-B0045 was cross checked by ETRI/KHU for set1 and set2 and matched. Set 3, set4, and set5 were cross checked by Qualcomm and matched results were obtained for set 3, and set5. Cross check result of Set 4 was mismatched.

# Closing Remarks

Illumination compensation shows better performance than weighted prediction. BD-rate gain of 1.86 % and 1.14 % for coded and synthesized view were observed for AVC based. BD -rate gain of 0.5% was achieved in HEVC based illumination compensation for coded and synthesized views.

Weighed prediction causes coding loss both in AVC based and HEVC based.