| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Shanghai, CN, 13–19 October 2012 | Document: JCT3V-B0023 |
| --- | --- |

| Title: | 3D-CE3: Summary report of Core Experiment 3 on Inter-Component Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 92130 ISSY LES MOULINEAUX France | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

This document summarizes the activities of the Core Experiment CE3 on Inter-Component Prediction. At the 1st JCT-3V meeting some input contribution proposed tools including inter-component predictions. Inter component predictions correspond to a prediction of a data of the component 1, using data (or computations of data) of component 2, where a component is either the texture or the depth.

The experiments are clustered in 2 categories: category A of the 3D-ATM, category H for the 3D-HTM. 10 companies were involved in the CE (2 of them contributing), and 6 related contributions have been registered.

# Document List

|  | Tester 1 | Tester 2 (cross-check) | x-check |
| --- | --- | --- | --- |
| Category A |  |  |  |
| A1 | Samsung – JCT3V-B146 | Merl – JCT3V-B184 | OK |
| H1 | Orange – JCT3V-B0068 | Intel – JCT3V-B0066 | OK |
| H2 | Orange – JCT3V- B0068 | Panasonic – JCT3V-B0143 | OK |
| H3 | Orange – JCT3V- B0068 | Samsung – JCT3V-B170 | OK |

Table 1: document list and x-check status.

# 3D-ATM: experiments and results

JCT3V-A0042 [Samsung] 3D-AVC results on improved inside view motion prediction

| Experiment | Proposed Method | Texture Coding | Depth Coding | Total Coded | Total Synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | OFF |  |  |  |  |  |  |  |
| A1 | Method 1 | 0% | -0.7% | -0.1% | 0% | 100% | 100% | NA |
|  |  |  |  |  |  | 100% | 106% |  |
| A2 | Method 2 | 0% | -0.8% | -0.1% | 0% | 100% | 100% | NA |
|  |  |  |  |  |  | 100% | 108% |  |

Table 2: Summary of the experiments (second line corresponds to the cross-check) – Section A.

Current design includes IVMP (Inside View Motion Prediction - m22583). In IVMP, the motion information, including mb_type, sub_mb_type, reference indices and motion vectors of the co-located MB in texture view component is inherited by the depth view component of the same view. IVMP is enabled when all of the corresponding four texture macroblocks are coded as skip or inter 16x16 or inter 16x8 or inter 8x16.

Method 1: experiment A1

Modifications to the existing IVMP tool are proposed to improve coding efficiency. The IVMP availability conditions and motion prediction method are changed: it is proposed to enable IVMP when at least one corresponding texture macroblock is coded as inter mode including P8x8 mode. In other words, IVMP is always enabled except when all corresponding four texture macroblocks are intra coded.

For the intra coded texture macroblock, motion information of inter coded neighboring macroblock is used. The macroblock which has more complex mb_type is selected when the multiple inter coded neighboring macroblocks are available. If the available neighboring macroblocks have same mb_type, the priority order is as follows: vertical, horizontal, diagonal.

The modified motion prediction uses 8x8 block size regardless of mb_type of corresponding texture. 
Instead, process that finds the reliable motion vector is added. The motion information of the 4x4 block that has motion vector of maximum magnitude is selected for motion prediction.

Method 2: experiment A2

Modifications to the existing IVMP tool are proposed to improve coding efficiency. The IVMP availability conditions and motion prediction method are changed. IVMP is always enabled.

For the intra coded texture macroblock, motion information of inter coded neighboring macroblock is used.  If the available neighboring macroblocks have same motion cost, the priority order is as follows: vertical, horizontal, diagonal.  The motion information of the 4x4 block that has motion vector of maximum magnitude is selected for motion prediction.

# 3D-HTM: experiments and results

| Experiment | Proposed Method |  |  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Applied on Intra Slices | Applied on Inter Slices | 2NxN simplif. |  |  |  |  |  |  |  |  |  |
| Anchor | OFF | OFF | OFF | - | - | - | - | - | - | - | - | - |
| H1 | ON | ON | OFF | 0,0% | 0,0% | 0,0% | 0,0% | 0,6% | -0,1% | 66,2% | 101,7% | 99,9% |
|  |  |  |  |  |  |  |  |  |  | 70.3% | 104.0% | NA |
| H2 | OFF | ON | OFF | 0,0% | 0,0% | 0,0% | 0,0% | 0,3% | -0,3% | 74,9% | 103,6% | 107,0% |
|  |  |  |  |  |  |  |  |  |  | 70.7% | 103.3% | 105.9 |
| H3 | OFF | ON | ON | 0,0% | 0,0% | 0,0% | 0,0% | 0,4% | -0,3% | 67,8% | 100,5% | 99,5% |
|  |  |  |  |  |  |  |  |  |  | 65.8% | 102.4% | NA |

Table 3: Summary of the experiments (second line corresponds to the cross-check) – Section H.

JCT3V-A0044 [Orange Labs] Depth Quadtree Prediction for HTM

This contribution presents a modification of the depth quadtree decoding process. For a given CTU, the quadtree of the depth is linked to the collocated CTU quadtree in the texture, in a way that a given CU of the depth cannot be split more than its collocated CU in the texture. The quadtree of the depth is predictively encoded relatively to the quadtree of the texture.

Experiment H1 applies the proposed method on both Intra and Inter slices.

Experiment H2 applies the proposed method on Inter slices only.

Experiment H3 is a simplification of experiment H2, with the following 2NxN and Nx2N partitions difference:

- H2 experiment: when the texture is split in 2NxN (resp Nx2N), depth allows 2NxN (resp Nx2N) partitioning and disallows Nx2N (resp 2NxN) partitioning and NxN partitioning.

- H3 experiment: when the texture is split in 2NxN (or Nx2N), depth disallows 2NxN, Nx2N, and NxN partitioning.

# Conclusion

Regarding category A, experiment A2 seems the most promising, providing gains of 0.8% for depth coding, 0.1% for total coded, with 100% runtime for encoder and decoder. The CE recommends reviewing the contribution.

Regarding category H, experiment H3 seems the most promising, providing gains of 0.3% on coded + synthesized views, along with a 67% runtime at the encoder, and a 101% runtime at the decoder. The CE recommends reviewing the contribution and in particular discussing the potential impact of the dependence between the parsing of the texture and the parsing of the depth.