| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0031 |
| --- | --- |

| Title: | CE2: CE2.A results on inter-view coding with adaptive luminance compensation Cross-check reference: by NTT, ETRI, KHU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mikhail Mishurovskiy Alexey Fartukov Igor Kovliga Jaejoon Lee | Tel: Email: | +7-495-7972500 3760 mike.mishky@samsung.com +7-495-7972500 3756 a.fartukov@samsung.com +7-495-7972500 3765 i.kovliga@samsung.com 031-280-1724 jaejoon1.lee@samsung.com |
| Source: | Samsung Moscow Research Center, Samsung Electronics Co. LTD |  |  |

_____________________________

# Abstract

This document describes an adaptive luminance compensation tool for inter-view video encoding implemented in the 3D-ATM 5.1 reference test model. Starting from 100 MPEG meeting, this technology has been studied to fulfill major requirements claimed to the tools being considered in the scope of 3D video coding standardization. Based on the outcomes of 1st JCT-3V meeting this technology was included into CE2 that deals with illumination compensation tools. The following aspects are covered by this description: mathematical background of the proposed approach, technical details of integration including but not limited to proposed syntax, semantic, decoding process. Final part of the document is devoted to measurements’ results, conclusions and recommendations given by authors. Measurements have been accomplished in accordance with CTC and additional requirements specified in the CE2. The following results are reported: total compression gain (coded PSNR): -1.86% BD-Rate, total compression gain (Synthetic PSNR): -1.14% BD-Rate; Compression Gain (Depended views): -4.68% BD-Rate, Maximum Compression Gain (dependent view): -10.2% BD-Rate for left view, S06. Encoder Complexity: 102.5%, Decoder Complexity: 101.2~101.6%.

# Introduction

It is well known that the majority of natural multi-view sequences and even some of synthesized multi-view sequences usually demonstrate inter-view mismatches in particular, local luminance discrepancy between object projections captured by adjacent cameras of 3D capturing system even if they are matched geometrically perfectly. Importance for compression can be explained easily by inevitable increasing of residual signal right after classical prediction stage if an inter-view luminance mismatch exists. Classical approach assumes that increased residuals have to be encoded explicitly in the bitstream, decreasing overall compression efficiency.

To cope with this and similar to this issues, several techniques have been proposed so far. Firstly, AVC standard [1] includes so-called weighted prediction that was originally proposed to compensate fade-up/out, flickering or scene-change effects in case of video sequence coding. In general, weighted prediction technique potentially suppresses illumination changes between coding and reference frames. Weighting factors are the same for all macroblocks of a particular slice, thus they can be considered as global. Weighting factors are determined and stored in a bitstream («explicit» weighted prediction). Nevertheless in case of local luminance changes existing in a sequence (video or multiview) global correction can be not so effective. This phenomenon is addressed in the [3] and was decided to be studied in the scope of CE2.

It is worth mentioning several techniques that make local correction of luminance discrepancy between frames (in 2D or 3D video): an adaptive block-wise illumination compensation technique [4], Multi- view One-Step Affine Illumination Compensation [5, 6], US 2011/0286678, US 2008/0304760.

Based on comprehensive analysis of approaches and techniques related with effective compensation of local luminance discrepancy, technology presented in this document and referred further as Adaptive Luminance Compensation tool (ALC) covers several aspects of this process such as: compensation model, luminance discrepancy estimation process, motion (disparity) vector derivation, integration details.

The following distinctive features of the proposed technology can be highlighted: compensation of local luminance changes existing between adjacent views, use of inter-view only reference without any synthetic frames, special technique for outliers rejection during estimation of compensation parameters in order to improve reliability of estimation, use of relatively simple calculations during compensation process that are based on solely integer shifts and multiplications, minor changes of syntax at macroblock and slice layers.

In order to compare efficiency of weighted prediction tool and proposed technology, additional measurements were conducted; achieved results are reported in the last part of the document.

# Syntax in tabular form

In this section, additional syntax elements introduced in the proposal are described. Syntax elements which are added or extended in comparison to AVC or 3D ATM v.5.1r2 [1][2],  are marked in yellow.

## Syntax

### Sequence parameter set 3DVC extension syntax

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| for( i = 0; i <= num_level_values_signalled_minus1; i++ ) |  |  |
| for( j = 0; j <= num_applicable_ops_minus1[ i ]; j++ ) |  |  |
| applicable_op_num_depth_views_minus1[ i ][ j ] |  | ue(v) |
| depth_info_present_flag | 0 | u(1) |
| if( depth_info_present_flag ) { |  |  |
| 3dv_acquisition_idc | 0 | ue(v) |
| for( i = 0; i < num_views_minus1; i++ ) |  |  |
| view_id_3dv[ i ] | 0 | ue(v) |
| if( 3dv_acquisition_idc ) { |  |  |
| depth_ranges( 2, 0 ) |  |  |
| if( profile_idc = = 139 &&num_views_minus1 ) |  |  |
| vsp_param( 2, 0 ) |  |  |
| } |  |  |
| if ( profile_idc = = 139 ) |  |  |
| for( i = 0; i<= num_views_minus1; i++ ) |  |  |
| depth_preceding_texture_flag[ i ] | 0 | u(1) |
| } |  |  |
| if( profile_idc = = 139 && depth_info_present_flag ) { |  |  |
| reduced_resolution_flag | 0 | u(1) |
| slice_header_prediction_flag | 0 | u(1) |
| inside_view_mvp_flag | 0 | u(1) |
| disp_flag | 0 | u(1) |
| psip_flag | 0 | u(1) |
| nonlinear_depth_representation_num | 0 | ue(v) |
| for( i = 1; i <= nonlinear_depth_representation_num; i++ ) |  |  |
| nonlinear_depth_representation_model[ i ] | 0 | ue(v) |
| seq_view_synthesis_flag | 0 | u(1) |
| } |  |  |
| if( profile_idc = = 139 && !depth_info_present_flag ) { |  |  |
| enable_alc_encoder_flag | 0 | u(1) |
| } |  |  |
| } |  |  |

### Slice data syntax

| slice_data( ) { | C | Descriptor |
| --- | --- | --- |
| if( entropy_coding_mode_flag ) |  |  |
| while( !byte_aligned( ) ) |  |  |
| cabac_alignment_one_bit | 2 | f(1) |
| CurrMbAddr = first_mb_in_slice * ( 1 + MbaffFrameFlag ) |  |  |
| moreDataFlag = 1 |  |  |
| prevMbSkipped = 0 |  |  |
| do { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| if( !entropy_coding_mode_flag ) { |  |  |
| mb_skip_run | 2 | ue(v) |
| prevMbSkipped = ( mb_skip_run > 0 ) |  |  |
| for( i=0; i<mb_skip_run; i++ ) |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && mb_skip_run > 0 && VspRefExist ) |  |  |
| mb_skip_type_flag | 2 | u(1) ¦ ae(v) |
| if( mb_skip_run > 0 ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| } else { |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && VspRefExist && leftMbVSSkipped && upMbVSSkipped ) { |  |  |
| mb_vsskip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_vsskip_flag |  |  |
| if( !mb_vsskip_flag ) { |  |  |
| mb_skip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_skip_flag |  |  |
| } |  |  |
| } else { |  |  |
| mb_skip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_skip_flag |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && VspRefExist && !mb_skip_flag ) { |  |  |
| mb_vsskip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_vsskip_flag |  |  |
| } |  |  |
| } |  |  |
| if ( enable_alc_encoder_flag && nal_unit_type == 21 && |  |  |
| slice_type == P && !DepthFlag && |  |  |
| !mb_vsskip_flag && mb_skip_flag == 1) { |  |  |
| mb_alc_skip_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| if( moreDataFlag ) { |  |  |
| if( MbaffFrameFlag && ( CurrMbAddr % 2 = = 0 ¦ ¦ ( CurrMbAddr % 2 = = 1 && prevMbSkipped ) ) ) |  |  |
| mb_field_decoding_flag | 2 | u(1) ¦ ae(v) |
| macroblock_layer( ) | 2 ¦ 3 ¦ 4 |  |
| } |  |  |
| if( !entropy_coding_mode_flag ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| else { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| prevMbSkipped = mb_skip_flag ¦ ¦ mb_vsskip_flag |  |  |
| if( MbaffFrameFlag && CurrMbAddr % 2 = = 0 ) |  |  |
| moreDataFlag = 1 |  |  |
| else { |  |  |
| end_of_slice_flag | 2 | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |  |
| } |  |  |
| } |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| } while( moreDataFlag ) |  |  |
| } |  |  |

### Macroblock layer syntax

| macroblock_layer( ) { | C | Descriptor |
| --- | --- | --- |
| if ( IvmpEnabledFlag && CheckIVMPAvailable ) |  |  |
| mb_ivmp_flag |  | u(1) ¦ ae(v) |
| if (!mb_ivmp_flag) { |  |  |
| if( DepthFlag && disp_flag ) |  |  |
| mb_disp_flag |  | u(1) ¦ ae(v) |
| if(!mb_disp_flag){ |  |  |
| mb_type | 2 | ue(v) ¦ ae(v) |
| if(nal_unit_type = = 21 && !DepthFlag && slice_type = = B && direct_spatial_mv_pred_flag && VspRefExist && mb_type = = B_Direct_16x16 ) |  |  |
| mb_direct_type_flag | 2 | u(1) ¦ ae(v) |
| if ( enable_alc_encoder_flag && nal_unit_type == 21 && |  |  |
| slice_type == P && !DepthFlag && |  |  |
| ( mb_type == P_L0_16x16 ¦¦ mb_type == P_L0_L0_16x8 ¦¦ |  |  |
| mb_type == P_L0_L0_8x16 ¦¦ mb_type == P_8x8 ¦¦ |  |  |
| mb_type == P_8x8ref0) ) { |  |  |
| mb_alc_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| if( !mb_disp_flag ) { |  |  |
| if( mb_type = = I_PCM ) { |  |  |
| while( !byte_aligned( ) ) |  |  |
| pcm_alignment_zero_bit | 3 | f(1) |
| for( i = 0; i < 256; i++ ) |  |  |
| pcm_sample_luma[ i ] | 3 | u(v) |
| for( i = 0; i < 2 * MbWidthC * MbHeightC; i++ ) |  |  |
| pcm_sample_chroma[ i ] | 3 | u(v) |
| } else { |  |  |
| noSubMbPartSizeLessThan8x8Flag = 1 |  |  |
| if( mb_type != I_NxN && MbPartPredMode( mb_type, 0 ) != Intra_16x16 && NumMbPart( mb_type ) = = 4 ) { |  |  |
| if( !mb_ivmp_flag ) |  |  |
| sub_mb_pred( mb_type ) | 2 |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 ) { |  |  |
| if( NumSubMbPart( sub_mb_type[ mbPartIdx ] ) > 1 ) |  |  |
| noSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else if( !direct_8x8_inference_flag ) |  |  |
| noSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else { |  |  |
| if( transform_8x8_mode_flag && mb_type = = I_NxN ) |  |  |
| transform_size_8x8_flag | 2 | u(1) ¦ ae(v) |
| if( !mb_ivmp_flag ) |  |  |
| mb_pred( mb_type ) | 2 |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) != Intra_16x16 ) { |  |  |
| coded_block_pattern | 2 | me(v) ¦ ae(v) |
| if( CodedBlockPatternLuma > 0 && transform_8x8_mode_flag && mb_type != I_NxN && noSubMbPartSizeLessThan8x8Flag && ( mb_type != B_Direct_16x16 ¦ ¦ direct_8x8_inference_flag ) ) |  |  |
| transform_size_8x8_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if( CodedBlockPatternLuma > 0 ¦ ¦ CodedBlockPatternChroma > 0 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| mb_qp_delta | 2 | se(v) ¦ ae(v) |
| residual( 0, 15 ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

### Macroblock prediction syntax

| mb_pred( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ){ |  |  |
| for( luma4x4BlkIdx=0; luma4x4BlkIdx<16; luma4x4BlkIdx++ ) { |  |  |
| prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] ) |  |  |
| rem_intra4x4_pred_mode[ luma4x4BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| for( luma4x4BlkIdx=0; luma4x4BlkIdx<16; luma4x4BlkIdx++ ) { |  |  |
| if( psip_flag && Psip4x4Available && Intra4x4PredMode[luma4x4BlkIdx] = = 2 ){ |  |  |
| psip4x4_dir[ luma4x4BlkIdx ] | 2 | u(1) |
| for( idxRowCol=0; idxRowCol<4; idxRowCol++ ) |  |  |
| psip4x4_code_diff[ luma4x4BlkIdx ][ idxRowCol ] | 2 | se(v) ¦ ae(v) |
| [Ed. CAVLC syntax missing] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ){ |  |  |
| for( luma8x8BlkIdx=0; luma8x8BlkIdx<4; luma8x8BlkIdx++ ) { |  |  |
| prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] ) |  |  |
| rem_intra8x8_pred_mode[ luma8x8BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| for( luma8x8BlkIdx =0; luma8x8BlkIdx <4; luma8x8BlkIdx ++ ) { |  |  |
| if( psip_flag && Psip8x8Available && Intra8x8PredMode[luma4x4BlkIdx] = = 2 ){ |  |  |
| psip8x8_dir[ luma8x8BlkIdx ] | 2 | u(1) |
| for( idxRowCol=0; idxRowCol<8; idxRowCol++ ) |  |  |
| psip8x8_code_diff[ luma8x8BlkIdx ][idxRowCol] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| if( psip_flag && Psip16x16Available && Intra16x16PredMode = = 2 ){ |  |  |
| psip16x16_dir | 2 | u(1) |
| for( idxRowCol=0; idxRowCol<16; idxRowCol++ ) |  |  |
| psip16x16_code_diff[ idxRowCol ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| if( ChromaArrayType = = 1 ¦ ¦ ChromaArrayType = = 2 ) |  |  |
| intra_chroma_pred_mode | 2 | ue(v) ¦ ae(v) |
| } else if( MbPartPredMode( mb_type, 0 ) != Direct ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L1 && mb_alc_flag == 0) |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode ( mb_type, mbPartIdx ) != Pred_L1 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |

### Sub-macroblock prediction syntax

| sub_mb_pred( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| sub_mb_type[ mbPartIdx ] | 2 | ue(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && mb_type != P_8x8ref0 && sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L1 && mb_alc_flag == 0) |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 ) |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L1 ) |  |  |
| for( subMbPartIdx = 0; subMbPartIdx < NumSubMbPart( sub_mb_type[ mbPartIdx ] ); subMbPartIdx++) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ subMbPartIdx ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 ) |  |  |
| for( subMbPartIdx = 0; subMbPartIdx < NumSubMbPart( sub_mb_type[ mbPartIdx ] ); subMbPartIdx++) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ subMbPartIdx ][ compIdx ] |  |  |
| } |  |  |

## Semantics

1-bit flag is added to the sequence parameter set 3DVC extension to indicate whether Adaptive Luminance Compensation tool is enabled or not:
enable_alc_encoder_flag being equal to 1 specifies that Adaptive Luminance Compensation tool is enabled for encoding MVD sequence. enable_alc_encoder_flag being equal to 0 specifies that Adaptive Luminance Compensation tool is not used for encoding of MVD sequence and no additional flags shall be existed in the slice data syntax and macroblock layer syntax.

1-bit flag is added in slice data syntax:
mb_alc_skip_flag determines the reference picture type, motion vector prediction and ALC if a macroblock is skipped. mb_alc_skip_flag equal to 1 specifies that the current macroblock is skipped,  reference is defined as a first interview reference in the reference picture list 0 and motion vector prediction for the first interview reference component is done as described below in the clause 3.2. mb_alc_skip_flag being equal to 0 specifies conventional  skip mode and no additional correct is applied to the current macroblock. When mb_alc_skip_flag is not present, it shall be inferred to be 0.

1-bit flag is added at the macroblock layer syntax:
mb_alc_flag being equal to 1 specifies the reference picture type and ALC for the current inter-coded non-skipped macroblock. mb_alc_flag equals to 1 specifies that the current macroblock uses first interview reference component in the reference picture list 0 as reference and ALC. mb_alc_flag being equal to 0 specifies that common rules are used to derive reference indices and ALC is not used for the current macroblock. When mb_alc_flag is not present, it shall be inferred to be 0.

# Decoding process

Proposed ALC tool aims to compensate local luminance changes between pictures from different views. ALC is performed for P_Skip mode when mb_alc_skip_flag is equal to 1. ALC is performed for P_L0_16x16, P_L0_L0_16x8, P_L0_L0_8x16, P_8x8, P_8x8ref0 modes when mb_alc_flag is equal to 1.

## Adaptive luminance compensation

ALC is performed for luma samples only. Each prediction area for sub-partition of ALC macroblock is divided into n non-overlapped parts (predPart[ x, y ]) which are ordered in the decoding order, where n – number of parts for each subpart of ALC macroblock.  Sizes of parts (psx, psy) depend on transform type (DCT4x4 or DCT8x8) and macroblock type:

| Name of mb_type | DCT8x8 | DCT4x4 |
| --- | --- | --- |
| P_L0_16x16 | psx=16, psy=16 (n=1) | psx=8, psy=8 (n=4) |
| P_L0_L0_16x8 | psx=16, psy=8 (n=1) | psx=4, psy=4 (n=8) |
| P_L0_L0_8x16 | psx=8, psy=16(n=1) | psx=4, psy=4 (n=8) |
| P_8x8, P_8x8ref0 | psx=8, psy=8(n=1) | psx=4, psy=4 (n=4) |
| P_Skip | psx=16, psy=16(n=1) |  |

ALC is performed for all values of each predParti[ x, y ] of the macroblock (i=1..n). Finally predicted samples predPartALCi [ x, y ] are derived by:

predPartALCi [ x, y ] =Min(255,  ( predParti [ x, y ] * Wi + 2logWDC − 1 ) >> logWDC )

where logWDC=15 and Wi is weight that is calculated for predParti [ x, y ]. >> is arithmetic right shift.

Weight Wi is calculated for each part as follows:

Wi=1,if(Ref_Refi>>CUTTH)==(Ref_Deci>>CUTTH)

where / - is integer division with rounding to zero,

Ref_Deci=1+k,m=[1..4,1..psy]LTDecik,m×1,LTDecik,m−LTRefik,m<TH

TH - is predefined threshold that is equal to 30; CUT_TH – is another predefined threshold that is equal to 4.

LTRefi - is a block located in the same reference frame as predParti. LTRefi  has width=4 and height the same as height of predParti. LTRefi  is shifted to the left relatively to predParti by 4 pixels.

UTRefi - is a block located in the same reference frame as predParti. UTRefi  has height=4 and width the same as width of predParti. UTRefi is shifted to the top relatively to predParti by 4 pixels.

LTDeci - is a block located in already decoded area of the currently decoding frame. LTDeci  has width=4 and height the same as height of predParti. LTDeci is shifted to the left by 4 pixels relatively to a block that corresponds to predParti in the decoded frame.

UTDeci - is a block located in already decoded area of the currently decoding frame. UTDeci  has height=4 and width the same as width of predParti. UTDeci is shifted to the top by 4 pixels relatively to a block that corresponds to predParti in the decoded frame.

Decoded Frame

Interview

Reference Frame

Decoded block correspond predParti

predParti

UTDeci

UTDeci

LT

Refi

UTRefi

Motion Vector

## Derivation of motion vector prediction and reference indices for ALC

If ALC tool is used, reference index refIdxL0 is derived as an interview picture that appears first in RefPicList0.

The depth-based derivation process for median luma motion vector prediction in subclause J.8.3.1.8 [2] is invoked for deriving motion vector prediction mvpL0 for P_Skip, P_L0_16x16, P_L0_L0_16x8, P_L0_L0_8x16, P_8x8, P_8x8ref0 modes.

predFlagL0 is set equal to 1.

mvL1 and refIdxL1 are marked as not available and predFlagL1 is set equal to 0.

# Parsing process

Three context models are added to the context table for writing mb_alc_skip_flag into bitstream by CABAC. These models are defined similar to the models of mb_skip_flag:

Let the variable condTermFlagN (with N being either A or B) be derived as follows:

- If mbAddrN is available and mb_alc_skip_flag or mb_alc_flag for the macroblock mbAddrN are equal to 1, condTermFlagN is set equal to 1.

- Otherwise (mbAddrN is not available or mb_alc_skip_flag and mb_alc_flag for the macroblock mbAddrN are equal to 0), condTermFlagN is set equal to 0.

The variable ctxIdxInc is derived as:

ctxIdxInc = condTermFlagA + condTermFlagB

In order to encode mb_alc_flag by CABAC, three separate context models are used. Definition of these models is the same as for mb_alc_skip_flag.

For initialization of each context models, the values of (m, n) are obtained experimentally. For three context models of mb_alc_skip_flag and mb_alc_flag, the values (14, 4), (14, 27), (10, 52) are used as (m, n).

# Encoding process

“Encoding process” being non-normative part of this proposal provides particular details on how encoding is implemented.

Similar to all standard encoding modes supported by 3D-ATM, macroblock is encoded in each of ALC encoding modes if only it belongs to texture from any of dependent views. Then RD cost is calculated similar to that of standard encoding modes followed by comparison with the best RD cost and the best encoding mode determined so far.

First inter-view reference frame in the reference picture list 0 is used as a source for the current macroblock prediction that is defined based on a certain prediction mode. In order to perform motion estimation procedure that generates displacement vector, search zone is defined based on the assumption that multiview video sequence is already rectified. In particular, relatively narrow search zone is implemented that has several quarter-pixels height and several integer pixels width.

Encoded Frame

Interview Reference Frame

Displacement Vector

2 x Search Zone Width

2 x Search Zone Height

Predicted block

Encoded block

Parameters that are used for search zone determination are defined in the configuration file for texture:

AlcSearchRangeX:

Defines allowable search range along X axis for motion estimation in Adaptive Luminance Compensation tool.

AlcSearchRangeY:

Defines allowable search range along Y axis for motion estimation in Adaptive Luminance Compensation tool.

Actual search range is defined as [-AlcSearchRangeX...AlcSearchRangeX] for horizontal direction and [-AlcSearchRangeY...AlcSearchRangeY] for vertical direction both expressed in QPels. Search zone center is defined by motion vector prediction described in sub-clause 3.2. In order to provide higher precision of displacement vector determination “Fast Full Search” [1] is used at the encoder side for ALC encoding modes. In order to emphasize potential discrepancy of luminance between encoded and predicted blocks that finally means more accurate estimation, we proposed to modify distortion metric from SAD to Mean-Removed SAD4x4 (MR_SAD4x4) and Mean-Removed SAD4x4_DC (MR_SAD4x4_DC).

In particular, MR_SAD4x4 is defined as a sum of mean-removed SADs over all sub-blocks 4x4 included into encoded block with size (H, W):

MRSAD4×4=k=0,m=0k=H4,m=W4i=0;j=0i=3;j=3X4k+i,4m+j−M4[X,4k,4m]−R4k+i+DVy,4m+j+DVx+M4[R,4k+DVy,4m+DVx]M4[X,x,y]=i=yy+3j=xx+3Xi,j>>4,

Here: R – reference frame; X- encoded frame; H – height of an encoded block; W – width of an encoded block.

Slightly enhanced distortion metric that uses the difference between middle levels of encoded and reference blocks is refereed as MR_SAD4x4_DC and defined as:

MRSAD4×4DC=MRSAD4×4+k=0,m=0k=H4,m=W4M4[X,4k,4m]−M4[R,4k+DVy,4m+DVx].

Our experiments show that whereas complexity of the encoder is not increased significantly in case of use MR_SAD4x4 or MR_SAD4x4_DC instead of SAD metric, compression gain slightly goes up that confirms use of such metrics in encoders. Nevertheless even standard SAD metric can be used effectively in order to define more or less accurate displacement vector. Details can be found in the appendix.

# Experimental results

PC (Server 1): Intel Core I7-3930K@3.20GHz; 32GB RAM, W7(64) SP1

PC: Inter Core I5-2500@3.3GHz; 16GB, W7(64) SP1

## T1: 3D-ATM 5.1r2 WP OFF vs. 3D-ATM 5.1r2 + ALC, WP OFF

| Compression Gain |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | Texture Coding |  | Depth Coding |  | Total Coding ( coded PSNR) |  | Total Coding ( synthesized PSNR) |  |
|  | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB |
| S01 | -2.70 | 0.09 | 0.00 | 0.00 | -2.52 | 0.08 | -1.68 | 0.06 |
| S02 | -0.64 | 0.02 | 0.00 | 0.00 | -0.65 | 0.02 | 0.06 | 0.00 |
| S03 | -0.37 | 0.01 | 0.00 | 0.00 | -0.36 | 0.01 | -0.17 | 0.01 |
| S04 | -0.22 | 0.01 | 0.00 | 0.00 | -0.21 | 0.01 | -0.20 | 0.01 |
| S05 | -3.39 | 0.16 | 0.00 | 0.00 | -3.03 | 0.14 | -2.13 | 0.09 |
| S06 | -3.62 | 0.18 | 0.00 | 0.00 | -3.42 | 0.17 | -2.23 | 0.10 |
| S08 | -2.93 | 0.13 | 0.00 | 0.00 | -2.80 | 0.12 | -1.61 | 0.06 |
| Average | -1.98 | 0.09 | 0.00 | 0.00 | -1.86 | 0.08 | -1.14 | 0.05 |

| Seq. | Complexity |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Encoder Time PC(Server 1) | Decoder Time |  |  |  |
|  |  | PC(Server 1) |  |  | (PC) 4th Run |
|  |  | 1st Run | 2nd Run | 3rd Run |  |
| S01 | 103.27 | 102.12 | 101.97 | 102.00 | 100.76 |
| S02 | 103.38 | 102.67 | 102.68 | 102.74 | 101.48 |
| S03 | 103.17 | 101.64 | 101.64 | 101.58 | 100.98 |
| S04 | 102.26 | 101.13 | 101.04 | 101.04 | 100.93 |
| S05 | 102.88 | 100.87 | 100.83 | 101.02 | 101.04 |
| S06 | 102.94 | 101.03 | 100.97 | 100.96 | 100.92 |
| S08 | 99.86 | 102.17 | 102.14 | 102.11 | 101.97 |
| Average | 102.54 | 101.66 | 101.61 | 101.64 | 101.16 |

| Compression Gain for dependent views |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Seq. | (Dep. Views both) |  | (Dep. Views Left) |  | (Dep. Views Right) |  |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -6.47 | 0.20 | -7.90 | 0.25 | -5.00 | 0.15 |
| S02 | -0.73 | 0.02 | -1.53 | 0.03 | 0.11 | 0.00 |
| S03 | -0.71 | 0.03 | -1.21 | 0.04 | -0.21 | 0.01 |
| S04 | -1.11 | 0.04 | -1.34 | 0.04 | -0.87 | 0.03 |
| S05 | -8.41 | 0.35 | -7.38 | 0.31 | -9.46 | 0.39 |
| S06 | -9.14 | 0.38 | -10.20 | 0.42 | -8.18 | 0.33 |
| S08 | -6.17 | 0.24 | -4.85 | 0.18 | -7.32 | 0.30 |
| Average | -4.68 | 0.18 | -4.91 | 0.18 | -4.42 | 0.17 |

## T2: 3D-ATM 5.1r2 WP ON vs. 3D-ATM 5.1r2 WP ON + ALC

| Compression Gain |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | Texture Coding |  | Depth Coding |  | Total Coding ( coded PSNR) |  | Total Coding ( synthesized PSNR) |  |
|  | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB |
| S01 | -3.02 | 0.10 | 0.00 | 0.00 | -2.83 | 0.09 | -2.01 | 0.07 |
| S02 | -0.73 | 0.02 | 0.00 | 0.00 | -0.73 | 0.02 | 0.00 | 0.00 |
| S03 | -0.74 | 0.02 | 0.00 | 0.00 | -0.72 | 0.02 | -0.29 | 0.01 |
| S04 | -0.14 | 0.01 | 0.00 | 0.00 | -0.14 | 0.01 | -0.08 | 0.00 |
| S05 | -3.87 | 0.19 | 0.00 | 0.00 | -3.48 | 0.16 | -2.55 | 0.11 |
| S06 | -3.62 | 0.18 | 0.00 | 0.00 | -3.41 | 0.17 | -2.24 | 0.10 |
| S08 | -3.39 | 0.15 | 0.00 | 0.00 | -3.19 | 0.14 | -1.92 | 0.07 |
| Average | -2.22 | 0.09 | 0.00 | 0.00 | -2.07 | 0.09 | -1.30 | 0.05 |

| Seq. | Complexity |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Encoder Time PC(Server 1) | Decoder Time |  |  |  |
|  |  | PC(Server 1) |  |  | PC 4th Run |
|  |  | 1st Run | 2nd Run | 3rd Run |  |
| S01 | 102.87 | 100.84 | 100.82 | 100.80 |  |
| S02 | 102.60 | 101.68 | 101.68 | 101.63 |  |
| S03 | 102.39 | 100.89 | 100.89 | 100.89 |  |
| S04 | 102.96 | 100.47 | 100.48 | 100.48 |  |
| S05 | 102.70 | 99.79 | 99.76 | 99.97 |  |
| S06 | 101.52 | 100.12 | 100.10 | 100.03 |  |
| S08 | 99.78 | 101.08 | 101.08 | 101.09 |  |
| Average | 102.12 | 100.70 | 100.69 | 100.70 |  |

## T3: 3D-ATM 5.1r2 WP OFF vs. 3D-ATM 5.1r2 WP ON

| Compression Gain |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | Texture Coding |  | Depth Coding |  | Total Coding ( coded PSNR) |  | Total Coding ( synthesized PSNR) |  |
|  | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB |
| S01 | 0.67 | -0.03 | 0.00 | 0.00 | 0.61 | -0.02 | 0.82 | -0.03 |
| S02 | 1.13 | -0.03 | -0.01 | 0.00 | 1.03 | -0.03 | 1.04 | -0.03 |
| S03 | 1.18 | -0.04 | 0.11 | -0.01 | 1.13 | -0.04 | 0.85 | -0.03 |
| S04 | 3.82 | -0.13 | 0.23 | -0.01 | 3.62 | -0.13 | 3.74 | -0.12 |
| S05 | 3.32 | -0.15 | 0.12 | 0.00 | 2.90 | -0.13 | 3.35 | -0.14 |
| S06 | 0.27 | -0.01 | 0.00 | 0.00 | 0.23 | -0.01 | 0.29 | -0.01 |
| S08 | 4.97 | -0.20 | 0.01 | 0.00 | 4.22 | -0.17 | 4.42 | -0.15 |
| Average | 2.19 | -0.09 | 0.07 | 0.00 | 1.96 | -0.08 | 2.07 | -0.07 |

| Seq. | Complexity |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Encoder Time PC(Server 1) | Decoder Time |  |  |  |
|  |  | PC(Server 1) |  |  | PC 4th Run |
|  |  | 1st Run | 2nd Run | 3rd Run |  |
| S01 | 101.38 | 118.96 | 118.93 | 118.97 |  |
| S02 | 101.51 | 121.47 | 121.51 | 121.56 |  |
| S03 | 101.08 | 116.68 | 116.67 | 116.63 |  |
| S04 | 100.70 | 116.23 | 116.17 | 116.16 |  |
| S05 | 100.76 | 119.09 | 119.13 | 119.13 |  |
| S06 | 102.38 | 118.89 | 118.93 | 118.89 |  |
| S08 | 100.83 | 123.01 | 122.99 | 122.93 |  |
| Average | 101.23 | 119.19 | 119.19 | 119.18 |  |

# Conclusions

This document reports recent results on Adaptive Luminance Compensation tool developed by Samsung Moscow Research Center and Samsung Electronics Co. Ltd. Reported tool has been integrated into 3D-ATM 5.1r2 followed by measurements of compression efficiency and complexity in accordance with CTC and CE2 descriptions. Additional experiments have been conducted for objective comparison of Weighted Prediction tool with Adaptive Luminance Compensation tool as well as for evaluating efficiency of Weighted Prediction being applied for multiview video coding. Normative sections like syntax changes, semantic and decoding process are also provided including recommendations on an encoder part implementation. Supplementary materials connected with the influence of distortion metric, template size, motion search type on compression gain and decoder complexity are given in the Appendix.

In the end, we would like to summarize main features of proposed tool:

- Adaptive Luminance Compensation tool provides additional compression gain by interview prediction followed by a particular correction of luminance components of a reference block;

- Technology is based on 32-bit integer ops including arithmetic shifts, simple arithmetic operations and conditional branches;

- There are two amendments of a syntax: 1-bit flag in the SPS, 1 additional 1-bit flag at the slice data level and 1 additional 1-bit flag at the macroblock level;

- Compression gain and complexity in comparison with 3D-ATM51r2 are estimated as follows:

- Texture bitrate: -1.98% BD-Rate; Total bitrate: -1.86% BD-Rate

- Total bitrate, synthetic texture: -1.14% BD-Rate

- Depended views only: -4.68% BD-Rate

- Complexity of the encoder is roughly 102.54%

- Complexity of the decoder is roughly 101.2~101.6%

- It has been observed that weighted prediction tool has negative impact – around 2% BD-Rate gain loss - being turned ON in all experimental setups. These results are also consistent with that of shown in [3] confirming uselessness of weighted prediction for multiview coding;

- No synergy effect has been observed between Weighted Prediction and Adaptive Luminance Compensation tools in all experimental setups.

Based on this we kindly recommend adopting Adaptive Luminance Compensation tool into subsequent release 5.2 of 3D-ATM reference model while keeping Weighted Prediction tool turning off.

# References

[1]	3D-ATM SVN repository http://mpeg3dv.research.nokia.com/svn/mpeg3dv/tags/3DV-ATMv5.1r2/

[2]	M. Hannuksela et al, “3D-AVC Draft Text 3”, JCT-3V, JCT3V-A1002, Stockholm, July 2012.

[3] “Region-based Weighted Prediction for Coding Video with Local Brightness Variations”, TCSVT, Sik-Ho Tsang et al, 1999

[4] US 7,924,923. Motion Estimation and Compensation Method and Device Adaptive to Change in Illumination. April, 2011

[5] Y. Lee, J. Hur, Y. Lee, R. Han, S. Cho, N. Hur, J. Kim, J. Kim, P. Lai, A. Ortega, Y. Su, P. Yin and C. Gomila. CE11: Illumination compensation. Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG JVT-U052, Oct. 2006.

[6] J. H. Kim, P. Lai, J. Lopez, A. Ortega, Y. Su, P. Yin, and C. Gomila. New coding tools for illumination and focus mismatch compensation in multiview video coding. IEEE Trans. on Circuits and Systems for Video Technology, vol. 17, no. 11, pp. 1519-1535, Nov. 2007.

[7] A. Fartukov, I. Kovliga, M. Mishurovskiy, J.Lee., “3D-AVC results on inter-view coding with adaptive luminance compensation algorithm”, ISO/IEC JTC1/SC29/WG11, Doc. M24816, May 2012, Geneva, Switzerland

# Patent rights declaration(s)

Samsung Moscow Research Center and Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## Results on study of weighted prediction in JM18.2 reference S/W

| Reference JMV18.2 vs. Reference JMV18.2 + WP Encoded View: Central (based on CTC) Encoder configuration: In accordance with CTC Number of encoded frames: In accordance with CTC WeightedPrediction = 1 WeightedBiprediction = 1 ChromaWeightSupport = 0 UseWeightedReferenceME = 0 WPMethod = 0 WPIterMC = 0 EnhancedBWeightSupport = 0 WPMCPrecision = 0 WPMCPrecFullRef = 0 WPMCPrecBSlice = 0 |  |  |
| --- | --- | --- |
| Seq. | Texture Coding |  |
|  | DBR, % | dPSNR, dB |
| S01 | 0.11 | 0.00 |
| S02 | 1.63 | -0.06 |
| S03 | 0.65 | -0.02 |
| S04 | 3.79 | -0.14 |
| S05 | 3.13 | -0.16 |
| S06 | 0.08 | 0.-00 |
| S08 | 4.88 | -0.23 |
| Average | 2.04 | -0.09 |

## Results on changes of distortion metric and search type at the encoder side

| Reference 3D-ATM 5.1r2 WPOFF vs. 3D-ATM 5.1r2 WPOFF + ALC ALC Configuration: TemplateSize: 4 Search Zone: +-6/+-20 QPel Search Type: Full Search ( Int-PelSub-PelQ_Pel) Cut_TH: 4 TH: 30 Metric Type: SAD |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Seq. | Texture Coding |  | Depth Coding |  | Total Coding ( coded PSNR) |  |
|  | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB |
| S01 | -2.74 | 0.09 | 0.00 | 0.00 | -2.56 | 0.08 |
| S02 | -0.59 | 0.02 | 0.00 | 0.00 | -0.60 | 0.02 |
| S03 | -0.34 | 0.01 | 0.00 | 0.00 | -0.33 | 0.01 |
| S04 | -0.27 | 0.01 | 0.00 | 0.00 | -0.25 | 0.01 |
| S05 | -2.86 | 0.14 | 0.00 | 0.00 | -2.54 | 0.12 |
| S06 | -3.21 | 0.16 | 0.00 | 0.00 | -3.02 | 0.15 |
| S08 | -2.59 | 0.11 | 0.00 | 0.00 | -2.49 | 0.10 |
| Average | -1.80 | 0.08 | 0.00 | 0.00 | -1.68 | 0.07 |

| Reference 3D-ATM 5.1r2 WPOFF vs. 3D-ATM 5.1r2 WPOFF + ALC ALC Configuration: TemplateSize: 4 Search Zone: +-6/+-20 QPel Search Type: Fast Search (EPSZ) Cut_TH: 4 TH: 30 Metric Type: MR_SAD4x4_DC |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Seq. | Texture Coding |  | Depth Coding |  | Total Coding ( coded PSNR) |  |
|  | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB |
| S01 | -2.43 | 0.08 | 0.00 | 0.00 | -2.25 | 0.07 |
| S02 | -0.54 | 0.02 | 0.00 | 0.00 | -0.55 | 0.02 |
| S03 | -0.32 | 0.01 | 0.00 | 0.00 | -0.30 | 0.01 |
| S04 | -0.20 | 0.01 | 0.00 | 0.00 | -0.19 | 0.01 |
| S05 | -3.03 | 0.15 | 0.00 | 0.00 | -2.68 | 0.12 |
| S06 | -3.32 | 0.16 | 0.00 | 0.00 | -3.11 | 0.15 |
| S08 | -2.65 | 0.11 | 0.00 | 0.00 | -2.52 | 0.11 |
| Average | -1.78 | 0.08 | 0.00 | 0.00 | -1.66 | 0.07 |

| Reference 3D-ATM 5.1r2 WPOFF vs. 3D-ATM 5.1r2 WPOFF + ALC ALC Configuration: TemplateSize: 4 Search Zone: +-6/+-20 QPel Search Type: Fast Search (EPSZ) Cut_TH: 4 TH: 30 Metric Type: SAD |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Seq. | Texture Coding |  | Depth Coding |  | Total Coding ( coded PSNR) |  |
|  | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB |
| S01 | -2.45 | 0.08 | 0.00 | 0.00 | -2.27 | 0.07 |
| S02 | -0.53 | 0.02 | 0.00 | 0.00 | -0.53 | 0.02 |
| S03 | -0.28 | 0.01 | 0.00 | 0.00 | -0.27 | 0.01 |
| S04 | -0.21 | 0.01 | 0.00 | 0.00 | -0.19 | 0.01 |
| S05 | -2.91 | 0.14 | 0.00 | 0.00 | -2.57 | 0.12 |
| S06 | -3.09 | 0.15 | 0.00 | 0.00 | -2.89 | 0.14 |
| S08 | -2.41 | 0.10 | 0.00 | 0.00 | -2.30 | 0.10 |
| Average | -1.70 | 0.07 | 0.00 | 0.00 | -1.57 | 0.07 |

## Dependency of compression gain on template size

| Reference 3D-ATM 5.1r2 WPOFF vs. 3D-ATM 5.1r2 WPOFF + ALC ALC Configuration: TemplateSize: 2 Search Zone: +-6/+-20 QPel Search Type: Full Search ( Int-PelSub-PelQ_Pel) Cut_TH: 4 TH: 30 Metric Type: MR_SAD4x4_DC |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Seq. | Texture Coding |  | Depth Coding |  | Total Coding ( coded PSNR) |  |
|  | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB |
| S01 | -2.53 | 0.08 | 0.00 | 0.00 | -2.36 | 0.08 |
| S02 | -0.62 | 0.02 | 0.00 | 0.00 | -0.63 | 0.02 |
| S03 | -0.40 | 0.01 | 0.00 | 0.00 | -0.38 | 0.01 |
| S04 | -0.20 | 0.01 | 0.00 | 0.00 | -0.19 | 0.01 |
| S05 | -3.29 | 0.16 | 0.00 | 0.00 | -2.94 | 0.14 |
| S06 | -3.58 | 0.18 | 0.00 | 0.00 | -3.38 | 0.17 |
| S08 | -2.83 | 0.12 | 0.00 | 0.00 | -2.71 | 0.11 |
| Average | -1.92 | 0.08 | 0.00 | 0.00 | -1.80 | 0.08 |

| Seq. | Complexity 3D-ATM 5.1r2 WPOFF + ALC |  |
| --- | --- | --- |
|  | Encoder Time | Decoder Time |
| S01 |  | 101.19 |
| S02 |  | 101.87 |
| S03 |  | 101.05 |
| S04 |  | 100.91 |
| S05 |  | 100.57 |
| S06 |  | 100.57 |
| S08 |  | 101.82 |
| Average |  | 101.14 |

| Reference 3D-ATM 5.1r2 WPOFF vs. 3D-ATM 5.1r2 WPOFF + ALC ALC Configuration: TemplateSize: 1 Search Zone: +-6/+-20 QPel Search Type: Full Search ( Int-PelSub-PelQ_Pel) Cut_TH: 4 TH: 30 Metric Type: MR_SAD4x4_DC |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Seq. | Texture Coding |  | Depth Coding |  | Total Coding ( coded PSNR) |  |
|  | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB |
| S01 | -2.51 | 0.08 | 0.00 | 0.00 | -2.34 | 0.08 |
| S02 | -0.59 | 0.02 | 0.00 | 0.00 | -0.61 | 0.02 |
| S03 | -0.39 | 0.01 | 0.00 | 0.00 | -0.37 | 0.01 |
| S04 | -0.18 | 0.01 | 0.00 | 0.00 | -0.17 | 0.01 |
| S05 | -3.26 | 0.16 | 0.00 | 0.00 | -2.91 | 0.13 |
| S06 | -3.57 | 0.18 | 0.00 | 0.00 | -3.37 | 0.16 |
| S08 | -2.78 | 0.12 | 0.00 | 0.00 | -2.67 | 0.11 |
| Average | -1.90 | 0.08 | 0.00 | 0.00 | -1.78 | 0.08 |

| Seq. | Complexity 3D-ATM 5.1r2 WPOFF + ALC |  |
| --- | --- | --- |
|  | Encoder Time | Decoder Time |
| S01 |  | 100.78 |
| S02 |  | 101.58 |
| S03 |  | 101.19 |
| S04 |  | 101.00 |
| S05 |  | 100.08 |
| S06 |  | 100.31 |
| S08 |  | 100.55 |
| Average |  | 100.78 |