| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0032 |
| --- | --- |

| Title: | AHG12: Proposal on the interlace texture plus interlace depth conformance stream |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Chieh Lin Fang-Chu Chen | Tel: Email: | +886-3-5914782 fcchen@itri.org.tw |
| Source: | Industrial Technology Research Institute |  |  |

_____________________________

# Abstract

This document proposes a conformance test stream related to interlace texture plus interlace depth for MV+D extension of AVC.

# Introduction

This document proposes an interlace test stream for MV+D extension of AVC. The current working draft JCT3V-A1001 [1] for MVC extension allows interlace depth map coding. However, there are currently no official interlace texture plus interlace depth test sequence published by MPEG. With this we would like to propose an interlace texture plus interlace depth stream for the conformance test.

# Interlace conformance test stream

In MPEG 100th meeting in Geneva, ITRI contributed a document M25069 [2] for generating a sequence with interlace texture and interlace depth from a progressive test sequence. The test sequence being used in the contribution is Undo_Dancer.

A sequence with true interlace texture and interlace depth implies that, for both its texture and depth, their top field and bottom field are captured at different time (for example 1/60 second apart). For this, we extract the even lines (line 0, line 2, line 4…) from the even frames (frame 0, frame 2, frame 4….) of the progressive sequence to form the top field; and the odd lines (line 1, line 3, line 5….) from the odd frames (frame 1, frame 3, frame 5….) to form the bottom field. This procedure is performed for both texture and depth. This results in an interlace texture and interlace depth sequence at a frame rate half of that of the original progressive sequence. For example, the original 25 frames per second of the original progressive sequence are now 25 fields per second, which is equivalent to 12.5 frames per second.

The interlace sequence is then encoded by the modified JMVC software adopted in MPEG 101st meeting[3].We would like to propose the encoded stream as one of the conformance streams for MV+D extension of AVC to test the decoder’s ability to decode interlace texture plus interlace depth stream.

# Conclusion

We recommend that the AHG to add an interlace texture plus interlace depth conformance stream for the conformance test related to the MV+D extensions of AVC.

# Reference

- T. Suzuki, M. Hannuksela, Y. Chen, S. Hattori, G. Sullivan," MVC Extension for Inclusion of Depth Maps Draft Text 4", JCT3V-A1001/ M26359, JCT meeting, Stockholm, July 2012.

- Fang-Chu Chen, Ching-Chieh Lin,” 3DV AVC based Interlace Multiview Plus Depth Coding (revised version 2)”, ISO/IEC JTC1/SC29/WG11 M25069, Geneva, Switzerland, April 2012.

- C.-C. Lin, F.-C. Chen, W.-H. Chung, “3DV technical input: JMVC software integration of coding interlace texture with interlace depth,” JCT3V-A0144/M26157, JCT Meeting, Stockholm, Sweden, July 2012.

# Patent rights declaration(s)

Industrial Technology Research Institute may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).