| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0033r1 |
| --- | --- |

| Title: | AHG7 3D-AVC: Loss Detection of Depth Parameter Sets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

3D-AVC (JCT3V-A1002) uses depth parameter sets to convey information for the closest and farthest depth as well as parameters related to view synthesis prediction. While parameter sets enable loss-resilient transmission of parameters, for example through retransmission possibility, it is asserted in this contribution that 3D-AVC includes no means to detect a loss of a depth parameter set. More specifically, if the depth_parameter_set_id value of the lost depth parameter set NAL unit has been used previously by another depth parameter set NAL unit, it is claimed that in such a case the decoder would continue decoding using the previously received depth parameter set NAL unit, which would be likely to result into clearly visible artifacts. This contribution proposes a parameter set loss detection approach based on constraints on depth_parameter_set_id value assignment.

Revision 1 of the contribution contained one change in the abstract only.

# Problem Setting

If the depth_parameter_set_id value of the lost depth parameter set NAL unit has been used previously by another depth parameter set NAL unit, it is claimed that in such a case the decoder would continue decoding using the previously received depth parameter set NAL unit, which would be likely to result into clearly visible artifacts.

There are currently no means to detect a loss of a parameter set when the lost parameter set re-used a parameter set ID value that had already been used earlier by another parameter set. Consequently, any reference to the parameter set ID value used in a lost parameter set NAL unit points to the previous parameter set NAL unit using the same ID value. Therefore wrong syntax element values would be used in the decoding process.

# Earlier Approaches

One option to overcome the loss resilience problem presented in the previous paragraph could be to avoid re-using of ID values in parameter set NAL units. However, this could potentially lead to a need for requiring decoders to support a large value range for ID values and hence increased memory consumption, as decoders must be prepared for storing a parameter set for each value in the allowed parameter set ID value range.

A parameter set checksum method was proposed in JVT-S037. The difference between the checksum method and this proposal can be summarized as follows:

- The parameter set checksum SEI message is an optional mechanism in both encoding and decoding ends, and it might end up not being used often as both ends have to support it. In our opinion, it would be good to have basic loss resilience tools mandatory, similarly as with the maximum POC difference constraint or reference picture sets. We think that the proposed constraints on parameter set ID usage provide basic loss resilience practically with no implementation or rate-distortion cost.

- On the other hand, the parameter set checksum SEI message has benefits that the proposed parameter set ID constraints cannot provide, such as:

  - No parameter set NAL unit loss can be detected with the proposed parameter set ID constraint before the parameter set ID of the lost parameter set NAL unit is referred from a coded slice NAL unit. In the parameter set checksum SEI, also those parameter sets that have already been transmitted but not yet activated can be included. Consequently, retransmission can be requested prior to the activation of parameter sets, for example.

  - The parameter set checksum SEI message can list only those parameter sets that may be activated in the future and omit other parameter sets. Hence, particularly software decoders could use the parameter set checksum SEI message to free dynamically allocated memory for parameter sets that won’t be activated subsequently in the bitstream.

# Proposal

It is proposed that the encoder specifies the value range of depth_parameter_set_id values with the max_dps_id syntax element within the sequence parameter set. In other words, the value of depth_parameter_set_id included in the depth parameter set RBSP shall be in the range of 1 to max_dps_id, inclusive. In 3D-AVC, value 0 of depth_parameter_set_id is inferred to refer to the depth parameters provided in the active sequence parameter set.

The encoder also specifies a range of depth_parameter_set_id values that are considered “used” and indicates that range to the decoder in max_dps_id_diff. The range is relative to the greatest depth_parameter_set_id (in modulo sliding-window arithmetic) among activated depth parameter set NAL units and hence specifies a kind of a sliding window of valid depth_parameter_set_id values. Depth parameter set NAL units that have an depth_parameter_set_id value outside the sliding-window range are considered “unused” and a new depth parameter set NAL unit with the same depth_parameter_set_id value may be transmitted. Depth parameter set NAL units having an depth_parameter_set_id value in the “used” range must repeat the content of the previous depth parameter set NAL unit of the same depth_parameter_set_id value. It is recommended that encoders increment the depth_parameter_set_id value of activated depth parameter sets by 1. As depth_parameter_set_id values may wrap over, modulo arithmetic is used in determining the depth_parameter_set_id values within the sliding-window range. The depth_parameter_set_id constraint is “reset” at IDR access units.

Thanks to the controlled marking which depth_parameter_set_id values can be reused for new depth parameter set NAL units, the above-mentioned error resilience problem is solved.

# Specification text

The proposed specification text changes are indicated with green highlights on top of the 3D-AVC specification text ( JCT3V-A1002).

## Sequence parameter set 3DVC extension

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| for( i = 0; i <= num_level_values_signalled_minus1; i++ ) |  |  |
| for( j = 0; j <= num_applicable_ops_minus1[ i ]; j++ ) |  |  |
| applicable_op_num_depth_views_minus1[ i ][ j ] |  | ue(v) |
| depth_info_present_flag | 0 | u(1) |
| if( depth_info_present_flag ) { |  |  |
| 3dv_acquisition_idc | 0 | ue(v) |
| if( 3dv_acquisition_idc != 1 ) { |  |  |
| max_dps_id | 0 | ue(v) |
| max_dps_id_diff | 0 | ue(v) |
| } |  |  |
| … |  |  |

max_dps_id + 1 specifies the maximum allowed depth_range_parameter_set_id value.

max_dps_id_diff specifies the value range of depth_range_parameter_set_id values marked as “used”. max_dps_id_diff * 2 shall be less than max_dps_id.

## Constraint on the use of dps_id in the slice header

- J.7.4.1.2.1	Order of 3DVC sequence parameter set RBSPs and picture parameter set RBSPs and their activation

- The specification of subclause I.7.4.1.2.1 applies.

- In addition, the following applies for the activiation of depth parameter set.

- A depth parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more texture view or depth view components of one or more coded pictures.

- Each depth parameter set RBSP is initially considered not active at the start of the operation of the decoding process and the variable MaxUsedDpsId is initially set equal to “no value” and the variable UsedDpsIdSet is initially an empty set of values. At most one depth parameter set RBSP is considered as the active depth parameter set RBSP at any given moment during the operation of the decoding process, and when any particular depth parameter set RBSP becomes the active depth parameter set RBSP, the previously-active depth parameter set RBSP (if any) is deactivated.

- When the current access unit is an IDR access unit, MaxUsedDpsId is set equal to “no value”, UsedDpsIdSet is an empty set of values, and all depth range parameter set RBSPs included in the bitstream or made available to the decoding process through external means prior to the access unit containing the IDR picture are marked unavailable to the decoding process.

- When MaxUsedDpsId is not equal to “no value”, the value of dps_id of the slice header is constrained and the variable updateMaxUsedDpsIdFlag is set as follows.

- If dps_id is equal to 0, the variable updateMaxUsedDpsIdFlag is set equal to 0.

- Otherwise, the variable zeroBasedDpsId is equal to dps_id – 1 and dps_id is constrained so that zeroBasedDpsId fulfills the following:

  - If MaxUsedDpsId >= max_dps_id_diff and MaxUsedDpsId <= max_dps_id – max_dps_id_diff, zeroBasedDpsId shall be in the range of MaxUsedDpsId – max_dps_id_diff to MaxUsedDpsId + max_dps_id_diff, inclusive. The variable updateMaxUsedDpsIdFlag is set equal to ( zeroBasedDpsId > MaxUsedDpsId ).

  - Otherwise, if MaxUsedDpsId < max_dps_id_diff, zeroBasedDpsId shall be either in the range of 0 to MaxUsedDpsId + max_dps_id_diff, inclusive, or in the range of max_dps_id – ( max_dps_id_diff – MaxUsedDpsId – 1 ) to max_dps_id, inclusive. The updateMaxUsedDpsIdFlag is set equal to ( zeroBasedDpsId > MaxUsedDpsId  &&  zeroBasedDpsId < max_dps_id – ( max_dps_id_diff – MaxUsedDpsId – 1 ) ).

  - Otherwise (MaxUsedDpsId > max_dps_id – max_dps_id_diff), zeroBasedDpsId shall be either in the range of 0 to max_dps_id_diff – ( max_dps_id – MaxUsedDpsId ) – 1, inclusive, or in the range of MaxUsedDpsId – max_dps_id_diff to max_dps_id, inclusive. The updateMaxUsedDpsIdFlag is set equal to ( zeroBasedDpsId > MaxUsedDpsId  ||  zeroBasedDpsId < max_dps_id_diff – ( max_dps_id – MaxUsedDpsId ) ).

- When a depth parameter set RBSP (with a particular value of depth_parameter_set_id equal to activeDpsId) is not the active depth parameter set RBSP and it is referred to by a coded slice NAL unit (when dps_id is present in a slice header), it is activated. This depth parameter set RBSP is called the active depth parameter set RBSP until it is deactivated when another depth parameter set RBSP becomes the active depth parameter set RBSP. A depth parameter set RBSP, with that particular value of depth_parameter_set_id, shall be available to the decoding process prior to its activation. When a depth parameter set RBSP is activated, the same depth parameter set RBSP shall remain active for subsequent coded slice NAL units of the same access unit.

- When updateMaxUsedDpsIdFlag is equal to 1, depth range parameter set RBSPs are marked as unavailable to decoding process as follows.

- prevMinUsedDpsId = MaxUsedDpsId – max_dps_id_diff
if( prevMinUsedDpsId < 0 )
	prevMinUsedDpsId += max_dps_id
minUsedDpsId = dps_id – 1 – max_dps_id_diff
if( minUsedDpsId < 0 )
	minUsedDpsId += max_dps_id
i = prevMinUsedDpsId
do {
	Mark depth range parameter set RBSP with depth_range_parameter_set_id equal to i + 1, 
	if present, as unavailable to decoding process.
	i = ( i + 1 ) % ( max_dps_id + 1 )
} while( i != minUsedDpsId )

- When updateMaxUsedDpsIdFlag is equal to 1 or MaxUsedDpsId is equal to “no value”, the following applies.

- MaxUsedDpsId is set equal to dps_id – 1.

- If MaxUsedDpsId is greater than or equal to max_dps_id_diff, UsedDpsIdSet is set to the values in the range of MaxUsedDpsId – max_dps_id_diff to MaxUsedDpsId, inclusive.

- Otherwise (MaxUsedDpsId is smaller than max_dps_id_diff), UsedDpsIdSet is set to the values in the range of 0 to MaxUsedDpsId, inclusive, and in the range of max_dps_id – ( max_dps_id_diff – MaxUsedDpsId ) to max_dps_id, inclusive.

- Any depth parameter set RBSP included in the bitstream or made available to the decoding process through external means and having depth_parameter_set_id equal to any value included in UsedDpsIdSet + 1 shall have the same content as the previous depth parameter set RBSP included in the bitstream or made available to the decoding process through external means having the same depth_range_parameter_id value.

- If any depth parameter set RBSP is present that is never activated in the bitstream (i.e., it never becomes the active depth parameter set), its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).