| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0037 |
| --- | --- |

| Title: | MV-HEVC: Vertical length restriction of inter-view vector for HEVC simple 3D extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Yoshitomo Takahashi Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com yoshitomo.takahashi@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes vertical length restriction of inter-view vector in order to reduce decoder complexity. It is believed that it makes decoder implementation easier regarding motion compensation process without any significant coding performance loss because most 3D sequences are captured by horizontally settled cameras and are rectified before encoder input. It is proposed to define as profile restriction for HEVC simple 3D extension.

It is also asserted that the proposed restriction enables low latency decode. With the restriction, decoders can start to decode a depending view picture on decoding the corresponding base view picture. Thus, the proposal suggests a parallelization tool for simple 3D.

It is reported that experimental results with the restriction value 8 show 0.1% and 0.01% BD-rate increase in the average of the common test condition sequences and of additionally tested commercial stereo contents, respectively.

# Introduction

According to the current draft or HTM4.0.1, “Stereo profile”, which is not defined yet, is supposed to require more than twice decoder complexity compared to Main profile. Because it requires handling inter-view prediction in addition to two base and dependent view decoding.

Inter-view prediction is an efficient coding tool for compression performance. The decoding process is basically same as motion compensation process defined in Main profile. One difference is that the inter-view prediction uses the decoded picture of the same POC as input. In decoder point of view, this difference might be critical. For example, it is difficult to estimate the additional bandwidth for the inter-view prediction since the vector can point any part of the reference view. The possible area referred by inter-view vector is shown in Figure 1. One can see that limiting the vector length makes implementation (e.g. cache design) easier.

Figure 1 Vertical length restriction of inter-view vector (Stereo case)

Low delay decode is another aspect. It is desirable feature even for 3D video codec in most applications.  See Figure 2. In HTM4.0.1, the delay to display the stereo views is at least 1 frame decode time in addition to re-order delay in time direction. This is due to the flexibility of the inter-view vector length. It is noted that the current draft supports NumLCUinHeight LCU lines delay in another expression of the 1 frame, where

NumLCUinHeight = PictureHeight / MaxCUHeight		(1-1)

When a HD frame (1920x1088 pixel) is coded with LCU size (=MaxCUHeight) equal to 64, the NumLCUinHeight is 17. By limiting the vertical vector length, the delay can be reduced much less than NumLCUinHeight since it enables to start dependent view decode in the middle of the base view decoding.

Figure 2 Low delay decode of dependent view picture

Taking these considerations into accounts, this contribution proposes to introduce the inter-view vector length restriction for the vertical component.

Regarding to the input image characteristics, most 3D sequences are captured with horizontally settled cameras. In addition, they are usually rectified before encoder input for comfortable 3D viewing. Thus, inter-view vectors have almost zero value for the vertical component in such cases. Therefore it is believed the vertical length restriction is reasonable.

# Proposal

It is proposed to define the maximum vertical length for inter-view vectors as a profile restriction. As a starting point, the alternative two methods are proposed.

## Method1

The following restriction is added in the annex. This text is for Stereo profile. In Method1, the restricting value is fixed.

- Profiles and levels

Bitstreams conforming to the Stereo profile at a specified tier and level shall obey the following constraints:

- In bitstreams conforming to the Stereo profile, the absolute value of vertical lengh of inter-view vectors in a dependent view is less than or equal to max_interview_vector_v_length in units of luma samples.

## Method2

In SPS (or VPS), the following syntax and semantics of the restriction are added. Method2 allows encoder to signal more precise restriction according to the characteristics of the coded sequence. And smaller restricting value enables lower decode delay for decoders.

### Syntax

if ( profile_idc == “Multi-view support Profile” ) {

interview_vector_v_restrict_flag				u(1)

if ( interview_vector_v_restrict_flag  == 1 ) {

max_interview_vector_v_length_div4			u(4)

}

}

### Semantics

interview_vector_v_restrict_flag equal to 1 represents vertical component of inter-view vectors, which are used in the dependent view pictures, are equal to or less than the value max_interview_vector_v_length in units of luma samples. If not present, the value shall be inferred to be 0. If profile_idc is equal to “Stereo profile”, the value shall be set equal to 1.

max_interview_vector_v_length_div4 multiplied by 4 represents the maximum value of vertical component of the inter-view vectors in units of luma samples. If not present, the value shall be inferred to be infinite value to represent that there is no restriction for inter-view vector length.

# Discussion

In Method1, the value max_interview_vector_v_length in the restriction is better to be discussed in JCT-3V. This contribution proposes max_interview_vector_v_length = 8 as a starting point.

Figure 3 shows an example of the length restriction of inter-view vector. Suppose an inter-view vector V = (Vx, Vy). When the following condition (2-1) is satisfied, decoders can start to decode dependent view with 2 LCU lines delay from the base view.

|Vy| < LCUsize – 8 	(2-1)

The bottom 4 lines of a LCU cannot be used yet when the LCU line is decoded because in-loop filters are not processed. In addition, the additional 4 bottom lines of the LCU cannot be referred since they use pixels after in-loop processing for interpolation filter input. Due to the loop filter of the base view, 2 LCU lines delay is necessary even if the vertical component of the inter-view vector is zero.

It is noted Vy is enough instead of Abs(Vy) for low delay decode since upper part of the base view is already decoded. For the low complexity decode, Abs(Vy) is used.

Figure 3 Vertical length restrictions for inter-view vector

Since the minimum size of LCU is 16, max_interview_vector_v_length = 8 is the minimum value.

When max_interview_vector_v_length = 16, 3 lines delay is necessary for LCUsize = 16 and 2 lines delay for LCUsize = 32 or 64. Figure4 shows the necessary LCU line delay due to the restriction value X and the LCU size.

Figure 4 LCU line delay and the restriction value

It is noted that the restriction may not contribute to reduce the decode delay when slice or tile partitioning is used. However, such slice partitioning may cause coding efficiency loss at the same time.

# Experimental results

The following results show the coding performance when vertical motion search range of inter-view vector X is set to 8 or 16 by changing encoder code. HTM4.0.1 is used as base. Regarding to horizontal search range of inter-view vectors and horizontal and vertical search range of inter vectors, the default parameter 96 is used. Other condition is same as common test condition.

## Common test condition sequence

Experimental results of the common test condition with X=8 and 16 are shown in Table 4-1 and 4-2, respectively.

Table 4-1 BD-rate on X=8

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Kendo | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Newspapercc | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| GhostTownFly | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| PoznanHall2 | 0.0% | 0.9% | 0.8% | 0.4% | 0.3% | 0.3% |
| PoznanStreet | 0.0% | -0.3% | 0.1% | 0.0% | 0.0% | 0.0% |
| UndoDancer | 0.0% | 0.3% | 0.2% | 0.1% | 0.2% | 0.2% |
| 1024x768 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% |
| average | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |

Table 4-2 BD-rate on X=16

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Kendo | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Newspapercc | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| GhostTownFly | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |
| PoznanHall2 | 0.0% | 0.7% | 0.1% | 0.2% | 0.2% | 0.2% |
| PoznanStreet | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| UndoDancer | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

## Additional test sequence

Ten internal stereo sequences are additionally tested. Table 4-3 shows the detail of the sequences. They are all lossless YUV images clipped from professionally captured contents. One or two different characteristic scenes are selected from the each content. Table 4-4 shows experimental results for X = 8 or 16.

Table 4-3 Specification of the additional sequences

| Image format | 1920 x 1088 pixels, 4:2:0, 8 bps |
| --- | --- |
| Frame rate | 30 fps |
| Number of views | 2 |
| Depth image | None |
| Frames to be encoded | 300 (10sec.) |
| Sequence | Music Live : 2 seq. |
|  | Sports (Soccer) : 1 seq. |
|  | Aquarium : 1 seq. |
|  | CG animation A : 2 seq. |
|  | CG animation B : 2 seq. |
|  | CG animation C : 2 seq. |

Table 4-4 BD-Rate of additional sequences

|  | X = 8 |  |  | X = 16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Base [%] | Dep. [%] | Total [%] | Base [%] | Dep. [%] | Total [%] |
| MusicLive_1 | 0.00 | 0.04 | 0.02 | 0.00 | -0.03 | 0.00 |
| MusicLive_2 | 0.00 | 0.25 | 0.09 | 0.00 | -0.04 | 0.00 |
| Soccer | 0.00 | -0.16 | -0.06 | 0.00 | -0.06 | -0.02 |
| Aquarium | 0.00 | -0.16 | -0.06 | 0.00 | -0.18 | -0.07 |
| CG_A_1 | 0.00 | 0.00 | 0.02 | 0.00 | 0.06 | 0.02 |
| CG_A_2 | 0.00 | -0.08 | 0.01 | 0.00 | -0.08 | 0.01 |
| CG_B_1 | 0.00 | -0.07 | 0.01 | 0.00 | -0.21 | -0.02 |
| CG_B_2 | 0.00 | 0.05 | -0.01 | 0.00 | -0.10 | -0.01 |
| CG_C_1 | 0.00 | 0.27 | 0.04 | 0.00 | 0.05 | 0.01 |
| CG_C_2 | 0.00 | 0.35 | 0.08 | 0.00 | -0.04 | -0.01 |
| avearage | 0.00 | 0.05 | 0.01 | 0.00 | -0.06 | -0.01 |

# Conclusion

It is believed that the vertical length restriction of inter-view vector introduce some decoder-side merit with negligible coding performance loss in most cases. Especially, it is asserted that the parallelization aspect is important to be discussed, which is not discussed in 3D. It is encouraged that JCT-3V considers the issue and introduces the restriction in future profile of HEVC simple 3D extension.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).