| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0038 |
| --- | --- |

| Title: | 3D-CE5.h related: MV memory reduction on motion/mode parameter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitomo Takahashi Sally Hattori Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 yoshitomo.takahashi@jp.sony.com Shinobu.Hattori@jp.sony.com ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

According to HTM-4.0.1, motion/mode parameter prediction (MPP) can refer to all MVs of inter-view reference pictures since the numbers of MVs are not compressed. On the other hand, temporal motion vector prediction (TMVP) can refer to only a MV of the first 4x4 block in each 16x16 block since the numbers of MVs are compressed. Thus it is asserted that MPP requires too much MV memory when compared with TMVP.

This contribution proposes MV memory reduction on MPP. It is suggested that the numbers of MV are compressed right after decoding a picture of each view. Two approaches with different compression ratios of MVs are proposed. The first approach can achieve higher compression ratio of MVs than the second one, yet there is more coding loss.

It is reported that experimental results show 0.6% BD-rate increase in the first approach and 0.1% BD-rate increase in the second approach. We recommend adopting one of them to the current HTM.

# Introduction

HTM-4.0.1 [1] includes both MPP and TMVP tools. These tools can add the MVs of other pictures to the candidate list on AMVP and merge mode. It is supposed that MPP adds the MVs of inter-view reference pictures and TMVP adds the MVs of inter reference pictures. It is essential that these MVs are stored in the memory until they are not used any more.

It is noted that there is a difference between MPP and TMVP about referring to the stored MVs. In MPP, all MVs included in the reference picture can be used since the numbers of MVs are not compressed. Thus it is necessary that all MVs are stored in the memory. On the other hand, in TMVP, only a MV of the first 4x4 block in each 16x16 block can be used since the numbers of MVs are compressed. Thus it is asserted that MPP requires too much MV memory when compared with TMVP.

Figure 1 shows the flow of MV compression in HTM-4.0.1 decoder. In this figure, it is assumed that the View1 refers to View0 and the View2 refers to both View0 and View1. The numbers of MVs are compressed after decoding the pictures of all views. Thus the uncompressed MVs can be used on MPP, whereas all such MVs must be stored in the memory. 

In HTM-4.0.1, which is based on HM-6.0, the sixteen MVs occur at most in a 16x16 block as shown in figure 1. MPP must store all sixteen MVs in the memory, whereas TMVP only has to store a MV of the first 4x4 block in a 16x16 block. It follows that MPP requires sixteen times as much MV memory as TMVP in this case.

Figure 1: MV compression on current method

# Proposal

It is proposed to reduce the MV memory on MPP. It is suggested that the numbers of MV are compressed right after decoding a picture of each view. 

Two approaches with different compression ratios of MVs are proposed. The first approach can achieve higher compression ratio of MVs than the second one, yet there is more coding loss.

## Approach 1

It is proposed that only the MV of the first 4x4 block in each 16x16 block is stored right after decoding a picture of each view. 

Figure 2 shows the flow of MV compression on this approach. Compared with the current method, the timing of MV compression is changed. The numbers of MVs are compressed right after decoding a picture of each view. There is no difference about the way of MV compression between both methods, which only the MV of the first 4x4 block in each 16x16 block is stored. 

Since MPP only refers to MVs which can be used in TMVP, it is beneficial that there is no increase of the MV memory by introducing MPP tool. In HTM-4.0.1, it is possible to reduce the MV memory to one-sixteenth at most compared with the current method in the case of the 4x4 PU.

Figure 2 : MV compression on approach 1

## Approach 2

The numbers of MVs are compressed in two parts, which are the time after decoding a picture of each view and the time after decoding the pictures of all views. In the first part, only the MVs of the block where the top-left sample is a multiple of 8 are stored. In the second part, the only MVs of the first 4x4 block in each 16x16 block is stored.

Figure 3 shows the flow of MV compression on this approach. In the same way as Approach 1, the numbers of MVs are compressed right after decoding a picture of each view, but there is a difference about the MV compression ratio between two approaches. At this time, only the MVs of the block where the top-left sample is a multiple of 8 are stored. Thus MPP can refer to more MVs than TMVP. After decoding the pictures of all views, the numbers of MVs are further compressed. Finally, the only MVs of the first 4x4 block in each 16x16 block are stored.

Compared with the current method, it is beneficial to reduce the MV memory with no significant coding loss. In HTM-4.0.1, it is possible to reduce the MV memory to the one-fourth at most compared with the current method in the case of the 4x4 PU.

It is possible to keep the compatibility on base view since the MVs stored after decoding the pictures of all views are not changed.

Figure 3 : MV compression on approach 2

# Experimental results

## Compression performance

The following results show the coding performance in comparison with anchor HTM-4.0.1. The proposal is implemented on the same software as anchor. The simulation was performed based on the common test condition [2]. 

Table 1 and Table 2 show the coding result of the Approach1 and Approach2, respectively. As shown in Table 1, there are 1.3% and 1.4% BD-rate increase in video1 and video2 respectively and 0.6% BD-rate increase in the total of coding views and synthesized views. As shown in Table 2, there are 0.0% and 0.3% BD-rate increase in video1 and video2 respectively and 0.1% BD-rate increase in the total of coding views and synthesized views.

Table 1 : BD-rate on Approach1

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 2.0% | 2.0% | 0.8% | 0.9% | 0.9% |
| Kendo | 0.0% | 1.4% | 1.9% | 0.7% | 0.8% | 0.8% |
| Newspapercc | 0.0% | 1.3% | 1.2% | 0.5% | 0.7% | 0.7% |
| GhostTownFly | 0.0% | 1.3% | 1.3% | 0.3% | 0.4% | 0.4% |
| PoznanHall2 | 0.0% | 0.7% | 0.6% | 0.3% | 0.2% | 0.2% |
| PoznanStreet | 0.0% | 0.4% | 0.6% | 0.2% | 0.4% | 0.4% |
| UndoDancer | 0.0% | 1.8% | 2.2% | 0.6% | 1.0% | 1.0% |
| 1024x768 | 0.0% | 1.6% | 1.7% | 0.7% | 0.8% | 0.8% |
| 1920x1088 | 0.0% | 1.1% | 1.2% | 0.4% | 0.5% | 0.5% |
| average | 0.0% | 1.3% | 1.4% | 0.5% | 0.6% | 0.6% |

Table 2 : BD-rate on Approach2

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Kendo | 0.0% | 0.0% | 0.2% | 0.0% | 0.2% | 0.2% |
| Newspapercc | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| GhostTownFly | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
| PoznanHall2 | 0.0% | 0.1% | 0.8% | 0.2% | 0.1% | 0.1% |
| PoznanStreet | 0.0% | -0.2% | 0.3% | 0.0% | 0.1% | 0.1% |
| UndoDancer | 0.0% | 0.1% | 0.3% | 0.1% | 0.1% | 0.1% |
| 1024x768 | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| 1920x1088 | 0.0% | 0.0% | 0.4% | 0.1% | 0.1% | 0.1% |
| average | 0.0% | 0.0% | 0.3% | 0.1% | 0.1% | 0.1% |

According to HM8.1, which is the newest HM software, the minimum PU size is not 4x4 but 4x8 or 8x4 for inter. Thus it is expected that the MV compression ration which can be achieved by the proposed method decrease, whereas the loss of BD-rate also decreases.

## Complexity analysis

The following results show the simulation time in comparison with anchor HTM-4.0.1. Table 3 and Table 4 show the coding result of the Approach1 and Approach2, respectively. As shown in each table, the encoding and decoding time is no significant difference between the proposal and the anchor.

Table 3 : Encoding and decoding time on Approach1

|  | enc time | dec time |
| --- | --- | --- |
| Balloons | 98.8% | 99.1% |
| Kendo | 98.9% | 95.8% |
| Newspapercc | 100.9% | 94.3% |
| GhostTownFly | 97.7% | 97.4% |
| PoznanHall2 | 94.1% | 99.1% |
| PoznanStreet | 99.5% | 97.3% |
| UndoDancer | 99.2% | 97.5% |
| 1024x768 | 99.5% | 96.4% |
| 1920x1088 | 97.6% | 97.8% |
| average | 98.4% | 97.2% |

Table 4 : Encoding and decoding time on Approach2

|  | enc time | dec time |
| --- | --- | --- |
| Balloons | 98.7% | 100.9% |
| Kendo | 98.7% | 97.3% |
| Newspapercc | 100.8% | 96.4% |
| GhostTownFly | 99.8% | 100.3% |
| PoznanHall2 | 94.3% | 100.6% |
| PoznanStreet | 94.3% | 99.7% |
| UndoDancer | 98.9% | 99.4% |
| 1024x768 | 99.4% | 98.2% |
| 1920x1088 | 96.8% | 100.0% |
| average | 97.9% | 99.2% |

# Conclusion

This contribution proposed MV memory reduction on MPP. It was suggested that the numbers of MV are compressed right after decoding a picture of each view. Two approaches with different compression ratios of MVs were proposed. The first approach can achieve higher compression ratio of MVs than the second one, yet there is more coding loss.

It was reported that experimental results show 0.6% BD-rate increase in the first approach and 0.1% BD-rate increase in the second approach. We recommend adopting either of them to the current HTM.

# Reference

[1] ISO/IEC JTC1/SC29/WG11, “3D-HEVC Test Model 1”, Doc. JCT3V-A1005, Stockholm, SE, July 2012.
[2] ISO/IEC JTC1/SC29/WG11, “Common Test Conditions of 3DV Core Experiments”, Doc. JCT3V-A1100, Stockholm, SE, July 2012.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).