| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0039 |
| --- | --- |

| Title: | CE6.H results on simplified Wedgelet search for DMM modes 1 and 3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Merkle, Karsten Müller, Xin Zhao, Ying Chen, Li Zhang, Marta Karczewicz | Email: | philipp.merkle@hhi.fraunhofer.de, karsten.mueller@hhi.fraunhofer.de, xinzhao@qualcomm.com, cheny@qualcomm.com, lizhang@qualcomm.com |
| Source: | Fraunhofer HHI and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is a joint proposal of methods for simplification of the Wedgelet search of DMM modes 1 and 3 by combining the methods proposed in JCT3V-A0098 and JCT3V-A0105. Under common test conditions this method reduces the encoder/decoder complexity by 1.1%/2.3% and by 30.8%/22.5% for all-intra configuration with no impact on the coding efficiency.

# Introduction

The additional depth intra coding modes in 3DV-HTM, referred to as DMM, are adapted to the specific characteristics of depth maps by applying non-rectangular block partitions for approximating the signal of a depth block containing an edge. DMM modes 1 and 3 use a minimum distortion Wedgelet search for finding the best matching partition for a block. For this purpose a Wedgelet pattern look-up list is provided that contains all possible Wedgelet partition patterns for a certain block size. By searching the full list of patterns the minimum distortion Wedgelet partition is definitely found, but such a full search requires a substantial amount of processing. In this contribution we combine the two methods proposed in JCT3V-A0098 [1] and JCT3V-A0105 [2] for significantly reducing the complexity of the minimum distortion Wedgelet search. The first method, reported in JCT3V-A0098, applies to DMM mode 3 and the case that the Co-located Texture Luma Block (CTLB) is intra coded. The search is then limited to a fixed range around the intra direction of the CTLB. The second method, reported in JCT3V-A0105, applies to DMM modes 1 and 3, using a two-step search strategy by first searching with a coarse Wedgelet subset and then refining the result with full accuracy. For the joint proposal, both methods are combined as follows: for DMM mode 3 JCT3V-A0098 is applied if the CTLB is intra coded, otherwise JCT3V-A0105 is applied. For DMM mode 1 JCT3V-A0105 is always applied.

# Proposed method

## CTLB intra direction aligned search

In [1], it is proposed to utilize the intra direction of the CTLB to skip a large portion of unnecessary Wedgelet searches at the decoder. Basically, the method consists of two parts: first, for each intra direction, a Wedgelet list is generated with Wedgelet patterns sharing the similar directionality of the intra direction, second, during the Wedgelet search, only the Wedgelet patterns included in the Wedgelet list for the intra direction of CTLB are checked.

Wedgelet list generation for each intra direction. To generate the Wedgelet list, each Wedgelet pattern index idxW is first mapped to an intra direction widx by applying the following steps:

- For each intra direction i from 2 to 34, Di is calculated as Di = | Vi×(Xs-Xe) - Hi×(Ye-Ys) |, where Hi and Vi are obtained from Table 1, (Xs, Ys) and (Xe, Ye) indicate the start and end point position of the Wedgelet pattern, respectively.

- Map the Wedgelet pattern index idxW to the intra direction widx which minimizes D among all intra directions.

Table 1: Values of H and V for angular intra directions

| intra direction | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| V | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | -2 | -5 | -9 | -13 | -17 | -21 | -26 | -32 |
| intra direction | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| H | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |  |
| V | -32 | -32 | -32 | -32 | -32 | -32 | -32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |  |

Then for each intra direction pidx, all the Wedgelet patterns with mapped intra direction widx satisfying | widx - pidx | ≤ tr, are included in the Wedgelet list for pidx, where tr is set to 1 in this proposal.

Wedgelet search function. With this proposed method, the intra direction of CTLB is first derived as pidx, then only the Wedgelet patterns in the Wedgelet list of intra direction pidx is used in the Wedgelet search function, thus large portion of unnecessary Wedgelet searches are skipped.

## Coarse search with refinement step

According to [2] this method basically consists of two steps: first, a coarse Wedgelet pattern list is searched for the minimum distortion partition and second, the resulting Wedgelet partition is refined. This requires modifying the Wedgelet pattern list initialization process as well as the search function.

Wedgelet pattern list initialization. In 3DV-HTM the Wedgelet pattern lists are generated during encoder and decoder initialization. For the proposed method, a coarse subset of Wedgelet patterns marked by a flag during the list generation process. The subset contains the Wedgelets for every second start and end position (see Fig. 1, left). Furthermore, a Wedgelet node is derived for each Wedgelet of the coarse subset, containing up to eight references to refinement Wedgelets (see Fig. 1, right).

Figure 1. Coarse subset of Wedgelet separation line start/end positions (left) and Wedgelet node with refinements (right).

Search function. In 3DV-HTM the minimum distortion Wedgelet search consist of testing all Wedgelet patterns contained in the list corresponding to the block size, i.e. full search.  For the proposed method the search strategy is modified as follows: In a first step the minimum distortion Wedgelet pattern is searched for the coarse subset by iterating over the Wedgelet node list and testing the referenced pattern. In a second step the Wedgelet partition found in the first step is refined. For this purpose the minimum distortion Wedgelet pattern is searched from the up to eight refinement patterns referenced in the corresponding node.

# Experimental results

According to the CE6.H description [3] and the common test conditions [4], the proposed method is evaluated for random access (CTC) and intra-only configuration.

Table 2. Result for 3-view scenario under CTC

|  | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.04% | 0.03% | 98.7% | 96.1% | 100.8% |
| Kendo | 0.0% | 0.01% | 0.01% | 99.0% | 98.7% | 100.1% |
| Newspapercc | 0.0% | 0.05% | 0.05% | 97.8% | 97.0% | 99.5% |
| GhostTownFly | 0.0% | -0.02% | -0.02% | 99.5% | 96.7% | 98.9% |
| PoznanHall2 | 0.0% | -0.03% | -0.03% | 99.6% | 99.2% | 99.5% |
| PoznanStreet | 0.0% | 0.03% | 0.01% | 98.5% | 98.5% | 99.8% |
| UndoDancer | 0.0% | 0.09% | 0.07% | 99.1% | 97.6% | 99.4% |
| 1024x768 | 0.0% | 0.03% | 0.03% | 98.5% | 97.2% | 100.1% |
| 1920x1088 | 0.0% | 0.02% | 0.01% | 99.2% | 98.0% | 99.4% |
| average | 0.0% | 0.02% | 0.02% | 98.9% | 97.7% | 99.7% |

Table 3. Result for 3-view scenario under all-intra configuration

|  | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.07% | 0.05% | 67.4% | 80.8% | 99.9% |
| Kendo | 0.0% | 0.06% | 0.05% | 70.8% | 88.1% | 100.0% |
| Newspapercc | 0.0% | 0.18% | 0.13% | 63.1% | 67.2% | 97.5% |
| GhostTownFly | 0.0% | 0.14% | 0.10% | 73.8% | 67.9% | 98.6% |
| PoznanHall2 | 0.0% | 0.05% | 0.03% | 79.5% | 92.0% | 100.8% |
| PoznanStreet | 0.0% | 0.06% | 0.04% | 64.3% | 73.5% | 100.8% |
| UndoDancer | 0.0% | 0.10% | 0.08% | 67.3% | 76.2% | 99.0% |
| 1024x768 | 0.0% | 0.10% | 0.08% | 67.0% | 78.2% | 99.1% |
| 1920x1088 | 0.0% | 0.09% | 0.06% | 71.0% | 76.9% | 99.8% |
| average | 0.0% | 0.09% | 0.07% | 69.2% | 77.5% | 99.5% |

# Conclusion

The proposal combines two methods for simplification of the Wedgelet search of DMM modes 1 and 3. The results for the combined method show that the encoder/decoder complexity is reduced by 1.1%/2.3% for random access and by 30.8%/22.5% for all-intra configuration. The BD-rate increase is negligible, being less than 0.1% for both configurations. Therefore, we recommend the adoption of this method to the 3D-HEVC Test Model [5].

# References

- Xin Zhao, Ying Chen, Li Zhang, Marta Karczewicz, "3D-CE6.h related: Depth Modeling Mode (DMM) 3 simplification for HTM", JCT3V-A0098, Stockholm, July 2012.

- Philipp Merkle, Hunn Rhee, Karsten Müller, Thomas Wiegand, "3D-CE6.h related: Fast Wedgelet search", JCT3V-A0105, Stockholm, July 2012.

- Philipp Merkle, " Description of Core Experiment 6 (CE6) on Depth Intra Coding", JCT3V-A1106, Stockholm, July 2012.

- Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, "Common Test Conditions of 3DV Core Experiments ", JCT3V-A1106, Stockholm, July 2012.

- Gerhard Tech, Krzysztof Wegner, Ying Chen, Sehoon Yea, “3D-HEVC Test Model 1,” JCT3V-A1005, Stockholm, Sweden, July 2012.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: syntax specification

The proposed method requires the following changes (highlighted in red) to the specification in Annex G, section G.8.4.4.2.8 of JCT3V-A1005 [5]:

G.8.4.4.2.8	Specification of Intra_DepthPartition (37, 38) prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = −1..2*nT−1,

–	a variable nT specifying the transform size,

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 37 or 38.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nT−1, are derived as the following ordered steps:

- A binary partition pattern  dmmWedgeletPattern[ x, y ], with x, y =0..nT−1,  is derived from the luma samples of the collocated texture video block videoLumaSamples[ x, y ], with x, y =0..nT−1.

Given the prediction mode of the collocated texture video block videoPredMode[ 0, 0 ], the list of wedgelet patterns dmmWedgeletPatternList[ k ] for transform size nT and with k = 0..nWP entries, the wedgelet node index list dmmWedgeletNodeIndexList[ l ] for transform size nT and with l = 0..nWN entries, and the wedgelet node refinements index list dmmWedgeletNodeIndexList[ l ][m] for transform size nT and with l = 0..nWN and m=0..nWR entries, the following applies:

–	If videoPredMode [ 0, 0 ] is equal to MODE_INTRA:

The list of wedgelet patterns is dmmWedgeletList = dmmWedgeletListIntraMode[ videoPredMode[ 0, 0 ] ] for transform size nT and with k = 0..nWM entries,

For i = 0..nWM the distortion of the candidate wedgelet pattern

dmmCandidatePattern = dmmWedgeletList[ i ]  	(G-34)

is calculated by invoking the texture wedgelet distortion process as specified in subclause G. 8.4.4.2.x with videoLumaSamples[ x, y ], dmmCandidatePattern[ x, y ], and nT as inputs and candidateSad as output.

- When candidateSad is smaller than minSad the following applies:

dmmWedgeletPattern = dmmCandidatePattern, 
and minSad = candidateSad		(G-35)

–	Otherwise:

For l = 0..nWN the distortion of the candidate wedgelet pattern

dmmCandidatePattern = dmmWedgeletPatternList[ dmmWedgeletNodeIndexList[ l ] ] 	(G-36)

is calculated by invoking the texture wedgelet distortion process as specified in subclause G. 8.4.4.2.x aswith videoLumaSamples[ x, y ], dmmCandidatePattern[ x, y ], and candidatePredSamples[ x, y ], with x, y = 0..nT−1 nT as inputs and candidateSad as output.

- When candidateSad is smaller than minSad the following applies:

dmmWedgeletPattern = dmmWedgeletPatternList[ dmmWedgeletNodeIndexList[ l ] ], 
nodeIndex = l  and minSad = candidateSad		(G-37)

For m = 0..nWR the distortion of the candidate wedgelet pattern

dmmCandidatePattern = dmmWedgeletPatternList[ dmmWedgeletNodeIndexList[ nodeIndex ][m] ] 	(G-38)

is calculated by invoking the texture wedgelet distortion process as specified in subclause G. 8.4.4.2.x with videoLumaSamples[ x, y ], dmmCandidatePattern[ x, y ], and nT as inputs and candidateSad as output.

- When candidateSad is smaller than minSad the following applies:

dmmWedgeletPattern = dmmWedgeletPatternList[ dmmWedgeletNodeIndexList[ nodeIndex ][m] ] and minSad = candidateSad		(G-39)

- The depth partition value derivation and assignment process as specified in subclause Error: Reference source not found is invoked with p[ x, y ], dmmWedgeletPattern[ x, y ], and nT as inputs and the output is assigned to predSamples[ x, y ].

G.8.4.4.2.x		Specification of texture wedgelet distortion process

Inputs to this process are:

–	sample values videoLumaSamples[ x, y ], with x, y = −1..2*nT−1,

–	binary pattern dmmCandidatePattern[ x, y ], with x, y =0..nT−1,

–	a variable nT specifying the transform size,

Output of this process is:

–	distortion value candidateSad.

The SAD between constant values of the binary pattern and the video luma sample values is derived as the following ordered steps:

- For x, y = 0..nT−1 the constant partition values of the candidate wedgelet  pattern are derived as:

- If dmmCandidatePattern[ x, y ] is equal to 1, the following applies:

sumDC2 += videoLumaSamples[ x, y ] and numSamplesDC2 += 1	(G-x1)

- Otherwise, the following applies:

sumDC1 += videoLumaSamples[ x, y ] and numSamplesDC1 += 1	(G- x2)

- The predicted constant partition values are derived as follows.

- If numSamplesDC1 is equal to 0, the following applies:

dmmPartitionDC1 = 1 << ( BitDepthY  − 1 )	(G- x3)

- Otherwise, the following applies:

dmmPartitionDC1 = sumDC1 / numSamplesDC1	(G-x4)

- If numSamplesDC2 is equal to 0, the following applies:

dmmPartitionDC2 = 1 << ( BitDepthY  − 1 )	(G- x5)

- Otherwise, the following applies:

dmmPartitionDC2 = sumDC2 / numSamplesDC2	(G- x6)

- For x, y = 0..nT−1 the constant partition values are assigned to the candidate prediction samples as:

- If dmmCandidatePattern[ x, y ] is equal to 1, the following applies:

candidatePredSamples[ x, y ] = dmmPartitionDC2	(G- x7)

- Otherwise, the following applies:

candidatePredSamples[ x, y ] = dmmPartitionDC1	(G- x8)

- The candidate SAD is calculated for x, y = 0..nT−1 as:

candidateSad = Abs( candidatePredSamples[ x, y ] - videoLumaSamples[ x, y ] )	(G- x9)