| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0040 |
| --- | --- |

| Title: | Specification of stereoscopic texture sub-bitstreams in MVC+D |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anthony Vetro Mitsubishi Electric 201 Broadway, 8th Floor Cambridge, MA, 02139 USA | Tel: Email: | +1-617-621-7591 avetro@merl.com |
|  | Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Mitsubishi Electric, Qualcomm |  |  |

_____________________________

# Abstract

This document proposes that the specification of the stereoscopic texture sub-bitstream be comprised of a base view and any non-base view, if the decoding of this non-base view does not depend on any other non-base view. The current specification of the stereoscopic texture bitstream limits the non-base view of the stereoscopic to a specific value of view order index (VOIdx). It is asserted that this limitation is too restrictive and the specification should allow for the stereoscopic sub-bitstream to be formed from other views that have different values of VOIdx.

# Introduction

The current specification of a stereoscopic texture sub-bitstream as defined in subclause I.8.5.5 of [1] invokes the sub-bitstream extraction process with viewIdTargetList containing the view_id values equal to the base view and the texture view having VOIdx equal to 1. Consider a typical three view bitstream with a center view (VOIdx equal to 0) and two dependent views (VOIdx 1 and VOIdx 2) as shown in the figure below.

VOIdx 2

(Left)

VOIdx 0

(Center)

VOIdx 1

(Right)

With the current specification, it would not be permissible for the left view (VOIdx 2) and the center view (VOIdx 0) to form a valid stereoscopic texture sub-bitstream. It is suggested that the current specification is too restrictive and should be changed to allow for the more general extraction of a stereoscopic texture bitstream.

# Proposal

Replace the current text:

“A bitstream that conforms to a profile as specified in Annex I shall contain a sub-bitstream that conforms to one or more of the profiles specified in Annex H with number of views equal to 2. This stereoscopic texture bitstream is derived by invoking the sub-bitstream extraction process as specified in subclause I.8.5.3 with depthPresentFlagTarget equal to 0 and viewIdTargetList containing the view_id values of the base view and the texture view having VOIdx equal to 1 as inputs and the stereoscopic texture bitstream being the output.”

with

“A bitstream that conforms to a profile as specified in Annex I shall contain at least one sub-bitstream that conforms to one or more of the profiles specified in Annex H with number of views equal to 2. This stereoscopic texture bitstream is derived by invoking the sub-bitstream extraction process as specified in subclause I.8.5.3 with depthPresentFlagTarget equal to 0 and viewIdTargetList containing the view_id values of the base view and a non-base view, the texture of which does not depend on any other non-base view for decodingthe texture view having VOIdx equal to 1 as inputs and the stereoscopic texture bitstream being the output.

Note – The texture view with VOIdx equal to 1 is a non-base texture view that does not depend on any other view except for the base view.”

# References

[1] T. Suzuki, M. Hannuksela, Y. Chen, S. Hattori, G. Sullivan (Eds.), “MVC Extension for Inclusion of Depth Maps Draft Text 4,” JCT3V-A1001, Stockholm, Sweden, July 2012.

# Patent rights declaration(s)

Mitsubishi Electric does not have any current or pending patent rights relating to the technology described in this contribution.

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.