| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0042 |
| --- | --- |

| Title: | 3D-CE6.h related: Early Termination of PU Prediction in Depth Intra Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongwei Li Ming Li Pingwu Yingjie Hong Guoqiang Shang | Email: | li.hongwei10@zte.com.cn li.ming42@zte.com.cn ping.wu@zte.com.cn hong.yingjie@zte.com.cn shang.guoqiang@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

This contribution presents an early termination of PU prediction for depth map intra prediction coding in 3D-HTM. An early termination strategy which set threshold conditions is implemented on 3D-HTM-4.0.1 to accelerate the process of depth intra mode decision. It is reported that using the intra-only configuration for CE6, the runtime is reduced by an average of 41.1% (encoder) and 24.2% (decoder) with an average loss in BD-BitRate of 0.8%.

# Introduction

The depth intra prediction modes include HEVC intra prediction mode and DMM(Depth Model Mode). DMM are adapted to the specific characteristics of depth maps by applying non-rectangular block partitions for approximating the signal of a depth block containing an edge. There are some characteristics in DMM in depth map intra prediction. The statistical observations show:

First, for depth intra prediction, DMM occupies more coding time than HEVC intra prediction modes. The bigger the size of the block is, the longer the runtime for DMM is. The possibility which the prediction mode of the big PU becomes the final prediction mode of this PU is small.

Secondly, when the difference between maximum and minimum pixel values in a PU is less than 10, the possibility which DMM is selected as the final prediction mode is small. Even if, the selected prediction mode is DMM when the difference between maximum and minimum pixel values in a PU is less than 10, the difference between the RDO cost of DMM and the RDO cost of HEVC intra prediction mode is small.

Finally, the RDO selections show that DMM mainly is applied to a depth block containing an edge, this kind of block contains two different constant pixel regions at least.

Therefore, this contribution presents an early termination of PU prediction in depth intra for DMM method that significantly reduces the runtime of depth intra prediction.

# Proposed method

To speed up the HTM4.0.1 encoder, depth intra prediction using DMM can be early terminated through  the following algorithm:

1. When the number of pixels with a same pixel value in a row or column more than a pre-set threshold value Thd, which is related to the depth map data range and the value is given as: for 16*16 block, Thd is set to 12; for 8*8 block, Thd is set to 6; and for 4*4 block, Thd is set to 3; the pixel value of this row or column is recorded. Then, several pixel values can be obtained by calculate all the row or column in a PU. Judging whether these obtained pixel values is same.

2. Judging whether the difference between maximum and minimum pixel values in a PU is less than 10,

3. Judging whether the size of PU is more than 16*16.

If the above three judgments all are true, the depth intra prediction mode only selects HEVC intra prediction mode, but not DMM.

# Experimental results

For intra-only configuration CE6[1], the experimental results, including the complexity measured by runtime and the rate-distortion performance between the proposed method and the HTM4.0.1, are summarized in Table 1. Detailed results are included in the attachment (JCT3V-B0042_approach_1_vs_anchor_4.0.1).

Table 1. Result for 3-view scenario under intra-only configuration

|  | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 1.0% | 0.7% | 58.0% | 80.9% | 95.1% |
| Kendo | 0.0% | 1.5% | 1.0% | 62.9% | 83.4% | 94.6% |
| Newspapercc | 0.0% | 1.8% | 1.2% | 56.7% | 62.3% | 99.2% |
| GhostTownFly | 0.0% | 1.2% | 0.9% | 72.2% | 73.4% | 98.5% |
| PoznanHall2 | 0.0% | 1.7% | 1.1% | 51.7% | 92.6% | 95.2% |
| PoznanStreet | 0.0% | 0.9% | 0.6% | 61.5% | 63.4% | 99.6% |
| UndoDancer | 0.0% | 0.5% | 0.3% | 52.1% | 79.4% | 101.6% |
| 1024x768 | 0.0% | 1.4% | 0.9% | 59.1% | 74.9% | 96.3% |
| 1920x1088 | 0.0% | 1.1% | 0.7% | 58.8% | 76.5% | 98.7% |
| average | 0.0% | 1.2% | 0.8% | 58.9% | 75.8% | 97.7% |

# Conclusion

The results show that the runtime is reduced by 41.1% (encoder) and 24.2% (decoder) in intra-only configuration for CE6. For the “synthesized only” condition and the “coded&synthesized” condition, BD-rate increase 0.8% and 1.2% respectively. In conclusion the early termination of PU prediction in depth intra coding leads to a significant runtime reduction with an average loss in BD-BitRate of 0.8%.

# References

- [1]	Philipp Merkle, “Description of Core Experiment 6 (CE6) on Depth Intra Coding” ISO/IEC JTC1/SC29/WG11 document JCT2-A1106, Stockholm, July 2012.

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).