| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0045 |
| --- | --- |

| Title: | 3D-CE2.h : Results of Illumination Compensation for Inter-View Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu (hongbin.liu@lge.com) Jiwook Jung (jiwook.jung@lge.com) Jaewon Sung (jw.sung@lge.com) Jie Jia (jie.jia@lge.com) Sehoon Yea (sehoon.yea@lge.com) |  |  |
| Source: | LG Electronics |  |  |

_____________________________

# Working Draft

G 7.3.5.1 General slice header syntax

Following highlighted modifications is applied.

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if(first_slice_in_pic_flag && ViewId && !depth_flag) |  |
| ic_enable_flag | u(1) |

G 7.3.9.1 General Coding unit syntax

Following highlighted modifications is applied.

| if ( !depth_flag && ViewId && multi_view_residual_pred_flag && ResidualCbfNonZero && PredMode != MODE_INTRA) |  |
| --- | --- |
| res_pred_flag | ae(v) |
| if( ic_enable_flag && PredMode[ x0 ][ y0 ] != MODE_INTRA) { |  |
| If( ( PartMode == PART_2Nx2N && ( (inter_pred_idc[ x0 ][ y0 ] != Pred_L1 && ref_viewid_l0[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0 ] != Pred_L0 && ref_viewid_l1[ x0 ][ y0 ] != ViewId) ) ) |  |
| ¦¦ ( PartMode == PART_2NxN && ( (inter_pred_idc[ x0 ][ y0 ] != Pred_L1 && ref_viewid_l0[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0 ] != Pred_L0 && ref_viewid_l1[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0+( nCbS / 2 ) ] != Pred_L1 && ref_viewid_l0[ x0 ][ y0+( nCbS / 2 ) ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0+( nCbS / 2 ) ] != Pred_L0 && ref_viewid_l1[ x0 ][ y0+( nCbS / 2 ) ] != ViewId) ) ) |  |
| ¦¦ ( PartMode == PART_Nx2N && ( (inter_pred_idc[ x0 ][ y0 ] != Pred_L1 && ref_viewid_l0[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0 ] != Pred_L0 && ref_viewid_l1[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0+( nCbS / 2 ) ][ y0 ] != Pred_L1 && ref_viewid_l0[ x0+( nCbS / 2 ) ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0+( nCbS / 2 ) ][ y0 ] != Pred_L0 && ref_viewid_l1[ x0+( nCbS / 2 ) ][ y0 ] != ViewId) ) ) |  |
| ¦¦ ( PartMode == PART_2NxnU && ( (inter_pred_idc[ x0 ][ y0 ] != Pred_L1 && ref_viewid_l0[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0 ] != Pred_L0 && ref_viewid_l1[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0+( nCbS / 4 ) ] != Pred_L1 && ref_viewid_l0[ x0 ][ y0+( nCbS / 4 ) ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0+( nCbS / 4 ) ] != Pred_L0 && ref_viewid_l1[ x0 ][ y0+( nCbS / 4 ) ] != ViewId) ) ) |  |
| ¦¦ ( PartMode == PART_2NxnD && ( (inter_pred_idc[ x0 ][ y0 ] != Pred_L1 && ref_viewid_l0[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0 ] != Pred_L0 && ref_viewid_l1[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0+( 3*nCbS / 4 ) ] != Pred_L1 && ref_viewid_l0[ x0 ][ y0+( 3*nCbS / 4 ) ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0+( 3*nCbS / 4 ) ] != Pred_L0 && ref_viewid_l1[ x0 ][ y0+( 3*nCbS / 4 ) ] != ViewId) ) ) |  |
| ¦¦ ( PartMode == PART_nLx2N && ( (inter_pred_idc[ x0 ][ y0 ] != Pred_L1 && ref_viewid_l0[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0 ] != Pred_L0 && ref_viewid_l1[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0+( nCbS / 4 ) ][ y0 ] != Pred_L1 && ref_viewid_l0[ x0+( nCbS / 4 ) ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0+( nCbS / 4 ) ][ y0 ] != Pred_L0 && ref_viewid_l1[ x0+( nCbS / 4 ) ][ y0 ] != ViewId) ) ) |  |
| ¦¦ ( PartMode == PART_nRx2N && ( (inter_pred_idc[ x0 ][ y0 ] != Pred_L1 && ref_viewid_l0[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0 ] != Pred_L0 && ref_viewid_l1[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0+(3* nCbS / 4 ) ][ y0 ] != Pred_L1 && ref_viewid_l0[ x0+( 3*nCbS / 4 ) ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0+(3* nCbS / 4 ) ][ y0 ] != Pred_L0 && ref_viewid_l1[ x0+( 3*nCbS / 4 ) ][ y0 ] != ViewId) ) ) |  |
| ¦¦ ( PartMode == PART_NxN && ( (inter_pred_idc[ x0 ][ y0 ] != Pred_L1 && ref_viewid_l0[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0 ] != Pred_L0 && ref_viewid_l1[ x0 ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0+( nCbS / 2 ) ][ y0 ] != Pred_L1 && ref_viewid_l0[ x0+( nCbS /2 ) ][ y0 ] != ViewId) ¦¦ (inter_pred_idc[ x0+( nCbS /2 ) ][ y0 ] != Pred_L0 && ref_viewid_l1[ x0+( nCbS /2 ) ][ y0 ] != ViewId) ) ¦¦ (inter_pred_idc[ x0 ][ y0+( nCbS / 2 ) ] != Pred_L1 && ref_viewid_l0[ x0 ][ y0+( nCbS / 2 ) ] != ViewId) ¦¦ (inter_pred_idc[ x0 ][ y0+( nCbS / 2 ) ] != Pred_L0 && ref_viewid_l1[ x0 ][ y0+( nCbS / 2 ) ] != ViewId) ¦¦ (inter_pred_idc[ x0+( nCbS / 2 ) ][ y0+( nCbS / 2 ) ] != Pred_L1 && ref_viewid_l0[ x0+( nCbS / 2 ) ][ y0+( nCbS / 2 ) ] != ViewId) ¦¦ (inter_pred_idc[ x0+( nCbS / 2 ) ][ y0+( nCbS / 2 ) ] != Pred_L0 && ref_viewid_l1[ x0+( nCbS / 2 ) ][ y0+( nCbS / 2 ) ] != ViewId) ) ) |  |
| cu_ic_enable_flag | ae(v) |
| } |  |

Variable ref_viewid_l0 [ x ][ y ] refers to the view index of list 0 reference picture for the current prediction unit, and variable ref_viewid_l1 [ x ][ y ] refers to the view index of list 1 reference picture for the current prediction unit. The array indices x, y specify the location ( x, y ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

7.4.5.1 	General slice header semantics

Following highlighted modifications is applied.

first_slice_in_pic_flag indicates whether the slice is the first slice of the picture. If first_slice_in_pic_flag is equal to 1, the variables SliceCbAddrZS and SliceCtbAddrRS are both set to 0 and the decoding starts with the first coding treeblock in the picture.

ic_enable_flag indicates whether illumination compensation is used for the picture. If ic_enable_flag is equal to 0, illumination compensation is not enabled for the picture; otherwise, if ic_enable_flag is equal to 1, illumination compensation is enabled for the picture. When ic_enable_flag is not present in the bitstream, its value shall be inferred to be equal to 0.

G 7.4.9.1

Following highlighted modifications is applied.

res_pred_flag equal to 0 specifies that residual prediction is not used. res_pred_flag equal to 1 specifies that residual prediction is used. When res_pred_flag is not present in the bitstream, its value shall be inferred to be equal to 0.

cu_ic_enable_flag indicates whether illumination compensation is used for the coding unit. If it is equal to 0, illumination compensation is not used; otherwise, if cu_ic_enable_flag is equal to 1, illumination compensation is used. When cu_ic_enable_flag is not present in the bitstream, its value shall be inferred to be equal to 0.

G 8.5.2.2 Decoding process for inter prediction samples

Following highlighted modifications is applied.

8.5.2.2.3 Weighted sample prediction process

Inputs to this process are:

–	a location ( xB, yB ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

–	the width and height of this prediction block, nPbW and nPbH,

–	two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1,

–	reference indices, refIdxL0 and refIdxL1,

–	a variable cIdx specifying colour component index.

–	a bit depth of samples, bitDepth.

–	a location ( xC, yC ) specifying the top-left sample of the current coding unit relative to the top-left sample of the picture,

–	the width of current coding unit, nCbW,

–	neighboring samples of current coding unit p[ x, y ], with x = -1, y = 0..nCbW−1, and y = -1, x = 0..nCbW−1,

–	reference picture with integer pixel accuracy, refL0 and refL1.

Derive variables avaiAboveRow, avaiLeftCol that indicates availability of above row and left column neighboring samples of current coding unit as follows:

–	If above prediction unit of the first prediction unit in current coding unit is available, avaiAboveRow is set to 1; otherwise, If above prediction unit of the first prediction unit in current coding unit is unavailable, avaiAboveRow is set to 0,

–	If left prediction unit of the first prediction unit in current coding unit is available, avaiLeftCol is set to 1; otherwise, if left prediction unit of the first prediction unit in current coding unit is unavailable, avaiLeftCol is set to 0.

In P slices, if the value of predFlagL0 is equal to 1, the following applies.

–	If weighted_pred_flag is equal to 0, the array predSample of the prediction samples is derived by invoking the default weighted sample prediction process as specified in subclause Error: Reference source not found is invoked with the location ( xB, yB ), the width and height of this prediction block, nPbW and nPbH, two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1, prediction list utilization flags, predFlagL0 and predFlagL1, and the bit depth of samples, bitDepth given as input.

–	if cu_ic_enable_flag is equal to 0, the array predSample of the prediction samples is derived by invoking the default weighted sample prediction process as specified in subclause Error: Reference source not found is invoked with the location ( xB, yB ), the width and height of this prediction block, nPbW and nPbH, two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1, prediction list utilization flags, predFlagL0 and predFlagL1, and the bit depth of samples, bitDepth given as input.

–	otherwise, if cu_ic_enable_flag is equal to 1, the array predSample of the prediction samples is derived by invoking the default weighted sample prediction process as specified in subclause Error: Reference source not found3 is invoked with the location ( xB, yB ), the width and height of this prediction block, nPbW and nPbH, two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1, prediction list utilization flags, predFlagL0 and predFlagL1, and the bit depth of samples, bitDepth, location ( xC, yC ), width of current coding unit, nCbW, reference picture with integer pixel accuracy, refL0 and refL1, availability of neighboring samples of current coding unit, avaiAboveRow, avaiLeftCol and neighboring samples of current coding unit p[ x, y ] given as input.

–	Otherwise (weighted_pred_flag is equal to 1), the array predSample of the prediction samples is derived by invoking the weighted sample prediction process as specified in subclause  is invoked with the location ( xB, yB ), the width and height of this prediction block, nPbW and nPbH, two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1, prediction list utilization flags, predFlagL0 and predFlagL1, reference indices, refIdxL0 and refIdxL1, colour component index, cIdx, and the bit depth of samples, bitDepth given as input.

In B slices, if predFlagL0 or predFlagL1 is equal to 1, the following applies.

–	If weighted_bipred_flag is equal to 0, the array predSample of the prediction samples is derived by invoking the default weighted sample prediction process as specified in subclause Error: Reference source not found is invoked with the location ( xB, yB ), the width and height of this prediction block, nPbW and nPbH, two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1, prediction list utilization flags, predFlagL0 and predFlagL1, and the bit depth of samples, bitDepth given as input.

–	if cu_ic_enable_flag is equal to 0, the array predSample of the prediction samples is derived by invoking the default weighted sample prediction process as specified in subclause Error: Reference source not found is invoked with the location ( xB, yB ), the width and height of this prediction block, nPbW and nPbH, two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1, prediction list utilization flags, predFlagL0 and predFlagL1, and the bit depth of samples, bitDepth given as input.

–	otherwise, if cu_ic_enable_flag is equal to 1, the array predSample of the prediction samples is derived by invoking the default weighted sample prediction process as specified in subclause Error: Reference source not found3 is invoked with the location ( xB, yB ), the width and height of this prediction block, nPbW and nPbH, two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1, prediction list utilization flags, predFlagL0 and predFlagL1, and the bit depth of samples, bitDepth, location ( xC, yC ), width of current coding unit, nCbW, reference picture with integer pixel accuracy, refL0 and refL1, availability of neighboring samples of current coding unit, avaiAboveRow, avaiLeftCol and neighboring samples of current coding unit p[ x, y ] given as input.

–	Otherwise, if weighted_bipred_flag is equal to 1, the array predSample of the prediction samples is derived by invoking the weighted sample prediction process as specified in subclause  is invoked with the location ( xB, yB ), the width and height of this prediction block, nPbW and nPbH, two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1, prediction list utilization flags, predFlagL0 and predFlagL1, reference indices, refIdxL0 and refIdxL1, colour component index, cIdx, and the bit depth of samples, bitDepth given as input.

8.5.2.2.3.3 Illumination compensation based sample prediction process

Inputs to this process are:

–	a location ( xB, yB ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

–	the width and height of this prediction block, nPbW and nPbH,

–	two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1,

–	reference indices, refIdxL0 and refIdxL1,

–	a variable cIdx specifying colour component index.

–	a location ( xC, yC ) specifying the top-left sample of the current coding unit relative to the top-left sample of the picture,

–	the width of current coding unit, nCbW,

–	neighboring samples of current coding unit p[ x, y ], with x = -1, y = 0..nCbW−1, and y = -1, x = 0..nCbW−1,

–	availability of above row and left column neighboring samples of current coding unit, avaiAboveRow, avaiLeftCol,

–	reference picture with integer pixel accuracy, refL0 and refL1.

–	a bit depth of samples, bitDepth.

Outputs of this process are:

–	the (nPbW)x(nPbH) array predSamples of prediction sample values.

Variables shift1, shift2, offset1 and offset2 are derived as follows.

–	The variable shift1 is set equal to 14 – bitDepth and the variable shift2 is set equal to 15 – bitDepth,

–	The variable offset1 is derived as follows.

- If shift1 is greater than 0, offset1 set equal to 1 << ( shift1 − 1 ).

- Otherwise (shift1 is equal to 0), offset1 is set equal to 0.

–	The variable offset2 is set equal to 1 << ( shift2 − 1 ).

In P slices, if the value of predFlagL0 is equal to 1, the following applies.

–	(xR0, yR0) specifying location top-left sample of the referenced coding unit in list 0 relative to the top left sample of the picture is derived as:

xR0 = xC + ( mvL0x >> (2 + (cIdx ? 1 : 0)) ),                                (8-xxx)

yR0 = yC + ( mvL0y >> (2 + (cIdx ? 1 : 0)) ),                                (8-xxx)

–	derive variables avaiAboveRowRefL0, avaiLeftColRefL0 that indicates avaiablity of above row and left column neighboring samples of referenced coding unit in refL0 as follows:

- If yR0 is larger than 0, avaiAboveRowRefL0 is set to 1; otherwise, if yR0 is equal to or smaller than 0, avaiAboveRowRefL0 is set to 0,

- If xR0 is larger than 0, avaiLeftColRefL0 is set to 1; otherwise, if xR0 is equal to or smaller than 0, avaiLeftColRefL0 is set to 0.

–	store above row and left column neighboring samples of referenced coding unit of refL0 to r0[ x, y ], with x = -1, y = 0..nCbW−1, and y = -1, x = 0..nCbW−1,

–	the array predSample of the prediction samples is derived by invoking the illumination compensation based sample prediction process as specified in subclause 1.

In B slices, if predFlagL0 or predFlagL1 is equal to 1, the following applies.

–	if predFlagL0 is equal to 1,

–	(xR0, yR0) specifying location top-left sample of the referenced coding unit in list 0 relative to the top left sample of the picture is derived as:

xR0 = xC + ( mvL0x >> (2 + (cIdx ? 1 : 0)) ),                               (8-xxx)

yR0 = yC + ( mvL0y >> (2 + (cIdx ? 1 : 0)) ),                               (8-xxx)

–	derive variables avaiAboveRowRefL0, avaiLeftColRefL0 that indicates avaiablity of above row and left column neighboring samples of referenced coding unit in refL0 as follows:

- If yR0 is larger than 0, avaiAboveRowRefL0 is set to 1; otherwise, if yR0 is equal to or smaller than 0, avaiAboveRowRefL0 is set to 0,

- If xR0 is larger than 0, avaiLeftColRefL0 is set to 1; otherwise, if xR0 is equal to or smaller than 0, avaiLeftColRefL0 is set to 0.

–	store above row and left column neighboring samples of referenced coding unit of refL0 to r0[ x, y ], with x = -1, y = 0..nCbW−1, and y = -1, x = 0..nCbW−1,

–	if predFlagL1 is equal to 1,

–	(xR1, yR1) specifying location top-left sample of the referenced coding unit in list 0 relative to the top left sample of the picture is derived as:

xR1 = xC + ( mvL1x >> (2 + (cIdx ? 1 : 0)) ),                              (8-xxx)

yR1 = yC + ( mvL1y >> (2 + (cIdx ? 1 : 0)) ),                              (8-xxx)

–	derive variables avaiAboveRowRefL1, avaiLeftColRefL1 that indicates avaiablity of above row and left column neighboring samples of referenced coding unit in refL1 as follows:

- If yR1 is larger than 0, avaiAboveRowRefL1 is set to 1; otherwise, if yR1 is equal to or smaller than 0, avaiAboveRowRefL1 is set to 0,

- If xR1 is larger than 0, avaiLeftColRefL1 is set to 1; otherwise, if xR1 is equal to or smaller than 0, avaiLeftColRefL1 is set to 0.

–	store above row and left column neighboring samples of referenced coding unit of refL1 to r1[ x, y ], with x = -1, y = 0..nCbW−1, and y = -1, x = 0..nCbW−1,

–	the array predSample of the prediction samples is derived by invoking the illumination compensation based sample prediction process as specified in subclause 1.

###### 8.5.2.2.3.3.1 Illumination compensation based sample prediction process

Inputs to this process are:

–	a location ( xB, yB ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

–	the width and height of this prediction block, nPbW and nPbH,

–	two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1,

–	reference indices, refIdxL0 and refIdxL1,

–	a variable cIdx specifying colour component index.

–	a location ( xC, yC ) specifying the top-left sample of the current coding unit relative to the top-left sample of the picture,

–	the width of current coding unit, nCbW,

–	neighboring samples of current coding unit p[ x, y ], with x = -1, y = 0..nCbW−1, and y = -1, x = 0..nCbW−1,

–	availability of neighboring samples of current coding unit, avaiAboveRow, avaiLeftCol,

–	reference picture with integer pixel accuracy, refL0 and refL1.

–	availability of neighboring samples of reference coding unit, avaiAboveRowRefL0, avaiLeftColRefL0, and avaiAboveRowRefL1, avaiLeftCol RefL1,

–	if predFlagL0 is equal to 1, neighboring samples of referenced coding unit of list 0 r0[ x, y ], with x = -1, y = 0..nCbW−1, and y = -1, x = 0..nCbW−1,

–	if predFlagL1 is equal to 1, neighboring samples of referenced coding unit of list 1 r1[ x, y ], with x = -1, y = 0..nCbW−1, and y = -1, x = 0..nCbW−1,

Outputs of this process are:

–	the (nPbW)x(nPbH) array predSamples of prediction sample values.

Derive nS0 and nS1 using following two order steps, with idx in the following variables nSidx, avaiAboveRowRefLidx and avaiLeftColRefLidx replaced by 0 and 1:

1.  if avaiAboveRowRefLidx is equal to 0 or avaiAboveRow is equal to 0, nSidx is set to 1; otherwise, if avaiAboveRowRefLidx is equal to 1 and avaiAboveRow is equal to 1, nSidx is set to 0,

2.  if avaiLeftColRefLidx is equal to 0 or avaiLeftCol is equal to 0, nSidx is set to nSidx+1; otherwise, if avaiAboveRowRefLidx is equal to 1 and avaiAboveRow is equal to 1, nSidx is set to nSidx,

The parameters of linear illumination compensation model are derived as the following ordered steps. if predFlagL0 is equal to 1 and nS0 is not equal to 2, (ica0, ick0, icb0) used for list 0 is derived by replacing idx in avaiAboveRowRefLidx, avaiLeftColRefLidx, nSidx, ridx, icaidx, ickidx and icbidx with 0; otherwise, set ica0 to 1, ick0 to 0 and icb0 to 0. if predFlagL1 is equal to 1 and nS0 is not equal to 2, (ica1, ick1, icb1) used for list 1 is derived by replacing idx in avaiAboveRowRefLidx, avaiLeftColRefLidx, nSidx, ridx, icaidx, ickidx and icbidx with 1; otherwise, set a1 to 1, k1 to 0 and b1 to 0.

- Variable k3 is derived as:

k3 = Max( 0, BitDepth + Log2( nCbW >> nSidx ) − 14 )	(8-xxx)

- Variables L, C, LL, LC and k2 are derived as follows:

  - If avaiAboveRowRefLidx is equal to 1, avaiAboveRow is equal to 1, avaiLeftColRefLidx is equal to 1 and avaiLeftCol is equal to 1

L =		(8-xxx)

C =		(8-xxx)

LL = 	(8-xxx)

LC = 	(8-xxx)

  - Else, if avaiAboveRowRefLidx is equal to 1 and avaiAboveRow is equal to 1, and avaiLeftColRefLidx is equal to 0 or avaiLeftCol is equal to 0,

L =		 (8-xxx)

C =		(8-xxx)

LL = 	(8-xxx)

LC = 	(8-xxx)

  - Else, if avaiAboveRowRefLidx is equal to 0 or avaiAboveRow is equal to 0, and avaiLeftColRefLidx is equal to 1 and avaiLeftCol is equal to 1,

L =		(8-xxx)

C =		(8-xxx)

LL = 	(8-xxx)

LC = 	(8-xxx)

k2 = Log2( (2*(nCbW >> nSidx)) >> k3 )		(8-xxx)

- Variables icaidx, icbidx and ickcidx are derived as:

a1 	= ( LC << k2 ) – L*C		(8-xxx)
a2 	= ( LL << k2 ) – L*L		(8-xxx)
k1 	= Max( 0, Log2( abs( a2 ) ) − 5 ) – Max( 0, Log2( abs( a1 ) ) − 14 ) + 2	(8-xxx)
a1s	= a1 >> Max(0, Log2( abs( a1 ) ) − 14 )	(8-xxx)
a2s	= abs( a2 >> Max(0, Log2( abs( a2 ) ) − 5 ) )	(8-xxx)
a3	= a2s < 1 ? 0 : Clip3( −215, 215−1, (a1s*icDivCoeff + ( 1 << ( k1 − 1 ) ) ) >> k1 )	(8-xxx)

icaidx	= a3 >> Max( 0, Log2( abs( a3 ) ) − 6 )		(8-xxx)
ickidx	= 13 – Max( 0, Log2( abs( icaidx ) ) − 6 )		(8-xxx)

icbidx	= ( L – ( ( icaidx*C ) >> k1 ) + ( 1 << ( k2 − 1 ) ) ) >> k2	(8-xxx)

where icDivCoeff is specified in Table 8- with the input a2s.

Table 8-9 – Specification of icDivCoeff

| a2s | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| icDivCoeff | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 | 2979 | 2731 | 2521 |
| a2s | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| icDivCoeff | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 | 1560 | 1489 | 1425 | 1365 | 1311 | 1260 |
| a2s | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| icDivCoeff | 1214 | 1170 | 1130 | 1092 | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 | 840 |
| a2s | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 |
| icDivCoeff | 819 | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 | 643 | 630 |
| a2s | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |  |
| icDivCoeff | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 | 537 | 529 | 520 | 512 |  |

The prediction sample predSamples[ x ][ y ] with x = 0..(nPbW)−1 and y = 0..(nPbH)−1 are derived as follows:

–	If the predFlagL0 is equal to 1 and predFlagL1 is equal to 0, the prediction samples are derived by:

predSamples[ x ][ y ] = Clip3( 0, ( 1 << bitDepth ) − 1, ( ( ( (predSamplesL0[ x ][ y ] + offset1 ) >> shift1) * ica0) >> ick0) + icb0 ) 	(8-xxx)

–	Otherwise, if the predFlagL0 is equal to 0 and predFlagL1 is equal to 1, the final predicted sample values predSamples [ x ][ y ] are derived by

predSamples[ x ][ y ] = Clip3( 0, ( 1 << bitDepth ) − 1, ( ( ( (predSamplesL1[ x ][ y ] + offset1 ) >> shift1) * ica1) >> ick1) + icb1 ) 	(8-xxx)

–	Otherwise, the final predicted sample values predSamples[ x ][ y ] are derived by

predSamples[ x ][ y ] = Clip3( 0, ( 1 << bitDepth ) − 1, 
	 ( ( (predSamplesL0[ x ][ y ] * ica0) >> ick0)  +  ( (predSamplesL1[ x ][ y ] * ica1) >> ick1) + offset2 +
	  ( (icb0 + icb1 ) << shift1) ) >> shift2 )	(8-xxx)

9.3.1.1 Initialization process for context variables

Following highlighted modification is applied.

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| coding_unit() | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | skip_flag | Table 9- |  | 0..2 | 3..5 |
|  | cu_ic_enable_flag | Table 9-x |  | 0..2 | 3..5 |
|  | cu_qp_delta_abs | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 | 1..3 | 4..6 |

Table 9-x – Values of variable initValue for cu_ic_enable_flag ctxIdx

| Initialization variable | cu_ic_enable_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 197 | 185 | 201 | 197 | 185 | 201 |

9.3.2 Binarization process

Following highlighted modification is applied.

Table 9-2 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| skip_flag | 1 | FL, cMax = 1 | 0 | Table 9- | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | 0 | Table 9- | 3 |
| cu_ic_enable_flag | 1 | FL, cMax = 1 | 0 | Table 9- | 0 |
|  | 2 |  | 0 | Table 9- | 3 |

9.3.3.1 Derivation process for ctxIdx

Following highlighted modification is applied.

| skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |
| cu_ic_enable_flag | 0 | na | na | na | na | na |