| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0045 |
| --- | --- |

| Title: | 3D-CE2.h : Results of Illumination Compensation for Inter-View Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu (hongbin.liu@lge.com) Jiwook Jung (jiwook.jung@lge.com) Jaewon Sung (jw.sung@lge.com) Jie Jia (jie.jia@lge.com) Sehoon Yea (sehoon.yea@lge.com) |  |  |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution reports result of illumination compensation (IC) proposed in JCT2-A0086. In the proposed method, linear IC model is employed to compensate illumination discrepancy between different views. Parameters in IC model are estimated for each PU (prediction unit) using available reconstructed neighbouring pixels and therefore are not need to be encoded. Applying IC or not is decided at CU (coding unit) level, and an IC flag is coded for CU in which inter-view prediction is used. A method is also designed to adaptively determine whether there is illumination discrepancy in a picture, and an IC flag is encoded for the first slice of each picture in dependent views.

Experiments are provided for weighted prediction, illumination compensation and illumination compensation with weighted prediction enabled. It is reported that illumination compensation can achieve -0.7% and -1.0% gains on two side views, and -0.4%, -0.3 and -0.3% improvement on video, synthesized view, and coded and synthesized view respectively when it is applied to both luma and chroma. The best performance is achieved on kendo, with -3.6% and -4.0% gain on two side views and -1.6%, -1.3% and -1.3% gains on video, synthesized view, and coded and synthesized view respectively.

# Introduction

Linear illumination compensation model was proposed to compensate illumination discrepancy between different views in JCT-A0086 [1]. In this contribution, we simplify this method at encoder to reduce the encoding time and extend the concept to UV component and depth view, and evaluate performance of it.

# Proposed Method

Linear IC model is used to compensate illumination discrepancy between different views. Parameters in IC model are estimated for each PU using available nearest reconstructed neighbouring pixels and therefore are not need to be encoded. Applying IC or not is decided at CU level, and an IC flag is coded for CU in which inter-view prediction is used. For adaptive IC, the encoder decides whether the IC should be applied or not to current picture and transmits the decision result to decoder.

To reduce the encoding time, instead of performing rate distortion optimized (RDO) selection between IC on and IC off for all inter modes, first perform RDO selection between IC on and IC off for Merge mode and Size_2Nx2N mode. If the best mode after checking these two modes does not employ IC, IC is switched off in the following inter modes; otherwise, RDO selection between IC on and IC off will be performed for all inter modes.

# Results

Proposed method is integrated into 3DV-HTM 4.0.1 software and compared with it. Six experiment sets are tested following the configuration and common test condition defined in [2].

- Set 1: weighted prediction is enabled

- Set 2: illumination compensation is enabled on luma

- Set 3: weighted prediction is enabled + illumination compensation is enabled on luma

- Set 4: illumination compensation is enabled on both luma and depth

- Set 5: illumination compensation is enabled on both luma and chroma

Simulation set 1~4 are tested on same set of computers, while set 5 is tested on another set of computers. Therefore, the computation time cannot be compared between them.

## Performance of weighted prediction

As shown in Table 1, when weighted prediction is enabled, there is 0.2% loss on video and 0.4% loss on coded and synthesized view. Encoding time is increased by 22.9% and decoding time is not changed much.

Table 1: performance of weighted prediction

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.1% | 0.4% | 0.4% | 0.2% | 0.5% | 0.5% | 106.3% | 99.5% |
| Kendo | 0.1% | 0.5% | 0.0% | 0.1% | 1.0% | 1.0% | 145.2% | 103.7% |
| Newspapercc | 0.1% | 0.4% | 0.4% | 0.2% | 0.4% | 0.4% | 118.2% | 101.7% |
| GhostTownFly | 0.4% | 0.3% | 0.3% | 0.4% | -0.2% | -0.1% | 152.6% | 102.3% |
| PoznanHall2 | 0.1% | 0.5% | 0.5% | 0.3% | 1.0% | 0.9% | 124.0% | 89.2% |
| PoznanStreet | 0.0% | 0.3% | 0.3% | 0.1% | 0.2% | 0.2% | 103.1% | 98.0% |
| UndoDancer | 0.0% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% | 119.0% | 108.9% |
| 1024x768 | 0.1% | 0.4% | 0.2% | 0.2% | 0.6% | 0.6% | 122.2% | 101.6% |
| 1920x1088 | 0.2% | 0.3% | 0.3% | 0.2% | 0.3% | 0.3% | 123.4% | 99.4% |
| average | 0.1% | 0.4% | 0.3% | 0.2% | 0.4% | 0.4% | 122.9% | 100.3% |

## Performance of illumination compensation on luma

As shown in Table 2, when illumination compensation is enabled on luma, there is -0.3% improvement on video and -0.3% improvement on coded and synthesized view. Encoding time is increased by 8.0% and decoding time is not changed much.

Table 2: performance of illumination compensation on luma

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.4% | -1.1% | -0.6% | -0.4% | -0.5% | 109.1% | 101.1% |
| Kendo | 0.0% | -3.1% | -3.3% | -1.4% | -1.0% | -1.1% | 114.2% | 100.9% |
| Newspapercc | 0.0% | -0.6% | -0.8% | -0.3% | -0.2% | -0.2% | 108.2% | 101.2% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 105.4% | 100.6% |
| PoznanHall2 | 0.0% | 0.3% | -0.8% | -0.2% | -0.1% | -0.1% | 108.6% | 93.2% |
| PoznanStreet | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | 0.0% | 106.1% | 96.9% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 105.0% | 104.8% |
| 1024x768 | 0.0% | -1.7% | -1.7% | -0.7% | -0.5% | -0.6% | 110.5% | 101.1% |
| 1920x1088 | 0.0% | 0.1% | -0.3% | 0.0% | 0.0% | 0.0% | 106.3% | 98.8% |
| average | 0.0% | -0.7% | -0.9% | -0.3% | -0.2% | -0.3% | 108.0% | 99.8% |

## Performance of illumination compensation on luma with weighted prediction enabled

As shown in Table 3, when illumination compensation is enabled on luma with weighted prediction enabled, there is -0.3% improvement on video and 0.2% loss on coded and synthesized view. Encoding time is increased by 28.5% and decoding time is increased by 1.3%.

Table 3: performance of illumination compensation on luma with weighted prediction enabled

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.1% | -0.9% | -0.5% | -0.3% | 0.1% | 0.0% | 117.0% | 100.9% |
| Kendo | 0.1% | -2.0% | -1.8% | -0.8% | 0.3% | 0.2% | 151.3% | 102.6% |
| Newspapercc | 0.1% | -0.1% | -0.4% | -0.1% | 0.2% | 0.2% | 125.2% | 101.3% |
| GhostTownFly | 0.4% | 0.3% | 0.3% | 0.4% | -0.1% | -0.1% | 154.6% | 104.2% |
| PoznanHall2 | 0.1% | 0.8% | -0.3% | 0.1% | 0.9% | 0.8% | 125.5% | 101.0% |
| PoznanStreet | 0.0% | 0.4% | 0.1% | 0.1% | 0.2% | 0.2% | 107.0% | 101.1% |
| UndoDancer | 0.0% | 0.2% | 0.2% | 0.1% | 0.0% | 0.0% | 125.5% | 98.2% |
| 1024x768 | 0.1% | -1.0% | -0.9% | -0.4% | 0.2% | 0.2% | 130.4% | 101.6% |
| 1920x1088 | 0.2% | 0.4% | 0.1% | 0.2% | 0.2% | 0.2% | 127.1% | 101.1% |
| average | 0.1% | -0.2% | -0.4% | -0.1% | 0.2% | 0.2% | 128.5% | 101.3% |

## Performance of illumination compensation on both luma and depth

As shown in Table 4, when illumination compensation is enabled on both luma and depth, there is -0.3% improvement on video and -0.6% improvement on coded and synthesized view. Encoding time is increased by 15.0% and decoding time is increased by 1.3%.

Table 4: performance of illumination compensation on luma and depth

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.4% | -1.1% | -0.6% | -0.6% | -0.6% | 115.8% | 100.9% |
| Kendo | 0.0% | -3.1% | -3.3% | -1.4% | -1.4% | -1.3% | 122.8% | 102.6% |
| Newspapercc | 0.0% | -0.6% | -0.8% | -0.3% | -1.2% | -1.0% | 116.9% | 101.3% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 108.9% | 104.2% |
| PoznanHall2 | 0.0% | 0.3% | -0.8% | -0.2% | -0.2% | -0.2% | 116.3% | 101.0% |
| PoznanStreet | 0.0% | 0.1% | -0.2% | 0.0% | -0.3% | -0.2% | 113.4% | 101.1% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 111.2% | 98.2% |
| 1024x768 | 0.0% | -1.7% | -1.7% | -0.7% | -1.1% | -1.0% | 118.4% | 101.6% |
| 1920x1088 | 0.0% | 0.1% | -0.3% | 0.0% | -0.2% | -0.1% | 112.4% | 101.1% |
| average | 0.0% | -0.7% | -0.9% | -0.3% | -0.6% | -0.5% | 115.0% | 101.3% |

## Performance of illumination compensation on both luma and chroma

As shown in Table 5, when illumination compensation is enabled on both luma and chroma, there is -0.4% improvement on video and -0.3% improvement on coded and synthesized view. Encoding time is increased by 6.6% and decoding time is not changed much.

Table 5: performance of illumination compensation on luma and chroma

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.2% | -1.5% | -0.6% | -0.4% | -0.5% | 107.8% | 99.8% |
| Kendo | 0.0% | -3.6% | -4.0% | -1.6% | -1.3% | -1.3% | 109.3% | 99.7% |
| Newspapercc | 0.0% | -0.6% | -1.1% | -0.4% | -0.2% | -0.3% | 106.5% | 100.0% |
| GhostTownFly | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 104.5% | 99.9% |
| PoznanHall2 | 0.0% | 0.1% | -0.1% | -0.1% | 0.1% | 0.0% | 106.3% | 99.8% |
| PoznanStreet | 0.0% | 0.0% | -0.4% | -0.1% | -0.1% | -0.1% | 106.0% | 94.9% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 105.9% | 99.8% |
| 1024x768 | 0.0% | -1.8% | -2.2% | -0.8% | -0.6% | -0.7% | 107.9% | 99.8% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 105.7% | 98.6% |
| average | 0.0% | -0.7% | -1.0% | -0.4% | -0.3% | -0.3% | 106.6% | 99.1% |

# Conclusion

This proposal improves coding efficiency of dependent views significantly and it has small influence on decoding time. We recommend that proposed method is adopted into 3DV-HEVC.

# Reference

[1] D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, ISO/IEC JTC1/SC29/WG11, Doc. JCT2-A1100, July 2012, Stockholm, Sweden.

[2] H. Liu, J. Jung, J. Sung, etc., “3D-CE1.h related: Illumination Compensation for Inter-View Prediction”, ISO/IEC JTC1/SC29/WG11, Doc. JCT2-A0086, July 2012, Stockholm, Sweden.

# Patent rights declaration(s)

LG Electronics / LG Electronics (China) R&D Center may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).