| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0059r1 M26751 |
| --- | --- |

| Title: | AHG8: Proposed Draft Amendment on Signaling of Alternative 3D Format in ISO/IEC 14496-10 (ITU-T H.264) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposed Draft |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Yasuyuki Ichihashi, Hisayuki Sasaki, Kenji Yamamoto Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan Masayuki Tanimoto, Kazuyoshi Suzuki Zehn 101, Kawana-cho 2-64, Showa-ku, Nagoya, 466-0856, Japan Nicolce Stefanoski Zurich, Switzerland | Tel: Email: | +81-42-327-7262 senoh@nict.go.jp +81-52-223-6639 tanimoto@nisri.jp stefanos@disneyresearch.com |
| Source: | National Institute of Information and Communications Technology(NICT) Nagoya Industrial Science Research Institute(NISRI) Disney Research Zurich (DRZ) |  |  |

_____________________________

# Abstract

This contribution proposes a solution for one of mandates for AHG8, which is

- to investigate possible signaling methods required to identify the type of 3D data format (e.g. global view/depth, warp) and to interpret such decoded 3D data by non-normative processing after decoding.

The solution proposes a new NAL_unit_type =24 for ISO/IEC 144996-10 (ITU-T H.264).

The insertion of one byte header including this NAL unit type and the following one byte flag identifies the alternative 3D format stream from the other streams conforming to ISO/IEC 144996-10 (ITU-T H.264).

# Introduction

At the 1st JCT-3V meeting (July 2012), AHG8 was established. One of its mandates is to investigate possible signaling methods required to identify the type of 3D data format (e.g. global view/depth, warp) and to interpret such decoded 3D data by non-normative processing after decoding. This contribution proposes a solution for this mandate in case of ISO/IEC 144996-10 (ITU-T H.264) streams by specifying a new NAL_unit_type and the following one byte flag. This signaling may be a new amendment for ISO/IEC 144996-10 (ITU-T H.264) or for ISO/IEC 23002-3:2007 The following text is prepared for the amendment for ISO/IEC 144996-10 (ITU-T H.264).

# Syntax (normative)

In 0.6, add the following paragraph after the last paragraph that starts with “An extension of Multiview video coding”:

A signaling method that supports the alternative 3D format is specified in Annex J allowing the construction of bitstreams that represent alternative 3D format based on the ISO/IEC 14496-10 specifications. Since the alternative 3D format uses codecs conforming to ISO/IEC 14496-10 in order to encode the pre-processed data such as global depth or warp map, a unique NAL_unit_type is specified to identify the stream from the other streams conforming to ISO/IEC 14496-10.

In 0.7, add the following paragraph after the paragraph that starts with “Annex I specifies”:

Annex J specifies a signaling method of alternative 3D format. The reader is referred to Annex J for the entire decoding process for Alternative 3D format, which is specified there with references being made to clauses 2-9 and Annexes A-E.

In 7.3.1, replace the syntax table with:

| nal_unit( NumBytesInNALunit ) { | C | Descriptor |
| --- | --- | --- |
| forbidden_zero_bit | All | f(1) |
| nal_ref_idc | All | u(2) |
| nal_unit_type | All | u(5) |
| NumBytesInRBSP = 0 |  |  |
| nalUnitHeaderBytes = 1 |  |  |
| if( nal_unit_type = = 14 ¦ ¦ nal_unit_type = = 20 ¦ ¦ nal_unit_type = = 21) { |  |  |
| svc_extension_flag | All | u(1) |
| if( svc_extension_flag && ( nal_unit_type = = 14 ¦ ¦ nal_unit_type = = 20 ) ) |  |  |
| nal_unit_header_svc_extension( ) /* specified in Annex Error: Reference source not found */ | All |  |
| else |  |  |
| nal_unit_header_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | All |  |
| nalUnitHeaderBytes += 3 |  |  |
| } |  |  |
| else if( nal_unit_type = = 24 |  |  |
| Alternative_3D_format_extension( ) /* specified in Annex J*/ |  |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| i += 2 |  |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | All | f(8) |
| } else |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| } |  |  |
| } |  |  |

Replace Table 7-1 with:

| nal_unit_type | Content of NAL unit and RBSP syntax structure | C | Annex Error: Reference source not found NAL unit type class | Annex Error: Reference source not found and Annex Error: Reference source not found NAL unit type class | Annex Error: Reference source not found NAL unit type class |
| --- | --- | --- | --- | --- | --- |
| 0 | Unspecified |  | non-VCL | non-VCL | non-VCL |
| 1 | Coded slice of a non-IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 | VCL | VCL | VCL |
| 2 | Coded slice data partition A slice_data_partition_a_layer_rbsp( ) | 2 | VCL | not applicable | not applicable |
| 3 | Coded slice data partition B slice_data_partition_b_layer_rbsp( ) | 3 | VCL | not applicable | not applicable |
| 4 | Coded slice data partition C slice_data_partition_c_layer_rbsp( ) | 4 | VCL | not applicable | not applicable |
| 5 | Coded slice of an IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3 | VCL | VCL | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | 5 | non-VCL | non-VCL | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | 0 | non-VCL | non-VCL | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | 1 | non-VCL | non-VCL | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | 6 | non-VCL | non-VCL | non-VCL |
| 10 | End of sequence end_of_seq_rbsp( ) | 7 | non-VCL | non-VCL | non-VCL |
| 11 | End of stream end_of_stream_rbsp( ) | 8 | non-VCL | non-VCL | non-VCL |
| 12 | Filler data filler_data_rbsp( ) | 9 | non-VCL | non-VCL | non-VCL |
| 13 | Sequence parameter set extension seq_parameter_set_extension_rbsp( ) | 10 | non-VCL | non-VCL | non-VCL |
| 14 | Prefix NAL unit prefix_nal_unit_rbsp( ) | 2 | non-VCL | suffix dependent | suffix dependent |
| 15 | Subset sequence parameter set subset_seq_parameter_set_rbsp( ) | 0 | non-VCL | non-VCL | non-VCL |
| 16..18 | Reserved |  | non-VCL | non-VCL | non-VCL |
| 19 | Coded slice of an auxiliary coded picture without partitioning slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 | non-VCL | non-VCL | non-VCL |
| 20 | Coded slice extension slice_layer_extension_rbsp( ) | 2, 3, 4 | non-VCL | VCL | VCL |
| 21 | Coded slice extension for depth view components /*specified in Annex I */ slice_layer_extension_rbsp( ) /* specified in Annex I */ | 2, 3, 4 | non-VCL | VCL | VCL |
| 22..23 | Reserved |  | non-VCL | non-VCL | VCL |
| 24 | Alternative 3D format extension /*specified in Annex J*/ | All | both | Not applicable | Not applicable |
| 25..31 | Unspecified |  | non-VCL | non-VCL | non-VCL |

# Syntax and semantics

Add Annex J:

- Annex J

- Signaling of alternative 3D format

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies the signaling of Alternative 3D Format for 3DVC.

  - Scope

Bitstreams and decoders conforming to the profile specified in this annex are completely specified in this annex with reference made to clauses Error: Reference source not found-Error: Reference source not found and Annexes Error: Reference source not found-E.

  - Normative references

The specifications in clause Error: Reference source not found apply.

  - Definitions

For the purpose of this annex, the following definitions apply additionally.

    - Alternative 3D format: is an alternative 3DV bitstream format for multi-view and depth maps which is specified in this annex..

    - Warp: is one of the alternative 3D format which replaces depth maps in the multi-view and depth map streams.

    - Global view and depth: is another alternative 3D format for multiview and depth map stream.

    - Znear: is a value of the nearest object distance from camera.

    - Zfar: is a value of the farthest object distance from camera.

    - trans: is a value of camera location in the world co-ordinate system.

    - Global view and depth bitstream: is a set of bitstreams for base view, global depth, residual and their SEI messages.

  - Abbreviations

The specifications in clause Error: Reference source not found apply.

  - Conventions

The specifications in clause Error: Reference source not found apply.

  - Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specifications in clause Error: Reference source not found apply.

  - Syntax and semantics

This clause specifies syntax and semantics for coded video sequences (CVD) and non-coded video sequences (non-CVD) that conform to the alternative 3D format specified in this annex.

    - Method of specifying syntax in tabular form

The specifications in sub-clause Error: Reference source not found apply.

    - Specification of syntax functions, categories, and descriptors

The specifications in sub-clause Error: Reference source not found apply.

    - Syntax in tabular form

      - NAL unit syntax

| Nal_unit(NumBytesInNALunit) { | C | Descriptor |
| --- | --- | --- |
| forbidden_zero_bit | All | f(1) |
| nal_ref_idc | All | u(2) |
| nal_unit_type | All | f(5) |
| alternative_3d _flag | All | u(1) |
| if( alternative_3d _flag = = 0) |  |  |
| warp_extenson() /* specified later */ | All |  |
| else |  |  |
| global_view_and_depth_extension() | All |  |
| } |  |  |

      - Global view and depth extension syntax

| Global_view_and_depth_extension(NumBytesInNALunit) { | C | Descriptor |
| --- | --- | --- |
| gvd_flag | All | u(2) |
| reserved_5bits | All | u(5) |
| for( i = 2; i < NumBytesInNALunit; i++ ) { |  |  |
| if (gvd_flag = = 00) |  |  |
| gvd_depth_payload [ i++ ] /* global depth stream */ |  |  |
| else if(gvd_flag = = 01) |  |  |
| gvd_residual_payload [ i++ ] /* residual stream */ |  |  |
| else if(gvd_flag = = 10) |  |  |
| gvd_sei_payload[ i++ ] /* SEI message */ |  |  |
| } |  |  |
| } |  |  |

      - Global view and depth SEI message syntax

| gvd_sei_payload() { | C | Descriptor |
| --- | --- | --- |
| if(reserved 5bits = = 00000) |  |  |
| camera_parameter() /* camera parameters */ |  |  |
| else if(reserved 5bits = = 00001) |  |  |
| cfg_info() /* configuration information */ |  |  |
| else if(reserved 5bits = = 00010) |  |  |
| variable_parameter() /* variable parameters */ |  |  |
| } |  |  |

        - Camera parameter SEI message syntax

| Syntax | Example |
| --- | --- |
| for(i=1; i <=N; i++) { | If(2-view) N=2, else if(3-view) N=3 |
| Camera_parameter_name_i | param_dancer_1, etc |
| focalLengthX skiwFactorX principalPointX | 2302. 852541609168 0.0 960.0, etc. |
| skiwFactorY focalLengthY principalPointY | 0.0 2302.852541609168 540.0, etc. |
| 0 0 1 | 0.0 0.0 1.0 |
| lensDistortionX | 0.0, etc. |
| lensDistortionY | 0.0, etc. |
| RotationMatrix[r11 r12 r13] translationX | 1.0 0.0 0.0 -75.0, etc. |
| RotationMatrix[r21 r22 r23] translationY | 0.0 1.0 0.0 0.0, etc. |
| RotationMatrix[r31 r32 r33] translationZ | 0.0 0.0 1.0 0.0, etc. |
| } |  |

        - Configuration information SEI message syntax

| Name | Value |
| --- | --- |
| Mode | QUARTER, QUARTER_ODD, QUARTER_STACK, HALF_QUARTER_STACK |
| if(!variable parameter) { |  |
| NearestDepthValue1 | 2289, etc. |
| FarthestDepthValue1 | 213500, etc. |
| NearestDepthValue2 | 2289, etc. |
| FarthestDepthValue2 | 213500, etc. |
| if(3-view) { |  |
| NearestDepthValue3 | 2289, etc. |
| FarthestDepthValue3 | 213500, etc. |
| } |  |
| } |  |

        - Variable parameter SEI message syntax

| Syntax | Example |
| --- | --- |
| for(v=1; v<=N; v++) { | If(2-view) N=2, else if(3-view) N=3 |
| Znear Zfar transX (no C/R, L/F) | 70625 662000 0, etc |
| } |  |

      - Global view and depth bitstream syntax

| Bitstream_for_global_view_and_depth() { | C | Descriptor |
| --- | --- | --- |
| byte_stream_nal_unit_for_camera_parameter() | All |  |
| byte_stream_nal_unit_for_cfg_info() | All |  |
| while(access unit) { |  |  |
| if(variable parameter) |  |  |
| byte_stream_nal_unit_for_variable_parameter() | All |  |
| while(NAL_unit_type != 18 /* != alternative 3d format */) |  |  |
| byte_stream_nal_unit_fo_ base_view() | All |  |
| while(NAL_unit_type = = 18 && gvd_flag = = 00 /* global depth */) |  |  |
| byte_stream_nal_unit_for_global_depth() | All |  |
| while(NAL_unit_type = = 18 && gvd_flag = = 01 /* residual */) |  |  |
| byte_stream_nal_unit_for_residual() |  |  |
| } | All | u(2) |
| } | All | u(5) |

      - Byte stream NAL unit syntax

Refer sub-clause B.1.1 in clause Annex B.

    - Semantics

Semantics associated with the syntax structures and syntax elements within these structures (in sub-clause J.7.3) are specified in this sub-clause. When the semantics of a syntax element are specified using a table or a set of tables, any values that are not specified in the table(s) shall not be present in the bitstream unless otherwise specified in this Recommendation | International Standard.

      - NAL unit semantics

The semantics for the syntax elements in sub-clause J.7.3.1 are specified here.

alternative_3d _flag is one-bit flag. It indicates whether this NAL unit is warp stream or global view and depth stream. If this flag = 0, this stream is a warp stream. If this flag = 1, this stream is a global view and depth stream.

warp_extention() is one of the alternative 3d format. This extension is specified later.

global_view_and_depth_extension() is another alternative 3d format. This extention is specified in Annex J.

      - Global view and depth extension semantics

gvd_flag is two-bit flag. It indicates whether this stream is global depth, residual, or SEI message. If this flag = 00, this stream is a global depth stream. If the flag = 01, this stream is a residual stream. If the flag = 10, this stream is a SEI message for global view and depth stream.

reserved_5bits is specified as follows. If gvd_flag = = 00 or 01, this value = 00000. If gvd_flag = = 10, this value is specified in sub clause J.7.3.3 and identifies the contents of gvd_sei_payload.

gvd_depth_payload is global depth data. This payload is identical to the byte stream NAL unit specified in ISO/IEC 14496-10 clause 2 to Annex E. Consequently, by removing the first two bytes after start_code_prefix_one__3bytes, this payload can be decoded by single view decoders conforming to ISO/IEC 14496019 clause 2 to Annex E.

gvd_residual_payload is residual data. This payload is identical to the byte stream NAL unit specified in ISO/IEC 14496-10 clause 2 to Annex E. Consequently, by removing the first two bytes after start_code_prefix_one__3bytes, this payload can be decoded by single view decoders conforming to ISO/IEC 14496-10 (ITU-T H.264). clause 2 to Annex E.

gvd_sei_payload is SEI message of 8-bit ASCII code text for global view and depth stream.

      - Global view and depth SEI message semantics

Gvd_sei_payload is ASCII code string of cfg_info(), camera_parameter() or variable_parameter(). It is identified by reserved_5bits when gvd_flag = = 10 as specified in subclause J.7.3.3.

        - Camera parameter SEI message semantics

- Camera parameter SEI message is an ASCII code text. It specifies camera parameter name, intrinsic matrix values, lens distortion values and external matrix values. Each value is separated by a space code or c/R and L/F code, as specified in sub-clause J.7. 3.3.1.

- Intrinsic parameter values are specified in three rows as follows.

focalLengthXskiewFactorXprincipalPointXskiewFactorYfocalLengthYprincipalPointY001

Lens distortion values are specified in two rows as follows.

lensDistortionXlensDistortionY

External matrix values are specified in three rows as follows.

RotationMatrixTranslationMatrix

Rotation matrix is specified as follows.

r11r12r13r21r22r23r31r32r33

Translation matrix is specified as follows.

txtytz

        - Configuration information SEI message semantics

- Configuration information SEI message is an ASCII code text. Each row specifies one parameter with the name and value separated by space code as specified in sub-clause J.7.3.3.2.

Mode specifies the coding mode of global view and depth bitstream with QUARTER, QUARTER_ODD, QUARTER_STACK and HALF_QUARTER_STACK. QUARTER is for 2-view streams. QUARTER_ODD is for 3-view streams with one global depth. QUARTER_STACK is for 3-view streams with two global depths. HALF_QUARTER_STACK is for 3-view streams with two global depths of half height. They can be used depending on the input sequence characteristics.

If the following parameters exist in the configuration information SEI message, this stream is a fixed parameter stream and no variable parameter SEI message follows. If not, this stream is a variable parameter stream and the following parameters are in the variable parameter SEI message for every frame.

NearestDepthValue1 is the nearest object distance in view 1.

FarthestDepthValue1 is the farthest object distance in view 1.

NearestDepthValue2 is the nearest object distance in view 2.

FarthestDepthValue2 is the farthest object distance in view 2.

If Mode != QUARTER (!=2-viewe), the following two parameters exist.

NearestDepthValue3 is the nearest object distance in view 3.

FarthestDepthValue3 is the farthest object distance in view 3.

        - Variable parameter SEI message semantics

- Variable parameter SEI message is ASCII code text for variable parameters of NearestDepthValue, FarthestDepthValue and TranslationX separated by space code for every input views of each frame.

      - Global view and depth bitstream semantics

- Global view and depth bitstream consists of byte stream NAL units for gvd_sei payloads of camera_parameter, cfg_info and byte stream NAL units for an access unit in this order. In the access unit, if this global view and depth bitstream is a variable parameter stream, a byte stream NAL unit for variable parameter exist first. Then, NAL units for the i-th frame of base view (B_View), byte stream NAL units for the i-th frame of global depth and byte stream NAL units for the i-th frame of residual repeat in this order. (i=0, 1,…, number of frames in access unit -1)

Byte_stream_NAL_unit for base view is identical to the byte stream NAL unit conforming to ISO/IEC 14496-10 (ITU-T H.264) clause 2 to Annex E. This NAL unit can be decoded by single view decoders conforming to ISO/IEC 14496-10 (ITU-T H.264) clause 2 to Annex E and realises forward compatibility with existing single view decoders.

Byte stream NAL unit for global depth and byte stream NAL unit for residual contain two NAL_unit _types. The first NAL_unit_type after start_code_prefix_one_3bytes is for the identification of global depth stream or residual stream. The second NAL_unit_type in the beginning of gvd_depth_payload or gvd_residual_payload is used to decode these streams by decoders conforming to ISO/IEC 14496-10 (ITU-T H.264) clauses 2 to E. When the first NAL_unit_type exists, the second NAL_unit_type is not recognized by decoders conforming to ISO/IEC 14496-10 (ITU-T H.264) clauses 2 to E since the second NAL_unit_type is not directly following the start_code_prefix_one_3bytes. Hence, mis-interpretation of global depth or residual bitstream is avoided.

      - Byte stream NAL unit semantics

- Refer sub-clause B.1.2 in clause Annex B.

  - Decoding process

- Outputs of this process are synthesized multi-view pictures from global view and depth bitstream.

An overview of the decoding process is given as follows:

- De-multiplexing process for global view and depth bitstream to byte stream NAL units for global view and depth SEI message, base view, global depth, and residual is specified in sub-clause J.8.1.

- Decoding of de-multiplexed byte stream NAL unit is specified in sub-clause J.8.2.

- View synthesis from decoded base view, global depth and residual is specified in sub-clause J.8.3.

Unless specified here, the decoding process specified in clause 8 and Annex B apply.

    - De-multiplexing process for global view and depth bitstream

- The global view and depth bitstream is separated to each byte stream NAL unit.

The byte_stream_nal_unit_for_camera_parameter() is parsed first. This NAL unit is identified by nal_unit_type =18, gvd_flag =10 and reserved_5bits =00000. Extracted camera parameter() is stored in a camera parameter buffer.

Then, byte_stream_nal_unit_for_cfg_info() is parsed. This NAL unit is identified by nal_unit_type =18, gvd_flag =10 and reserved_5bits =00001. Extracted cfg_info() is stored in a configuration information buffer.

Then, bitstream for an access unit is parsed.

If the parsed cfg_info() contains no NearestDepthValuei nor FarthestDepthValuei, byte_stream_nal_unit_for_variable_parameter() exists at the beginning of every frame. This NAL unit is identified by nal_unit_type =18, gvd_flag =10 and reserved_5bits =00010. Extracted variable parameter() is stored in a variable parameter buffer.

Then, byte_stream_nal_unit_for_base_view() is parsed. This NAL unit is identified by NAL_unit_type !=18. Identified byte_stream_nal_unit_for_base_view() is transferred to a decoder conforming to ISO/IEC 14496-10 (ITU-T H.264) clauses 2 to Annex E as the base view stream.

Then, byte_stream_nal_unit_for_global depth() is parsed. This NAL unit is identified by NAL_unit_type =18 and gvd_flag =00. Then, the first two bytes after start_code_prefix_one_3bytes are removed. The resulted byte stream NAL unit i is transferred to a decoder conforming to ISO/IEC 14496-10 (ITU-T H.264) clauses 2 to Annex E as the global depth stream.

Then, byte_stream_nal_unit_for residual() is parsed. This NAL unit is identified by NAL_unit_type =18 and gvd_flag =01. Then, the first two bytes after start_code_prefix_one_3bytes are removed. The resulted byte stream NAL unit is transferred to a decoder conforming to ISO/IEC 14496-10 (ITU-T H.264) clauses 2 to Annex E as the residual stream.

At the beginning of every access unit, there might exist plural of byte_stream_nal_unit_for_base_view(), byte_stream_nal_unit_for_global_depth() or byte_stream_nal_unit_for_rersidual(), including SEI messages for each stream.

    - Decoding process for de-multiplexed byte stream NAL unit

- Decoding of de-multiplexed byte stream NAL units is specified in clause 8 and Annex B.

Decoded yuv frames of base view are stored in a base view buffer.

Decoded yuv frames of global depth are stored in a global depth buffer.

Decoded yuv frames of residual are stored in a residual buffer.

    - View synthesis process for global view and depth format (Informative)

- Target views including input left view and right view are synthesized from decoded base view, global depth map and residual. Figure J.8.1 illustrates the view synthesis process.

- Figure J.8.1 View synthesis process

- First, decoded global depth map is up-sampled. During the up-sampling, if the difference of contiguous two depth samples are smaller than th (=10, etc.), the average value is inserted between them. Otherwise, a larger sample is repeated. After the up-sampling, 2D median filter of 3x3 and 5x5 are applied to smooth the shape of depth edge. Then, each depth pixel D(x,y) in the global depth map is projected to left target depth pixel D(xL, y) and right target depth pixel D(xR, y). The corresponding address xL, xR in the target views are given by the following equation. Where, f is focal length of camera, BL and BR are baseline lengths between the target views and global depth, D(x,y) is depth value of global depth pixel (x,y).

δK=fBK255(1Znear−1Zfar)D(x,y), (K=L, or R)xL=x+δL, xR=x−δR	(J.8.1)

- After the projection, holes are inpainted with a smaller existing sample of left or right edge of the hole.

- Then, decoded base view is projected to left and right target views by the left and right target depth maps. Before the projection, Chroma components are up-sampled to the same size as Luma component. View projection of sub-pixel accuracy is done by averaging two contiguous pixels weighted by the mantissa of disparity value calculated from the depth value. The corresponding pixel address (x, y) in the base view is calculated from the equation (J.8.1) by solving it for x.

- During the projection, occlusion holes are detected by checking the target depth map by the following equation. Here, k=1,2,3 denotes left target view, base view and right target view. Bk denotes the baseline length between the target view and base view.

- if (Dk(x+δ,y)−Dk(x,y))≥th, (x,y) is in occlusion hole.th=255fBk(1Znear−1Zfar)δ, (0≤δ≤fBkZnear−fBkZfar)	(J.8.2)

Figure J.8.2 illustrates a case for left target view (k=1).

Figure J.8.2 Occlusion hole detection for left target view

- If a neighbour depth value D1(x+δ, y) is larger than the currently checked depth value D1(x, y) by th, the current pixel (x, y) is in an occlusion hole, where th is the depth difference value of the disparity difference δ.

- Then, in order to refine the occlusion area, a disparity differenceεis re-calculated from the depth difference D1(x+δ, y)- D1(x, y) as follows.

- ε=fB1255(1Znear−1Zfar)D1(x+δ,y)−D1(x,y)	(J.8.2)

- If D1(x+ε, y)-D1(x, y) ≧k*th, the pixel (x, y) is in an occlusion hole. Where, k=0.6 is a margin for a gap between depth map and texture.

- Since an occlusion hole exists in the left side of foreground objects when the base view resides in the right-hand side of the depth map view, only right neighbour depth values are checked. If the base view resides in the left-hand side, the left-side neighbour depth values are checked by changing the sign of δ. The search area of δ is 0 to the maximum disparity difference given by fBk(1/Znear - 1/Zfar). The detected hole areas are registered to left and right base view hole masks.

On the other hand, decoded and framed residual views are separated to decoded left and right residuals. Then, they are up-sampled to up-sampled left and right residuals of the original size with simple two-tap (0.5, 0.5) filter vertically and horizontally. The up-sampled left and right residuals are projected to the target views by left and right target depth maps as same as the base view projection. During the residual projection, occlusion holes are also detected and registered to left and right residual hole masks as well.

- Then, the left and right base view hole masks are dilated by two pixels in order to ease the mismatch between the depth map and the texture. Projected residuals are over-written in the hole area of left and right target views according to the left and right base view hole masks. Then, common hole masks are created by ANDing the left and right base view hole masks and the left and right residual hole masks. Common holes in the left and right target views are inpainted according to these common hole masks. Then, horizontal two pixels on the both sides of base hole mask edge are filtered by three-tap filter(0.25, 0.5, 0.25) to reduce the hole edge contour if their difference is smaller than threshold (=20). If the difference is equal or larger than threshold, it is left as the object edge. Finally, the target views are output as synthesized views.

- In case of 2-view configuration, read above process without right target depth, right residual, and right target view.

    - Encoding process for global view and depth (Informative)

- The encoding process is described in JCT2-A00072 | ISO/IEC JTC1/SC29/WG11 M26104 and ISO/IEC JTC1/SC29/WG11 M22573.

  - Informative references

- JCT2-A00072 | ISO/IEC JTC1/SC29/WG11 M26104, Proposal on Simple Test Model for 3DV Coding based on CE7.

- ISO/IEC JTC1/SC29/WG11 M22573, Technical description of 3DV proposal P-27

# Patent rights declaration(s)

NICT (National Institute of Information and Communications Technology), NISRI (Nagoya Industrial Science Research Institute) and DRZ (Disney Research Zurich) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).