| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0063 |
| --- | --- |

| Title: | View dependency change SEI message for MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Jeonghoon Park | Email: | b.d.choi@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a SEI message for view dependency changes. The view dependency change SEI message is specified in H.264/MVC, and useful to immediately change the dependencies between views in an access unit. The SEI message is always interpreted with respect to the active sequence parameter set in H.264/MVC. In the latest MV-HEVC text, however, all dependency information is signalled in VPS. Therefore, a new SEI message for view dependency changes, which are compliant with the current VPS design, is proposed.

# View dependency change SEI message compliant with VPS

This SEI message indicates that the view dependency information changes starting with the current access unit containing the SEI message and is always interpreted with respect to the active video parameter set. When present, the view dependency change SEI message applies to the target access unit set that consists of the current access unit and all the subsequent access units, in decoding order, until the next view dependency change SEI message or the end of the coded video sequence, whichever is earlier in decoding order. If, according to the view dependency information indicated in the active video parameter set, view component A does not directly depend on view component B and vice versa, the view dependency change SEI message shall not specify view dependency relationship between view components A and B. Note that the dependent views for any view are always a subset of those indicated by the active video parameter set.

| view dependency_change( payloadSize) { | Descriptor |
| --- | --- |
| active_vps_id | u(4) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| for( j = 0; j < num_direct_ref_layers[ i ] && i; j++ ) |  |
| ref_layer_disable_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |

active_vps_id identifies an active video parameter set that contains the inter-view dependency relationship information.

ref_layer_disable_flag[ i ][ j ] equal to 1 indicates that the j-th entry of the inter-view reference picture of the view component with layer_id equal to i will not be present after reference picture list modification. ref_layer_disable_flag[ i ][ j ] equal to 0 indicates that the j-th entry of the inter-view reference picture of the view component with layer_id equal to i will be present after reference picture list modification.

# Patent rights declaration

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).