| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IECJTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0064 |
| --- | --- |

| Title: | 3D-CE6.h related: Simplification of Depth Modeling Mode 3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shiqi Wang (sqwang@jdl.ac.cn) Hongbin Liu (hongbin.liu@lge.com) Siwei Ma (swma@jdl.ac.cn) Jie Jia (jie.jia@lge.com) |  |  |
| Source: | LG Electronics, Peking University |  |  |

_____________________________

# Abstract

This contribution proposes to further simplify Depth Modeling Mode (DMM) 3 based on JCT2-A0098. In JCT2-A0098, instead of performing exhaustive search from a huge number of candidate Wedgelet patterns, only candidate Wedgelet patterns that are within a predefined search range are searched. The search range is defined by intra prediction mode of the co-located texture luma block. This contribution proposes to further simplify the Wedgelet pattern search at decoder by signalling the best pattern in the predefined search range. In this way, the decoder does not need to perform any Wedgelet pattern search. It is reported that proposed method has negligible influence on compression efficiency (0.0%, -0.1% and 0.0% on video, synthesized view, coded and synthesized view respectively) when compared with JCT2-A0098 and HTM-4.0.1.

# Introduction

The basic principle of DMM3 mode is to predict best Wedgelet pattern of depth block from co-located texture luma block (CTLB). In DMM3, the prediction unit is partitioned into two regions by a straight line and each region is predicted by a constant value, as shown in Fig. 1. The partition information, i.e., the “best” Wedgelet pattern of DMM3 is not signaled to the decoder. Instead, both encoder and decoder derive the “best” Wedgelet pattern by searching all candidate Wedgelet patterns on reconstructed CTLB. The Wedgelet pattern with the minimum cost is selected as the “best” pattern. In this way, no bit is required for indicating “best” Wedgelet pattern. However, the exhaustive pattern search increases the decoder complexity significantly.

Fig.1. Example of Wedgelet Pattern。

To reduce the complexity of DMM3, instead of performing exhaustive search from a huge number of candidate Wedgelet patterns, authors of [1] propose to only search candidate Wedgelet patterns that are within a predefined search range. The search range is defined by using the intra prediction mode of the top-left 4×4 block of the CTLB. If the top-left 4×4 block in CTLB is not intra-coded, the DMM3 mode is disabled. The search range is defined as follows. Assuming that the starting point of the Wedgelet pattern is S(Xs, Ys) and the ending point is E(Xe, Ye), then there is a pair of (S, E) corresponding to each Wedgelet pattern.

For each intra prediction mode (mode 2 to 34), the direction information (Hi, Vi) is defined as in Table 1. The fitness between the Wedgelet pattern (defined by (Xs, Ys) and E(Xe, Ye)) and the intra mode (defined by (Hi, Vi)) is defined as:

D[i] = | Vi×(Xs-Xe) - Hi×(Ye-Ys) |                      (1)

Then, each Wedgelet pattern can be mapped to the nearest intra prediction mode by searching minimum D among all intra prediction modes. Correspondingly, for each intra prediction mode j, there exists a Wedgelet pattern set S(j) in which every pattern’s nearest intra prediction mode is j. For intra prediction mode 0 and 1, S(0) and S(1) are defined to be contain only Wedgelet index 0, which corresponds to the pattern that has top-left sample in one partition and all the remaining pixels in the other partition.

Authors of [1] propose to only search Wedgelet pattern in set S(j) for DMM3, where j is the intra prediction mode of the top-left 4×4 block of the CTLB.

Table 1. H,V For each intra mode

| Intra Mode Index | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| V | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | -2 | -5 | -9 | -13 | -17 | -21 | -26 | -32 |
| Intra Mode Index | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| H | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |  |
| V | -32 | -32 | -32 | -32 | -32 | -32 | -32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |  |

# Proposed Solution

In this contribution, we further simplify the DMM3 mode based on [1]. The motivation behind our approach lies in two points. Firstly, in DMM3, the “best” Wedgelet prediction pattern is searched on reconstructed CTLB rather than on original depth block. Therefore, the selected “best” pattern may not be the optimal pattern. Secondly, at the decoder, although [1] reduces the number of candidate Wedgelet patterns dramatically, it still has to perform Wedgelet pattern search. Based on above two considerations, we make two modifications based on [1] to further simplify the decoder of DMM3.

- At the encoder, we search the best Wedgelet pattern on original depth block (instead of on reconstructed CTLB) as follows:

(2)

where ID(i) represents the original depth value of pixel i, ID(R1) represents the DC value of the first partition region and ID(R2) represents the DC value of the second partition region.

- Signal index of the best Wedgelet pattern in the predefined candidate Wedgelet pattern set. Therefore, the decoder does not perform any Wedgelet pattern search.

# Experimental Results

Proposed method is implemented based on JCT2-A0098 and is integrated into HTM 4.0.1 software, and proposed method is also compared with HTM 4.0.1 and JCT2-A0098. The test condition is CTC [2].

Proposed method is compared with HTM-4.0.1 in Table 1. As can be seen, proposed method can save 2% decoding time with negligible influence on compression efficiency.

Table 1: comparison of HTM-4.0.1 with proposed method

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97.6% | 99.2% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 102.5% | 98.4% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.7% | 95.4% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.4% | 96.5% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 100.3% | 99.7% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.3% | 98.5% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 97.8% | 98.8% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.6% | 97.6% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.2% | 98.4% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 98.0% |

We also compare proposed method with JCT2-A0098 in Table 2. As can be seen, proposed method has negligible influence on compression efficiency or even -0.1% gain on synthesized view. Though the decoding time is not reduced obviously, calculations in Wedgelet pattern search are avoided at decoder.

Table 1: comparison of HTM-4.0.1 with proposed method

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 100.3% | 100.5% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 100.3% | 100.4% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 102.4% | 100.0% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 101.1% | 100.4% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | 98.6% | 100.5% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 100.2% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 99.6% | 100.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.0% | 100.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 99.7% | 100.3% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 100.3% | 100.3% |

# Conclusion

This proposal avoids Wedgelet pattern search in DMM3 at decoder and it has negligible influence on compression efficiency. We recommend that proposed method is adopted into 3DV-HEVC.

# Reference

[1] X. Zhao, Y. Chen, L. Zhang, M. Karczewicz, 3D-CE6.h related: Depth Modeling Mode (DMM) 3 simplification for HTM, ISO/IEC JTC1/SC29/WG11 MPEG, Doc. JCT2-A0098, Stockholm, 16–20, July 2012.

[2] D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, ISO/IEC JTC1/SC29/WG11, Doc. JCT2-A1100, July 2012, Stockholm, Sweden.

# Patent rights declaration(s)

LG Electronics / LG Electronics (China) R&D Center and Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).