| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0065 |
| --- | --- |

| Title: | Motion Parameter Inheritance Improvement by Intel |  |  |
| --- | --- | --- | --- |
| Status: | Input Document by the JCT2 |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 Zhipin Deng 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Xiaoxia Cai 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Hong Jiang 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com 86-10-82171372 zhipin.deng@intel.com 86-10-82171425 lidong.xu@intel.com 86-10-82171657 wenhao.zhang@intel.com 86-10-82171388 yu.han@intel.com 86-10-82171674 xiaoxia.cai@intel.com 916-386-0457 hong.h.jiang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

In 3DV-HTM 4.0, there is an MPI mode to allow the information of motion parameters of depth CU to be inherited from the collocated video CU. The MPI mode limits the partition size to NxN for the processes of motion compensation and TU size derivation. This limitation prevents the use of MPI mode when AMP mode is on, but it causes 3DV-HTM 4.0 to crash when "QuadtreeTUMaxDepthInter" is set to 1. The contribution provides a fix by proposing when MPI to be indicated at the level of the coding tree of depth map relative to the CU size that is same as the CU size used in video channel, the information of the partition size and the motion data are inherited from the collocated video channel. This proposal reports the removal of the software crash problem and the enabling of MPI mode when AMP is on, at the same time that an average 0.1% BD bitrate reduction for the coded and synthesized views is achieved when compared to the anchor under common test condition.

# Problem Statement and the Proposed Method

In 3DV-HTM 4.0, there is an MPI mode to allow treeblock subdivision and motion parameters of depth CU to be inherited from the collocated video CU [1]. MPI mode could be indicated at two cases. In the case when MPI mode is indicated at a higher level of coding tree of depth map relative to the CU size that is larger than the CU size used in video channel, the information of CU/PU subdivision and of motion data can be inherited from the video channel. In the other case when MPI mode is indicated at the level of the coding tree of depth map relative to the CU size that is same as or smaller than the CU size used in video channel, only the information of motion data can be inherited from the video channel. In 3DV-HTM 4.0, the partition size of an MPI CU is defined as 2Nx2N. But there is a reported problem about software crash for MPI mode when RQT is off.

## The potential problem if AMP on

In 3DV-HTM 4.0, the MPI mode assigns inter partition size to be NxN for the processes of motion compensation and of TU size derivation after the motion data is inherited from the video channel [2, 3]. The NxN assignment of motion compensation for MPI mode may result in inaccurate motion vectors when AMP is active. For example, if the inter partition size of the collocated video CU is nLx2N, as shown in Figure 1, the inherited motion data of the area shaded in slashes is MV2. But 3DV-HTM 4.0 will produce the mapping of the area shaded in slashes to erroneously use MV1 for the process of motion compensation. This cause inaccurate coding results because that the area shaded in slashes use inaccurate motion parameters during the process of motion compensation.

Figure 1 - Illustration of the motion compensation of an MPI CU in 3DV-HTM 4.0 software

This contribution proposes that when MPI to be indicated at the level of the coding tree of depth map relative to the CU size that is same as the CU size used in video channel, the information of the partition size and the motion data are inherited from the collocated video channel. With this, the current inter partition size is the same as the co-located video signal for the process of motion compensation, as depicted in Figure 2.

Figure 2 - Illustration of the motion compensation of an MPI CU of the proposed method

## Implicit TU split process

In 3DV-HTM 4.0, the inter partition size, assigned to NxN, of TU size derivation for MPI mode has a potential issue. In HEVC, there is an implicit split decision [4, 5] to apply to the derivation process of TU's size in the case when "QuadtreeTUMaxDepthInter" is set to 1. In the case of "QuadtreeTUMaxDepthInter" equal to 1, RQT root location moves from CU level to the lower subdivision level inside the CU for the scenario of current inter partition size to be 2NxN, Nx2N, or NxN. It implies “transform_split_flag/ trafoDepth” to be implicitly changed from 0 to 1 for the cases of 2NxN, Nx2N, or NxN. On the other hand, in the case of the current inter partition size to be 2Nx2N, the “transform_split_flag/ trafoDepth” to be implicitly 0.

3DV-HTM 4.0 has been reported a software crash problem when "QuadtreeTUMaxDepthInter" equal to 1. In the above-mentioned, in the case of "QuadtreeTUMaxDepthInter" equal to 1, with the assigned inter partition size to be NxN, the implicit value of “transform_split_flag/ trafoDepth” is 1. On the other hand, in the process of coefficient encoding/decoding, the partition size of an MPI CU, defined to be 2Nx2N, leads to the implicit value of “transform_split_flag/ trafoDepth” to be 0. This contradicts “transform_split_flag/ trafoDepth” between the TU size derivation process and the coefficient encoding/decoding process, but it causes undesirable software crash.

This contribution proposes that the partition size of an MPI CU in the process of coefficient encoding/decoding should be set as the same size as the assigned partition size in the process of TU size derivation. With this, the value of “transform_split_flag/ trafoDepth” in the process of TU size derivation is coincident with the value of “transform_split_flag/ trafoDepth” in the process of coefficient encoding/decoding.

# Simulation results

The proposed method is integrated into HTM-4.0 [3] and the tests are pursued under the common test conditions [5], and results are shown in Table 1 and Table 2. It reports 0.1% BD bitrate reduction is achieved for the coded and synthesized views when compared to the anchor, HTM-4.0 with AMP on. The detail data can be found in the spreadsheet of JCT3V-B0065.xls.

Table 1 – Simulation result of the proposed method compared with 3DV-HTM 4.0 anchor (3-view case)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 102.8% | 100.4% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 102.5% | 98.3% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 103.1% | 99.0% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 101.8% | 102.0% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 103.1% | 101.2% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 102.9% | 101.9% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | 102.8% | 103.3% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 102.8% | 99.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 102.7% | 102.1% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 102.7% | 100.9% |

Table 2 – Simulation result of the proposed method compared with 3DV-HTM 4.0 anchor (2-view case)

|  | video 0 | video 1 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| UndoDancer | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |

# Conclusions

In 3DV-HTM 4.0, there is an MPI mode to allow the information of motion parameters of depth CU to be inherited from the collocated video CU. The MPI mode limits the partition size to NxN for the processes of motion compensation and TU size derivation. This limitation prevents the use of MPI mode when AMP mode is on, but it causes 3DV-HTM 4.0 to crash when "QuadtreeTUMaxDepthInter" is set to 1. The contribution provides a fix by proposing when MPI to be indicated at the level of the coding tree of depth map relative to the CU size that is same as the CU size used in video channel, the information of the partition size and the motion data are inherited from the collocated video channel. This proposal reports the removal of the software crash problem and the enabling of MPI mode when AMP is on, at the same time that an average 0.1% BD bitrate reduction for the coded and synthesized views is achieved when compared to the anchor under common test condition.

# References

- "Test Model under Consideration for HEVC based 3D video coding v3.0," ISO/IEC JTC1/SC29/WG11 MPEG2012/N12744, Apr. 2012.

- 3DV-HTM version 4.0.1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-4.0.1/.

- 3DV-HTM version 4.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-4.0/.

- K. Panusopone, X. Fang, L. Wang, “Efficient TU representation,” JCT-VC document, JCTVC-D250, Jan. 2011.

- K. Panusopone, X. Fang, L. Wang, “Proposal on RQT root location,” JCT-VC document, JCTVC-E364, March 2011.

- "Common test conditions for 3DV experimentation," ISO/IEC JTC1/SC29/WG11 JCT2-A1100, July. 2012.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).