| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0067 |
| --- | --- |

| Title: | AHG12: On conformance testing of the MVC plus depth extension of AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki and Shinobu Hattori 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com Shinobu.Hattori@jp.sony.com |
|  | Dmytro Rusanovskyy Visiokatu 3, 33720Tampere, Finland | Tel: Email: | +358 7180 08000 Dmytro.Rusanovskyy@nokia.com |
| Source: | Sony corporation and Nokia corporation |  |  |

_____________________________

# Abstract

This contribution proposes the initial idea of conformance testing of Multiview plus depth extension of AVC. The features to be tested by conformance testing are proposed as a starting point of the discussion. The volunteers for missing features should be identified during Shanghai meeting. The initial draft of the conformance testing is also attached to this contribution.

# Conformance bitstreams

This contribution proposes to check the following features by conformance testing.

1. Depth Resolutions

a. Full Resolution texture + Reduced Resolution depth

b. FR texture + FR depth

2. Views configurations

a. C2 (2 view)

b.C3 (3 view)

3. Inter-view prediction configurations

a.	PIP inter-view prediction

b.	IBP inter-view prediction

4. Inter prediction configurations

a.	IPPP

b.	IBP

5. Interlace cofing

a.	Progressive texture + interlace depth

b.	Interlace texture + interlace depth

To summarize, the following is the proposed table of bitstreams.

| Categories | Bitstream | Donated by | File Name | Multi-view Depth High | Level | Frame Rate (Frame/Sec) |
| --- | --- | --- | --- | --- | --- | --- |
| Depth Resolution | Ballons (d0.5) | Nokia |  | X | 3 and higher | 30 |
|  | Street (d0.5) | Nokia |  | X | 4 and higher | 25 |
|  | Ballon (d1.0) | Sony |  | X | 3 and higher | 30 |
|  | Street (d1.0) | Sony |  | X | 4 and higher | 25 |
|  |  |  |  | X |  |  |
|  |  |  |  | X |  |  |
| Views Configuration | Ballon (C2) | Sony |  | X | 3 and higher | 30 |
|  | Street (C2) | Sony |  | X | 4 and higher | 25 |
|  | Ballon (C3) | Nokia |  | X | 3 and higher | 30 |
|  | Street (C3) | Nokia |  | X | 4 and higher | 25 |
| Inter View Prediction | Street (PIP) | Nokia |  | X | 4 and higher | 25 |
|  | Ballon (IBP) | Sony |  | X | 3 and higher | 30 |
|  |  |  |  | X |  |  |
|  |  |  |  | X |  |  |
|  |  |  |  | X |  |  |
|  |  |  |  | X |  |  |
| Inter Prediction | IPP |  |  | X |  |  |
|  | IBP |  |  | X |  |  |
| Interlaced Coding Tools | Prog view + interlace d |  |  |  |  |  |
|  | Interlace view + interlace d |  |  |  |  |  |

Volunteers to generate the missing features should be identified during Shanghai meeting.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).