# WD text of the proposal (corresponding to H3 experiment)

Coding quadtree syntax (added syntax is marked in red)

| coding_quadtree( x0, y0, log2CbSize, ctDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples && log2CbSize > Log2MinCbSize && NumPCMBlock = = 0 && (TextureModeDepth[x0 >> Log2MinCbSize][y0 >> Log2MinCbSize] = = -1 ¦¦ ctDepth < TextureModeDepth[x0 >> Log2MinCbSize][y0 >> Log2MinCbSize]) && (!depth_flag ¦¦ split_texture_cu_flag[ x0 ] [ y0 ] == 1)) |  |
| split_cu_flag[ x0 ][ y0 ] | ae(v) |
| if( ( cu_qp_delta_enabled_flag && log2CbSize >= Log2MinCuQpDeltaSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDelta = 0 |  |
| } |  |
| if( split_cu_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| coding_quadtree( x0, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_quadtree( x1, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x0, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x1, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| } else { |  |
| if( NumPCMBlock = = 0 ) |  |
| coding_unit( x0, y0, log2CbSize , ctDepth) |  |
| else |  |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } |  |
| } |  |

Coding unit syntax (added syntax is marked in red)

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I && TextureModeDepth[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] = = -1) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| if( TextureModeDepth[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] = = 1 ) { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) { |  |
| if ( !depth_flag ¦¦ (log2CbSize == Log2MinCbSize && PartModeTexture[ x0 ][ y0 ] == SIZE_NXN) ¦¦ split_texture_cu_flag[ x0 ][ y0 ] == 1) |  |
| part_mode | ae(v) |
| Else |  |
| PartMode = PART_2Nx2N |  |
| } |  |
| … |  |