| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 October 2012 | Document: JCT3V-B0068 |
| --- | --- |

| Title: | 3D-CE3.h: Depth Quadtree Prediction for 3DHTM 4.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Jung, E. Mora 38 Rue Général Leclerc 92130 Issy-les-Moulineaux France | Tel: Email: | +33145295366 joelb.jung@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

At the 1st JCT-3V meeting, a modification of the depth quadtree decoding process has been proposed in JCT3V-A0044. For a given CTU, the quadtree of the depth is linked to the collocated CTU quadtree in the texture, in a way that a given CU of the depth cannot be split more than its collocated CU in the texture. The quadtree of the depth is predictively encoded relatively to the quadtree of the texture. Experimental results in the new 3DHTM 4.1 are provided. Additionally, a simplification of the rectangular partitions handling is proposed. A 68% encoder runtime is reported compared to the reference (100% for the decoder), while BDR gains of 0.3% are observed for coded + synthesized views, under common test conditions.

# Introduction

HEVC splits a given CTB into smaller CUs, based on RD optimized decisions. A corresponding quadtree (QT) is obtained for the texture, and another one for the depth. The HTM4.1 makes full investigation of every possible configuration, for both texture and depth, and encodes both QTs separately. A modification of the depth quadtree decoding process is proposed to save bits and encoding runtime.

# Proposed method

The proposed method performs a prediction of the depth QT, using the texture QT. Figure 1 shows a simple example of QT partitioning for depth and texture. Table 1 shows how both QTs are represented,  in terms of split flags (0 = not split ; 1 = split) and partitions sizes (0 = 2Nx2N ; 1 = Nx2N ; 2 = 2NxN ; 3 = NxN), before being sent to the entropy coder.

Figure 1: Example of texture (left) and depth (right) QTs.

| DepthLevel | 0 | 1 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 2 | 1 | 1 | 1 | 2 | 2 | 2 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TextureSplitFlag | 1 | 1 | 0 | 0 | 1 | - | - | - | - | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| TexturePartSize | - | - | 0 | 0 | - | 0 | 0 | 0 | 0 | 1 | 2 | 0 | - | 0 | 0 | 0 | 0 |
| DepthSplitFlag | 1 | 1 | 0 | 0 | 0 |  |  |  |  | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| DepthPartSize | - | - | 0 | 0 | 0 |  |  |  |  | 1 | 0 | 0 | - | 0 | 0 | 0 | 0 |

Table 1: Representation of depth and texture QTs corresponding to Figure 1.

The proposed method forces the encoder to limit the partitioning of the depth at the same level as the partitioning of the texture, as shown in Figure 2. For a given CTU, the quadtree of the depth is linked to the collocated CTB quadtree in the texture, so that a given CU of the depth cannot be split more than its collocated CU in the texture. The example in Figure 1 respects this principle.

Figure 2: Texture partitions and corresponding possible depth partitions using our tool

The proposed method applies the convention from Table 2 to predict depth from texture. The basic idea is not to send a depth split flag if the CU of the texture is not further split. The depth partition size could also not be sent if the texture is not split and its partition size is 2Nx2N.

| Texture SplitFlag | Texture PartSize | Depth SplitFlag | Depth PartSize | ResidualDepth SplitFlag | ResidualDepth PartSize |
| --- | --- | --- | --- | --- | --- |
| 1 | - | 1 | - | 1 | - |
| 1 | - | 0 | 0, 1, 2 or 3 | 0 | 0, 1, 2 or 3 resp |
| 0 | 0 or 3 | 0 | 0 | - | - |
| 0 | 1 | 0 | 0 or 1 | - | 0 or 1 resp |
| 0 | 2 | 0 | 0 or 2 | - | 0 or 2 resp |
| - | 0 | - | 0 | - | - |
| - | 1 | - | 0 or 1 | - | 0 or 1 resp |
| - | 2 | - | 0 or 2 | - | 0 or 2 resp |
| - | 3 | - | 0, 1, 2 or 3 | - | 0,1, 2 or 3 resp |

Table 2: Computation of the residual depth.

Applying this convention to the example in Figure 1, the residual depth will be composed of the following:

| ResidualDepthSplitFlag | 1 | 1 | - | - | 0 |  |  |  |  | - | - | - | 1 | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ResidualDepthPartSize | - | - | - | - | 0 |  |  |  |  | 1 | 0 | - | - | - | - | - | - |

Table 3: Residual depth for Figure 1.

The quadtree limitation and the corresponding predictive coding of the depth quadtree is applied to Inter slices. It could also be applied to Intra slices (H1 experiment) or not (H2 experiment).

## Simplification of rectangular partition handling

A slight simplification to the above proposal is proposed, dealing with 2NxN and Nx2N partitions, which is the following:

- Above proposal: when the texture is split in 2NxN (resp Nx2N), depth allows 2NxN (resp Nx2N) partitioning and disallows Nx2N (resp 2NxN) partitioning and NxN partitioning.

- Proposed simplification: when the texture is split in 2NxN (or Nx2N), depth disallows 2NxN, Nx2N, and NxN partitioning.

Figure 3: Texture partitions and corresponding possible depth partitions using our tool with the simplification of rectangular partition handling

This simplification allows us to save more bits as the partition size for depth does not need to be sent in case the texture is not split and its partition size is equal to 2Nx2N, Nx2N or 2NxN. The prediction now follows the even simpler convention described in Table 4.

| Texture SplitFlag | Texture PartSize | Depth SplitFlag | Depth PartSize | ResidualDepth SplitFlag | ResidualDepth PartSize |
| --- | --- | --- | --- | --- | --- |
| 1 | - | 1 | - | 1 | - |
| 1 | - | 0 | 0, 1, 2 or 3 | 0 | 0, 1, 2 or 3 resp |
| 0 | 0,1, 2 or 3 | 0 | 0 | - | - |
| - | 0, 1 or 2 | - | 0 | - | - |
| - | 3 | - | 0, 1, 2 or 3 | - | 0,1, 2 or 3 resp |

Table 4: Computation of the residual depth with the proposed simplification

With this additional constraint on depth partitioning, the example in Figure 1 becomes as follows:

Figure 4: Example of texture (left) and depth (right) QTs with simplification

Applying the convention in Table 4 to the example in Figure 4, the residual depth will be composed of the following:

| ResidualDepthSplitFlag | 1 | 1 | - | - | 0 |  |  |  |  | - | - | - | 1 | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ResidualDepthPartSize | - | - | - | - | 0 |  |  |  |  | - | - | - | - | - | - | - | - |

Table 5: Residual depth for Figure 1 using the proposed simplification

The H3 experiment studies this simplification, while disabling the entire tool (depth limiting + predictive coding) on Intra slices.

# Experimental results

## Quick summary

| 3D HTM4.1 | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- |
| H3 experiment | 0.4% | -0.3% | 68% | 100% | 100% |
| H2 experiment | 0.3% | -0.3% | 75% | 104% | 107% |
| H3 experiment without predictive coding | 0.7% | 0% | 71% | 102% | 104% |
| H2 experiment without predictive coding | 0.7% | 0% | 72% | 107% | 111% |

## Experiment H3 of CE3: simplification for rectangular partitions

It is observed that the BDR gains remain nearly similar, and the runtimes are slightly reduced. The major impact of this modification is however to avoid some conditional processing during the parsing.

Reference: 3DHTM4.1

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 0,0% | 1,0% | 0,1% | 69,1% | 99,7% | 94,5% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,0% | 0,6% | -0,1% | 68,4% | 99,3% | 97,3% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 0,0% | 1,6% | 0,3% | 65,6% | 101,0% | 104,6% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,6% | 68,8% | 102,3% | 112,2% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | -0,8% | 68,8% | 99,9% | 96,1% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,4% | 67,3% | 97,5% | 90,9% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,4% | 66,5% | 104,3% | 102,4% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,0% | 1,0% | 0,1% | 67,7% | 100,0% | 98,7% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,6% | 67,9% | 101,0% | 100,1% |
| average | 0,0% | 0,0% | 0,0% | 0,0% | 0,4% | -0,3% | 67,8% | 100,5% | 99,5% |

Table 6: Experimental results - 3 views test case (negative values are gains)

|  | video 0 | video 1 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 1,3% | 0,3% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,5% | -0,2% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 2,3% | 0,5% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,1% | -0,4% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,3% | -0,7% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,0% | -0,4% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,2% | -0,2% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 1,4% | 0,2% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,2% | -0,4% |
| average | 0,0% | 0,0% | 0,0% | 0,7% | -0,2% |

Table 7: Experimental results - 2 views test case (negative values are gains)

The following tables provide the results when the predictive coding is turned off (only the QT limitation is on):

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 0,0% | 1,3% | 0,4% | 73,3% | 102,9% | 105,9% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,0% | 0,9% | 0,2% | 66,8% | 101,2% | 104,1% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 0,0% | 2,0% | 0,7% | 69,2% | 102,7% | 120,0% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | -0,4% | 75,2% | 99,6% | 96,4% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,0% | 0,6% | -0,3% | 67,8% | 100,7% | 99,4% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,0% | 0,2% | -0,2% | 73,0% | 100,3% | 100,2% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,0% | 0,2% | -0,2% | 74,1% | 107,8% | 101,1% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,0% | 1,4% | 0,4% | 69,7% | 102,3% | 109,8% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,0% | 0,3% | -0,3% | 72,5% | 102,0% | 99,3% |
| average | 0,0% | 0,0% | 0,0% | 0,0% | 0,7% | 0,0% | 71,3% | 102,1% | 103,6% |

Table 8: Experimental results without predictive coding- 3 views test case (negative values are gains)

|  | video 0 | video 1 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 1,6% | 0,5% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,8% | 0,1% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 2,7% | 0,9% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,3% | -0,2% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,8% | -0,2% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,2% | -0,2% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,4% | 0,0% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 1,7% | 0,5% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,4% | -0,2% |
| average | 0,0% | 0,0% | 0,0% | 1,0% | 0,1% |

Table 9: Experimental results without predictive coding - 2 views test case (negative values are gains)

## Experiment H2 of CE3: method disabled on Intra slices

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 0,0% | 0,9% | 0,1% | 77,8% | 107,7% | 110,2% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,0% | 0,5% | -0,2% | 71,7% | 102,2% | 104,8% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 0,0% | 1,6% | 0,3% | 69,3% | 103,3% | 116,2% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,0% | -0,2% | -0,6% | 76,7% | 102,6% | 110,8% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -1,0% | 76,2% | 101,6% | 103,3% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,4% | 72,6% | 105,3% | 100,2% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,0% | -0,2% | -0,5% | 80,7% | 102,4% | 104,6% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,0% | 1,0% | 0,1% | 72,8% | 104,4% | 110,3% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,0% | -0,2% | -0,6% | 76,5% | 103,0% | 104,7% |
| average | 0,0% | 0,0% | 0,0% | 0,0% | 0,3% | -0,3% | 74,9% | 103,6% | 107,0% |

Table 10: experimental results - 3 views test case (negative values are gains)

|  | video 0 | video 1 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 1,3% | 0,2% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,4% | -0,2% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 2,3% | 0,6% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,0% | -0,4% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,2% | -0,8% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,0% | -0,4% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,1% | -0,3% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 1,3% | 0,2% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,1% | -0,5% |
| average | 0,0% | 0,0% | 0,0% | 0,6% | -0,2% |

Table 11: Experimental results - 2 views test case (negative values are gains)

The following tables provide the results when the predictive coding is turned off (only the QT limitation is on):

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 0,0% | 1,2% | 0,4% | 74,8% | 102,1% | 104,8% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,0% | 0,8% | 0,1% | 67,5% | 101,8% | 106,2% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 0,0% | 2,0% | 0,7% | 76,7% | 112,0% | 129,3% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | -0,4% | 73,1% | 102,4% | 115,4% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,0% | 0,4% | -0,4% | 67,3% | 107,8% | 109,4% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | -0,2% | 72,0% | 118,2% | 111,1% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | -0,3% | 75,8% | 104,6% | 104,7% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,0% | 1,3% | 0,4% | 72,9% | 105,2% | 112,9% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | -0,3% | 72,0% | 108,1% | 110,1% |
| average | 0,0% | 0,0% | 0,0% | 0,0% | 0,7% | 0,0% | 72,4% | 106,8% | 111,3% |

Table 12: Experimental results without predictive coding- 3 views test case (negative values are gains)

|  | video 0 | video 1 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 1,5% | 0,5% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,7% | 0,1% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 2,7% | 0,9% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,2% | -0,2% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,7% | -0,3% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,2% | -0,2% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,3% | -0,1% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 1,6% | 0,5% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,3% | -0,2% |
| average | 0,0% | 0,0% | 0,0% | 0,9% | 0,1% |

Table 13: Experimental results without predictive coding - 2 views test case (negative values are gains)

## Experiment H1 of CE3: method enabled on Intra slices

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 0,0% | 1,1% | 0,2% | 71,6% | 105,2% | 99,9% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,0% | 0,6% | -0,1% | 64,9% | 99,9% | 92,9% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 0,0% | 2,1% | 0,6% | 64,5% | 100,3% | 103,5% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,5% | 71,6% | 109,2% | 110,6% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,0% | 0,3% | -0,7% | 60,2% | 96,8% | 87,7% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,0% | 0,4% | -0,1% | 67,9% | 99,8% | 99,9% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | -0,4% | 63,4% | 101,5% | 106,4% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,0% | 1,3% | 0,2% | 66,9% | 101,8% | 98,7% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,0% | 0,2% | -0,4% | 65,6% | 101,7% | 100,8% |
| average | 0,0% | 0,0% | 0,0% | 0,0% | 0,6% | -0,1% | 66,2% | 101,7% | 99,9% |

Table 14: Experimental results - 3 views test case (negative values are gains)

|  | video 0 | video 1 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 1,5% | 0,3% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,5% | -0,2% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 2,9% | 0,8% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,1% | -0,4% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,6% | -0,5% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,4% | -0,2% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,3% | -0,2% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 1,7% | 0,3% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,3% | -0,3% |
| average | 0,0% | 0,0% | 0,0% | 0,9% | 0,0% |

Table 15: Experimental results - 2 views test case (negative values are gains)

The following tables provide the results when the predictive coding is turned off (only the QT limitation is on):

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 0,0% | 1,4% | 0,5% | 72,9% | 106,7% | 112,2% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,0% | 1,0% | 0,2% | 67,6% | 102,0% | 106,8% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 0,0% | 2,6% | 1,1% | 65,5% | 102,8% | 115,6% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | -0,3% | 74,0% | 101,4% | 104,5% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,0% | 0,9% | -0,1% | 68,2% | 100,1% | 100,4% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,0% | 0,8% | 0,2% | 65,8% | 99,2% | 98,0% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,0% | 0,3% | -0,1% | 72,6% | 101,0% | 110,1% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,0% | 1,6% | 0,6% | 68,6% | 103,8% | 111,5% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,0% | 0,5% | -0,1% | 70,1% | 100,4% | 103,1% |
| average | 0,0% | 0,0% | 0,0% | 0,0% | 1,0% | 0,2% | 69,4% | 101,9% | 106,6% |

Table 16: Experimental results without predictive coding- 3 views test case (negative values are gains)

|  | video 0 | video 1 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 1,8% | 0,6% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,9% | 0,1% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 3,4% | 1,3% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,3% | -0,2% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 1,1% | 0,0% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,7% | 0,1% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,5% | 0,0% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 2,0% | 0,7% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,7% | 0,0% |
| average | 0,0% | 0,0% | 0,0% | 1,2% | 0,3% |

Table 17: Experimental results without predictive coding - 2 views test case (negative values are gains)

# Conclusion

# A modification of the depth quadtree decoding process is proposed. For a given CTU, the quadtree of the depth is linked to the collocated CTB quadtree in the texture, so that a given CU of the depth cannot be split more than its collocated CU in the texture. The quadtree of the depth is predictively encoded. 68% runtime encoder is achieved compared to the reference, while BDR gains of 0.3% are observed for coded + synthesized views, under common test conditions.

Given the runtime improvement of the encoder jointly with BDR gains, we propose this method for adoption in the HTM and in the working draft.

# Patent rights declaration(s)

Orange Labs may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).