| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0072 |
| --- | --- |

| Title: | 3D-CE1.h: Cross check report of JCT2-B0045 on Illumination Compensation for Inter-View Prediction of LG |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to the JCT2 |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gun Bang, *Young Su Heo, *Kyung Yong Kim, *Gwang Hoon Park, Won-Sik Cheong, Namho Hur ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5069 {gbang, wscheong, namho} @etri.re.kr +82-31-201-3680 {*medist, *kimky, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for CE2 experiment Illumination Compensation for Inter-View Prediction [1], [2]. The R-D performance matched with the data provided by LG. We found no problem in compiling, building, encoding and decoding. Common test conditions specified in JCT3V-A1100 [3] is used for the evaluation. In the simulation, the anchor is 3DV-HTM v4.0.1. Bit rates and PSNR results were compared: our experiment results were almost exactly matched with those provided by LG.

The following sets are tested:

- Set 1: enable weighted prediction

- Set 2: enable illumination compensation on Luma

Table 1.Simulation result of IC
(Anchor: 3DV-HTM v4.0.1, Tested: enable weighted prediction (Set 1))

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.1% | 0.4% | 0.4% | 0.2% | 0.5% | 0.5% | 109.3% | 103.0% | 100.0% |
| Kendo | 0.1% | 0.5% | 0.0% | 0.1% | 1.1% | 1.0% | 153.9% | 102.1% | 100.0% |
| Newspapercc | 0.1% | 0.4% | 0.4% | 0.2% | 0.5% | 0.5% | 120.8% | 106.2% | 100.0% |
| GhostTownFly | 0.4% | 0.3% | 0.3% | 0.4% | -0.1% | 0.0% | 165.0% | 105.5% | 100.0% |
| PoznanHall2 | 0.1% | 0.5% | 0.5% | 0.3% | 1.0% | 0.9% | 129.4% | 102.3% | 100.0% |
| PoznanStreet | 0.0% | 0.3% | 0.3% | 0.1% | 0.2% | 0.2% | 106.5% | 96.0% | 100.0% |
| UndoDancer | 0.0% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% | 128.0% | 97.7% | 100.0% |
| 1024x768 | 0.1% | 0.4% | 0.2% | 0.2% | 0.7% | 0.6% | 126.7% | 103.7% | 100.0% |
| 1920x1088 | 0.2% | 0.3% | 0.3% | 0.2% | 0.3% | 0.3% | 130.6% | 100.3% | 100.0% |
| average | 0.1% | 0.4% | 0.3% | 0.2% | 0.4% | 0.4% | 128.9% | 101.8% | 100.0% |

Table 2.Simulation result of IC
(Anchor: 3DV-HTM v4.0.1, Tested: enable illumination compensation on Luma (Set 2))

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.4% | -1.1% | -0.6% | -0.4% | -0.5% | 109.1% | 101.1% | 100.0% |
| Kendo | 0.0% | -3.1% | -3.3% | -1.4% | -1.0% | -1.1% | 114.2% | 100.9% | 100.0% |
| Newspapercc | 0.0% | -0.6% | -0.8% | -0.3% | -0.2% | -0.2% | 108.2% | 101.2% | 100.0% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 105.4% | 100.6% | 100.0% |
| PoznanHall2 | 0.0% | 0.3% | -0.8% | -0.2% | -0.1% | -0.1% | 108.6% | 93.2% | 100.0% |
| PoznanStreet | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | 0.0% | 106.1% | 96.9% | 100.0% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 105.0% | 104.8% | 100.0% |
| 1024x768 | 0.0% | -1.7% | -1.7% | -0.7% | -0.5% | -0.6% | 110.5% | 101.1% | 100.0% |
| 1920x1088 | 0.0% | 0.1% | -0.3% | 0.0% | 0.0% | 0.0% | 106.3% | 98.8% | 100.0% |
| average | 0.0% | -0.7% | -0.9% | -0.3% | -0.2% | -0.3% | 108.0% | 99.8% | 100.0% |

# Reference

[1]	H. Liu, J. Jung, J. Sung, J. Jia and S. Yea, “3D-CE1.h related: Illumination Compensation for Inter-View Prediction”, The 1st JCT-3V meeting, JCT3V-A0086, Stockholm, Sweden, July 2012.

[2]	H. Liu, J. Jung, J. Sung, J. Jia and S. Yea, “3D-CE2.h : Results of Illumination Compensation for Inter-View Prediction”, The 2nd JCT-3V meeting, JCT3V-B0045, Shanghai, China, October 2012.

[3]	D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, The 1st JCT-3V meeting, JCT3V-A1100, Stockholm, Sweden, July 2012.