| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0073 |
| --- | --- |

| Title: | 3D-CE5.a related: Simplification on the disparity vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to the JCT2 |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gun Bang, *Kyung Yong Kim, *Young Su Heo, *Gwang Hoon Park, Won-Sik Cheong, Namho Hur ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5069 {gbang, wscheong, namho} @etri.re.kr +82-31-201-3680 {*kimky, *medist, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution proposes a simplified disparity vector derivation method for saving the memory access bandwidth. The implementation of the proposed process can be achieved with sharing a depth sample to derivate the disparity vector in all sub-blocks inside each MB. It is indicated that the proposed approach reduces searching and comparison operations to find maximum depth samples in every sub-blocks and additionally reduces of the encoder complexity. It is reported that the proposed algorithm increased slightly 0.07% BD rate for texture views and increased slightly 0.02% BD rate for synthesized views with 98% encoding time, and 100% decoding time.

# Introduction

In the last meeting, a simplified disparity vector derivation method was already adopted [1]. The disparity vector is derived by using the maximum depth value in four corner depth samples within each sub-block as shown in Figure 1.

Fig. 1 Derivation of the maximum depth value within the depth block.

By using the method, the number of the accessed depth samples can be reduced from 256 to 4 in the case of a 16x16 MB. But, the 16x16 MB can consist of the 4 sub-blocks as 8x8 partitions, and the 8x8 block can be further partitioned into 4x4 sub-blocks. Because each partitioned block can be coded as the inter-prediction mode, it must provide four depth samples of the corner to find a maximum depth value. Therefore, the number of the required depth sample will be increased by the increment of the number of sub-block.

As shown in Figure. 2(a), in case a 16x16 MB is partitioned into 8x8 partitions, the number of the depth samples to be accessed is 16. As shown in Figure. 2(b), in case a 16x16 MB is partitioned into 4x4 partitions, the number of the depth samples to be accessed is 64. Table 1 show the number of the depth samples to be accessed in a 16x16 MB depending on partition size.

| (a) | (b) |  |
| --- | --- | --- |

Fig. 2 Derivation of the maximum depth value within the depth block: 
(a) in case a 16x16 MB is partitioned into 8x8 partitions, 
(b) in case a 16x16 MB is partitioned into 4x4 partitions

Table 1. The number of the depth samples to be accessed in a 16x16 MB

| partition size | ATM5.0 |
| --- | --- |
| 16x16 partitions | 4 |
| 8x8 partitions | 16 |
| 4x4 partitions (Worst Case) | 64 |

# Proposed Methods

In worst case, the number of the depth samples to be accessed in a 16x16 MB is 64. We propose the method to reduce the memory access bandwidth using only one depth sample in a 16x16 MB.

Fig. 3 Derivation of the depth value in case a 16x16 MB is partitioned into 4x4 partitions

In Fig. 3, each 4x4 partitions use a right down depth sample (green pixel) in the 16x16 MB instead of the four corner depth samples (gray pixel) in 4x4 partition block. Thus, all the partitions inside a 16x16 MB shares a depth sample for deriving the disparity vector. Table 2 show the comparison of the number of the depth samples to be accessed in a 16x16 MB depending on partition size.

Table 2. Comparison of the number of the depth samples to be accessed in a 16x16 MB

| partition size | ATM5.0 | Proposed Method |
| --- | --- | --- |
| 16x16 partitions | 4 | 1 |
| 8x8 partitions | 16 | 1 |
| 4x4 partitions | 64 | 1 |

Main advantages of this simplification are described as follows:

- Memory access bandwidth reduction: significantly reduce the number of the depth samples to be accessed (in worst case, from 64 to 1)

- Remove the searching operation of the depth samples using partition size

- Remove the comparing operation of the depth samples

- Minor coding loss: 0.07% and 0.02% BD-rate increase for texture and synthesized views, respectively

- Complexity Reduction for encoding

- Simple WD text

# Simulation Results

The following cases are tested as shown in Figure. 4:

- Case 1: a depth sample of the top left corner

- Case 2: a depth sample of the top right corner

- Case 3: a depth sample of the bottom left corner

- Case 4: a depth sample of the bottom right corner

- Case 5: a depth sample of the center

| (a) case 1 | (b) case 2 | (c) case 3 | (d) case 4 | (e) case 5 |
| --- | --- | --- | --- | --- |

Fig. 4. Cases to be tested

Common test conditions specified in JCT2-A1100 [2] is used for the evaluation. The following Table 3 shows the simulation results of the proposed method. The best performance is case 4. The following Table 4 shows the simulation result of the case 4 method. The proposed method brings slight BD rate loss.

Table 3. Results of the proposed method

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity estimate (ratio to anchor) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR | dPSNR | dBR | dPSNR | dBR | dPSNR | dBR | dPSNR | Enc. time | Dec. time |
| Case 1 | 0.22 % | -0.01 dB | 0.00 % | 0.00 dB | 0.21 % | -0.01 dB | 0.13 % | 0.00 dB | 96 % | 101 % |
| Case 2 | 0.18 % | -0.01 dB | 0.00 % | 0.00 dB | 0.17 % | -0.01 dB | 0.09 % | 0.00 dB | 92 % | 102 % |
| Case 3 | 0.14 % | -0.01 dB | 0.00 % | 0.00 dB | 0.13 % | 0.00 dB | 0.09 % | 0.00 dB | 94 % | 101 % |
| Case 4 | 0.07 % | 0.00 dB | 0.00 % | 0.00 dB | 0.06 % | 0.00 dB | 0.02 % | 0.00 dB | 98 % | 100 % |
| Case 5 | 0.12 % | 0.00 dB | 0.00 % | 0.00 dB | 0.11 % | 0.00 dB | 0.05 % | 0.00 dB | 95 % | 101 % |

Table 4. Results of the proposed method (Case 4: bottom right depth sample)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.03 | 0.00 | 0.00 | 0.00 | -0.02 | 0.00 | -0.06 | 0.00 | 99% | 98% | 100% |
| S02 | 0.06 | 0.00 | 0.00 | 0.00 | 0.06 | 0.00 | 0.04 | 0.00 | 99% | 103% | 100% |
| S03 | 0.14 | 0.00 | 0.00 | 0.00 | 0.13 | 0.00 | 0.09 | 0.00 | 101% | 101% | 100% |
| S04 | 0.12 | 0.00 | 0.00 | 0.00 | 0.12 | 0.00 | -0.12 | 0.00 | 95% | 99% | 100% |
| S05 | 0.09 | 0.00 | 0.00 | 0.00 | 0.08 | 0.00 | 0.11 | 0.00 | 98% | 100% | 100% |
| S06 | 0.06 | 0.00 | 0.00 | 0.00 | 0.05 | 0.00 | 0.04 | 0.00 | 101% | 100% | 100% |
| S08 | 0.02 | 0.00 | 0.00 | 0.00 | 0.02 | 0.00 | 0.04 | 0.00 | 96% | 99% | 100% |
| Average | 0.07 | 0.00 | 0.00 | 0.00 | 0.06 | 0.00 | 0.02 | 0.00 | 98% | 100% | 100% |

# Conclusion

In this contribution, a simplified derivation of the disparity vector is proposed. All the partitions inside a 16x16 MB shares a depth sample for the disparity vector derivation. It is reported that this proposed simplification shows slight 0.07% BD rate loss for texture views and slight 0.02% BD rate loss for synthesized views. It is indicated that the proposed approach reduces the memory access bandwidth and removes the searching and comparing operation of the depth samples using partition size.

# Reference

[1]	J.-L. Lin, Y.-W. Chen, Y.-L. Chang, Y.-P. Tsai, Y.-W. Huang and S. Lei, “3D-CE5.a related: Simplification on the disparity vector derivation for AVC-based 3D video coding”, 1st JCT3V Meeting, JCT3V-A0046, Stockholm, SE, July 2012.

[2]	D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments”, 1st JCT3V Meeting, JCT3V-A1100, Stockholm, SE, July 2012.

# Patent rights declaration(s)

ETRI and KyungHee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex:	Proposed text changes

Insert the yellow marked text into section J.8.3.1.7.1

Remove the cyan marked text in section J.8.3.1.7.1

J.8.3.1.7.1	Modification process for inter view motion vector in median luma motion vector prediction

Inputs to this process are

–	depth reference view component depthPic,

–	the location of a top-left sample ( dbx1, dby1 ) of a partition,

–	the location of a top-left sample (mbx1, mby1 ) of a macroblock partition,

–	a motion vector mv,

Outputs of this process are:

–	the motion vector mv.

Let refViewId be the view_id value of depthPic.

The following ordered steps apply:

- Let numSamples be partWidth * partHeight.

- The variable depthValuemaxDepth is specified as follows:

depthValue = depthPic[ mbx1 + 7, mby1 + 7 ]

maxDepth = INT_MIN
for( j = 0; j < partHeight; j++ )
	for( i = 0; i < partWidth; i++ )		(J-8-XX)
		if( depthPic[ dbx1 + i, dby1 + j ] > maxDepth ) maxDepth = depthPic[ dbx1 + i, dby1 + j ]

- The variable mv is specified as follows:

index = ViewIdTo3DVAcquisitionParamIndex( view_id )
refIndex = ViewIdTo3DVAcquisitionParamIndex( refViewId )

mv[ 0 ] = Disparity( NdrInverse[ depthValuemaxDepth ], ZNear[ frame_num, index ], ZFar[ frame_num, index ],
	FocalLengthX[ frame_num, index ], AbsTX[ index ] – AbsTX[ refIndex ] )
mv[ 1 ] = 0