| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0075 m26809 |
| --- | --- |

| Title: | AHG08: Technical Description of GVD (Global View and Depth) 3D Format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuyoshi Suzuki, Masayuki Tanimoto Yotsuya-dohri 1-13, Chikusa-ku, Nagoya 464-0819, Japan Takanori Senoh Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan | Tel: Email: | +81-52-753-9980 kazuyoshi1028@gmail.com tanimoto@nisri.jp +81-42-327-7262 senoh@nict.go.jp |
| Source: | Nagoya Industrial Science Research Institute (NISRI) National Institute of Information and Communications Technology(NICT) |  |  |

_____________________________

# Abstract

This contribution describes a data format for 3DV coding besides AVC-based or HEVC based. Global View and Depth (GVD) format has been proposed by Nagoya and NICT. In this document, a new GVD format is proposed.

# Introduction

Some proposals including Global View and Depth approach (GVD) and Warp approach proposed their own view synthesizers suitable for their coding scheme [1-7]. Through the core experiments and subjective quality evaluations, these approaches were approved as the Alternative 3D Format, since they are simple and fast comparing to the current 3DV standardization tracks. GVD format has been proposed by Nagoya and NICT [8]. In the previous meeting, subjective evaluation on GVD was made. In this meeting, Nagoya and NICT propose a new GVD format that contains Residual Depths as well as Base View, Base Depth and Residual Views.

# Data Format and System Description

Input Data format is same as those of 3D video coding, three views and associated depth maps as illustrated in Figure 1. These input data format are transformed into GVD format. Global View and Depth generator consists of Base view generator, Base depth generator, Core codec, Residual mask generator, Residual view generator and Residual depth generator. Here, the Core codec is 3DV AVC or HEVC. For a convenience, current test model is using 3DV ATM or HTM with single stream mode.

Components of Global View and Depth are encoded by Core codec. Generated base view, residual views, base depth and residual depths are encoded by using Core codec individually. Here, it should be noted that Residual view 1 (Left residual view) and Residual view 3 (Right residual view) are combined to one sequence, which is encoded for a single stream. In the same manner, Residual depth 1 and Residual depth 3 are combined to one sequence, which is encoded for a single stream.

Target view synthesizer generates target views from decoded Global View and Depth data. Target view synthesizer consists of View2 generator, Depth2 generator, Residual mask generator, View2 projector, Residual view projectors and View synthesizer.

Figure 1. GVD generator

Figure 2. Encode/Decode of Global View and Depth data

Figure 3. Target view synthesizer

# Data Format Transformation and Target View Generation Algorithm

## Global View and Global Depth Generation

Figure 4 indicates a relationship between MVD data format and GVD format for three-view case. Global View and Depth data consists of Global View and Global Depth as shown in Fig. 4. Global view consists of a base view and residual views. Base view may be an averaged view of all input views at the base viewpoint by projecting the input views according to the input depth maps and camera parameters, or may the original view at the base viewpoint. For a simplicity, center view is directly used for base view here. Global depth consists of a base depth map and residual depth maps. Base depth map is an average of all input depth maps by projecting them to the base depth point, or may be the original depth map at the base viewpoint.

Hole masks are generated for extracting residual pixels from non-base views and non-base depth maps. Proposed procedure for generating hole masks is presented in Fig. 5. The base-depth map is sub-sampled in quarter size. The sub-sampled one is encoded and decoded by 3DV ATM or HTM. Decoded base-depth map is up-sampled. The up-sampled one in Fig. 5.(a) is projected to non-base viewpoints. Projected depth maps in Fig. 5. (b1) and (b2) have hole pixels. Minute holes are filled by Median filter as shown in Fig. 5. (c1) and (c2). Remaining hole regions such as occlusion areas are registered to the hole mask of each non-base viewpoint as shown in Fig. 5. (d1) and (d2). Those remaining large holes are dilated. Fig. 5. (e1) and (e2) indicate final residual hole masks.

Non-base views and non-base depth maps are masked by those final hole masks. Left and right residual views and left and right residual depths are stacked and are vertically bound up into single sequences, respectively. Figure 6 provides an example of GVD format data for Kendo sequence.

Obtained base view, base depth map, residual views and residual depth maps are encoded individually by AVC or HEVC. Here, it should be noted that hole masks that are generated at sender side are not sent to receiver side and they are reconstructed at the receiver side.

MVD

Left view           Center view          Right view

Multi view

Left view            Center view           Right view

Multi depth

GVD

Left residual view         Base view         Right residual view

Global view

Left residual depth       Base view       Right residual depth

Global depth

Figure 4. Example of MVD and GVD

(b1) Left projected depth map  (a) Base depth map  (b2) Right projected depth map

Median filter                                Median filter

(c1) Left smoothed depth map                  (c2) Right smoothed depth map

Hole detection                               Hole detection

(d1) Left hole mask                          (d2) Right hole mask

Hole dilation                                Hole dilation

(e1) Final Left residual hole mask                (e2) Final Right residual hole mask

Figure 5. Residual hole masks

- Base View              (b) Base depth

- Binded residual views      (d) Binded residual depths

Figure 6. Components of GVD

## View Synthesis

Figure 7 indicates GVD format and hole masks for synthesizing target views. Residual hole masks, which are completely same as those generated at sender side, are reconstructed from decoded base depth map again at receiver side in the same manner as those ones are generated on the GVD generating process at the sender side. The obtained hole masks are used to specify the areas having residual pixels in residual views and residual depth maps. View synthesizer generates target views by using base view, base depth map, residual views, residual depth maps, and residual hole masks.

First, base view is projected to each target viewpoint with base depth map. The base depth map is projected to target views and resultant holes in target depth maps are filled by Median filter and inpainting. After that, base view is backwardly projected to target views by using the completely filled target depth maps. Only residual pixels of left or right residual views, that are in hole regions of hole masks, are projected to target views by using corresponding residual depth map. The obtained target views are overwritten by the projected residual pixels.

Figure 7. View Synthesis

V1                        V3

Figure 8. Example of Synthesized Views at V1 and V3

# Conclusion

In this document, a new GVD format has been proposed, which contains Residual Depths as well as Base View, Base Depth and Residual Views. Although the previous version of GVD format requires around 50 to 60% of whole picture size for Residual Data, proposed new data format requires around a half of it for Residual Data. Experiment for video sequences has not yet been finished. Nagoya and NICT are going to report the experimental results in the next meeting.

# Patent rights declaration(s)

NISRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NICT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Gary Sullivan and Jens-Rainer Ohm, “Meeting report of the first meeting of the Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V),” ISO/IEC JTC1/SC29/WG11, ITU-T SG 16 WP JCT2-A1000, Stockholm, SE, 16-20 July 2012.

- M. Tanimoto, T. Senoh and N. Stefanoski, “AHG8 Report on Alternative 3D Format,” ISO/IEC JTC1/SC29/WG11 m26749, ITU-T SG 16 WP JCT2-B0008, Shanghai, CN, October 2012.

- ISO/IEC JTC1/SC29/WG11 MPEG2011/N12353: "Description of Core Experiments for 3D-AVC," (CE08: Global Depth and View Prediction), November 2011.

- T. Senoh, K. Yamamoto, R. Oi, Y Ichihashi and T. Kurita, "Technical Description of 3DV proposal P-27," ISO/IEC JTC1/SC29/WG11 MPEG2011/m22573, November 2011.

- N. Stefanoski and A. Smolic, “Coding of Warps for IDW-based View Synthesis in 3DV Applications," ISO/IEC JTC1/SC29/WG11 MPEG2011/m23558, January 2012.

- T. Senoh, "3D-CE7.a summary report on Global Depth and View Prediction," ISO/IEC JTC1/SC29/WG11 MPEG2011/m24984, April 2012.

- T. Senoh, K. Yamamoto, R. Oi, Y. Ichihashi, “3DV CE7.h results on Global Depth and View Prediction by NICT”, ISO/IEC JTC1/SC29/WG11 m26011, ITU-T SG 16 WP JCT2-A0069, Stockholm, SE, July 2012.

- T. Senoh, K. Yamamoto, R. Oi, Y. Ichihashi, M. Tanimoto and K. Suzuki, “Proposal of Depth-aware 3DV Test Condition and Evaluation”, ISO/IEC JTC1/SC29/WG11 m26016, ITU-T SG 16 WP JCT2-A0072, Stockholm, SE, July 2012.

- ISO/IEC JTC1/SC29/WG11 MPEG2011/N12561: "Description of Core Experiments for 3D Video Coding," February 2012.

- ISO/IEC JTC1/SC29/WG11 MPEG2011/N12744: "Test Model under consideration for HEVC based 3D Video Coding V3.0," February 2012.

- ISO/IEC JTC1/SC29/WG11 MPEG2011/N12754: "Common Test Conditions for 3DV Experimentation," May 2012.