| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2st Meeting: Shanghai, SE, 13–19 Oct 2012 | Document: JCT3V-B0084 |
| --- | --- |

| Title: | 3D-CE3.h related: Simplification on motion parameter inheritance |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Jicheng An North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 Jian-Liang Lin, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Siwei Ma School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 Debin Zhao Department of Computer Science and Technology, Harbin Institute of Technology, Harbin, China 150001 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { jicheng.an, jl.lin, shawmin.lei} @mediatek.com Siwei Ma swma@pku.edu.cn Debin Zhao {liang.zhao,dbzhao}@hit.edu.cn |
| Source: | MediaTek Inc, Harbin Institute of Technology, Peking University |  |  |

_____________________________

# Abstract

In HTM4.0.1, the motion parameter inheritance (MPI) mode is signalled as a merge/skip candidate. In this contribution, two methods are proposed to simplify and improve the MPI mode. In the first method, we simplify the MPI mode by limiting the MPI mode only to the non-residual case and moving the MPI mode syntax before skip mode syntax. In the second method, we fixed one bug for the MPI mode when asymmetric partition (AMP) is enabled. In the first method, the experimental results reportedly show that 0.1% BD-Rate gains is achieved for coded and synthesized views with 3% encoding time reduction. In the second method of bug fix, the experimental results reportedly show 0.1% BD-Rate gains for coded and synthesized views is achieved with no impact for complexity.

# Introduction

In the reference software of the HEVC-based 3D video coding version 4.0.1 (HTM v4.0.1) [1], motion parameter inheritance (MPI) mode is employed to explore the correlation between the video signal and its associated depth map. In order to encode the depth map efficiently, MPI allows inheritance of the treeblock subdivision into CUs and PUs and their corresponding motion parameters from the video signal [2]. For each CU of the depth map, the encoder can adaptive decide whether the motion data are inherited from the co-located region of the video signal or if new motion data are transmitted as illustrated in Figure 1. To signal the MPI coding mode, the syntax is integrated into the merge and skip mode. The merging candidate list has been extended by adding the MPI coding mode as the first candidate in the merging candidate list for depth map coding. The syntax table related to MPI mode in HTM4.0.1 is illustrated in Table 1.

Figure 1: Illustration of the concept of motion parameter inheritance [2]

Table 1. Syntax related to MPI

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I && TextureModeDepth[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] = = -1) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| if( TextureModeDepth[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] = = 1 ) { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| ………………………………………………………………………………………. |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Proposed Methods

## Simplificaiton on MVP mode

In this contribution, we simplify the MPI mode by restricting the MPI mode to the skip mode only and modifying the syntax for MPI signalling. The MPI flag is proposed to be signaled before the skip_flag. When the MPI flag is on, the current depth CU will be merged with the corresponding block from the associated video signal and no residual data will be transmitted to the decoder.

Because we remove the MPI signaling from the skip/merge candidate index, for depth map coding, the merging candidate list size is reduced by 1. The parsing process related to the MPI mode for a leaf CU signaled before the skip_flag in this proposed method is illustrated in Table 2.

Table 2. Syntax related to MPI in proposed method

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I && TextureModeDepth[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] = = -1) { |  |
| if (use_mvi_flag) { |  |
| mpi_flag[ x0 ][ y0 ] | ae(v) |
| if(!mpi_flag[ x0 ][ y0 ]) |  |
| skip_flag[ x0 ][ y0 ] |  |
| }else /* !enable_mpi_flag */ |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| if( !( mvi_flag[ x0 ][ y0 ] ¦ ¦ skip_flag[ x0 ][ y0 ] ) ) { |  |
| if( TextureModeDepth[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] = = 1 ) { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| ………………………………………………………………………………………. |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

mpi_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, when decoding a P or B slice, no more syntax elements are parsed after mpi_flag[ x0 ][ y0 ]. mpi_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not MPI mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When mpi_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

## AMP partition for MPI

In HTM4.0.1, if the MPI is selected, the CU will be firstly split into leaf CUs as the splitting in the co-located texture region, and then the leaf CU in depth map will always use Size_NxN partition to perform the motion compensation regardless the partition for the co-located texture block. This implementation method follows the spirit of MPI only when the asymmetrical partition (AMP) is not used in texture coding since the Size_NxN partition has the finest granularity MV assignment for all the symmetrical partitions but not for AMP.

In this method, we fixed this bug by simple change. The implementation is described as follows: if the leaf CU size in depth map is equal to the associated texture CU size, the leaf CU in depth map uses the same partition as the texture to perform the motion compensation; else if the leaf CU size in depth map is less than the associated texture CU size, Size_NxN is used to perform the motion compensation.

# Experiments

The proposed methods are integrated into HTM 4.0.1 [1]. The test of method 1 is conducted under common test conditions [3], and the test of method 2 is conducted under common test condition with AMP enabled. Table 3 and Table 4 shows the results for test of method 1 and method 2 respectively, and the Table 5 shows the results for AMP enabled compared to common test condition. Since the proposed methods only affect the coding depth, in the first method, 0.1% BD-Rate saving can be observed for coded and synthesized views with about 3% encoding time reduction while the second method of bug fix obtained about 0.1% BD-Rate gains for coded and synthesized views with no impact for complexity.

Table 3 Performance of proposed method 1 based on HTM4.0.1

Table 4 performance of proposed method 2 based on HTM4.0.1 with AMP enabled

Table 5 performance of AMP enabled compared to common test condition based on HTM4.0.1

# Conclusion

In this contribution, two methods are proposed to simplify and improve the MPI mode. In the first method, we simplified the MPI mode by about 3% encoder complexity reduction with 0.1% BD-Rate gains for coded and synthesized views. In the second method, we fixed one bug for the MPI mode and obtained about 0.1% BD-Rate gains for coded and synthesized views.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Harbin Institute of Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] HTM-4.0.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-4.0.1/

[2] Heiko Schwarz, Krzysztof Wegner, “Test Model under Consideration for HEVC based 3D video coding v3.0”, ISO/IEC JTC1/SC29/WG11 MPEG2011/N12744, April 2012, Geneva, Switzerland

[3] Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments”,  Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-A1100, July 2012, Stockholm.