| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, SE, 13–19 Oct 2012 | Document: JCT3V-B0086 |
| --- | --- |

| Title: | 3D-CE5.h: Pruning process for the inter-view candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { jl.lin, yiwen.chen,yuwen.huang shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents results of CE5.h experiments related to the improvement of the pruning process proposed by MediaTek in JCT3V-A0048. Seven experiments are conducted for the removal of redundant candidates in the reconstruction of the AMVP candidate list, merging candidate list, and the combinations. Among seven experiments, Test6 provides the best performance in terms of complexity reduction and coding gain. The experiments results of Test6 reportedly show 0.3% and 0.2% BD-rate saving for view 1 and view 2, respectively, and 0.1% BD-rate saving for the coded and synthesized views, while also reducing the required comparisons in the pruning processes of the inter, merge and skip modes.

# Introduction

In the HEVC-based 3D video coding, HTM-4.0.1 [1], a number of parallelizable motion information comparisons between the spatial merging candidates are performed for the removal of redundant candidates in the merging candidate list of the merge and skip modes. The temporal and inter-view merging candidates are exempted from the pruning process. The spatial merging candidates are inserted into the merging candidate list depending on the following specific conditions.

As shown in Figure 1, the conditions for each spatial merging candidate to be excluded from the merging candidate list are as follows:

•	A1: No change (which means A1 is always inserted as long as it is available.)

•	B1: B1 has the same motion information as A1

•	B0: B0 has the same motion information as B1

•	A0: A0 has the same motion information as A1

•	B2: B2 has the same motion information as A1 or has the same motion information as B1 (Note that   B2 is considered only if any of A1, B1, B0 or A0 is excluded from the candidate list.)

Figure 1. Spatial neighbors which are the potential candidates for the merging candidate list. Arrows indicate which spatial merging candidate(s) are to be compared.

As for inter mode in HTM-4.0.1, the temporal motion vector predictor (MVP) candidate is exempted from the pruning process. The pruning process is applied to compare all the spatial and inter-view candidates. A spatial or an inter-view candidate has to be compared to all previous candidates and be removed if any of previous candidates is the same. This pruning process is hard to be implemented in parallel and it also requires more logic to decide the inclusion of a spatial or interview candidate. After that, only the first three available predictors are included in the candidate set. If the number of available predictors is smaller than three, the zero predictor is inserted.

# Experiments

In respond to CE5.h [2], to further investigate ways to improve the AMVP and merging candidate list reconstructions based on the proposed JCT3V-A0048 [3], seven experiments are tested. The tests are integrated into 3DV-HTM-4.0.1 software [1] and the simulations are run under the common test conditions [4].

In Test1, one additional comparison between the inter-view merging candidate and the first spatial merging candidate, A1, is added in the pruning process as shown in Figure 2.

Figure 2. Proposed pruning process for merge mode in Test1.

In Test2, based on Test1, we further remove two comparisons for the spatial candidate B2. Figure 3 illustrates the pruning processes for Test2.

Figure 3. Proposed pruning process for merge mode in Test2.

In Test3, two comparisons between the inter-view merging candidate and the spatial merging candidate A1 and B1 are added in the pruning process as shown in Figure 4.

Figure 4. Proposed pruning process for merge mode in Test3.

Test4 conducts the simplified pruning process in the inter mode. The inter-view MVP candidate and the second spatial MVP candidate, if available, are both directly compared to the first spatial MVP candidate. Instead of comparing all spatial and inter-view candidates to remove the redundancy, at most two parallelizable motion information comparisons are required for the removal of redundant candidates in the MVP candidate list.

The combination of the simplified pruning process in the inter mode, Test4, and the improved pruning processes in the skip and merge modes, Test1, Test2 and Test3, are also conducted in Test5, Test6 and Test7, respectively. The combination of Test1 and Test4 is conducted in Test5. The combination of Test2 and Test4 is conducted in Test6 and the combination of Test3 and Test4 is conducted in Test7.

The results of the seven experiments are summarized in Table 1. The results of Test6 shows that 0.3% and 0.2% coding gains is achieved for view 1 and view 2 and 0.1% coding gain is achieved for the coded and synthesized views while the number of required comparisons for the pruning process in the inter is reduced from 3 to 2 and the number of required comparisons for the pruning process in the merge and skip is also reduced from 5 to 4. Moreover, all comparisons can be performed in parallel.

Table 1 Summarized results (BD-rate savings) of the seven experiments

| Experiment Number | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 99.7% | 99.7% | 99.6% |
| 2 | -0.3% | -0.1% | -0.1% | -0.1% | -0.1% | 99.0% | 98.8% | 98.9% |
| 3 | -0.3% | -0.4% | -0.2% | -0.1% | -0.2% | 99.2% | 100.2% | 99.9% |
| 4 | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.9% | 100.4% | 99.8% |
| 5 | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 99.9% | 99.6% | 99.5% |
| 6 | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 99.1% | 99.2% | 100.0% |
| 7 | -0.3% | -0.4% | -0.2% | -0.1% | -0.2% | 99.8% | 99.4% | 99.9% |

Table 2 Results of Test1

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.3% | -0.3% | -0.2% | -0.1% | -0.2% | 99.6% | 100.2% | 100.4% |
| Kendo | -0.3% | -0.3% | -0.2% | -0.1% | -0.2% | 97.3% | 100.4% | 98.0% |
| Newspapercc | -0.3% | -0.4% | -0.2% | 0.0% | -0.1% | 100.1% | 99.8% | 97.3% |
| GhostTownFly | -0.2% | 0.0% | -0.1% | 0.0% | 0.0% | 100.0% | 96.0% | 100.3% |
| PoznanHall2 | 0.1% | -0.7% | -0.2% | -0.2% | -0.2% | 99.9% | 99.9% | 99.2% |
| PoznanStreet | -0.4% | 0.0% | -0.1% | -0.1% | -0.1% | 101.1% | 101.5% | 103.1% |
| UndoDancer | -0.4% | -0.4% | -0.1% | -0.1% | -0.1% | 99.9% | 100.2% | 99.0% |
| 1024x768 | -0.3% | -0.3% | -0.2% | -0.1% | -0.1% | 99.0% | 100.1% | 98.5% |
| 1920x1088 | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | 100.2% | 99.4% | 100.4% |
| average | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 99.7% | 99.7% | 99.6% |

Table 3 Results of Test2

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | 99.5% | 98.6% | 92.5% |
| Kendo | -0.3% | -0.1% | -0.1% | -0.1% | -0.1% | 99.3% | 100.2% | 95.8% |
| Newspapercc | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 99.9% | 98.3% | 100.9% |
| GhostTownFly | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 94.3% | 93.9% | 101.9% |
| PoznanHall2 | -0.3% | 0.2% | 0.0% | -0.2% | -0.1% | 100.0% | 99.6% | 101.7% |
| PoznanStreet | -0.5% | -0.3% | -0.1% | -0.1% | -0.1% | 100.1% | 99.7% | 102.4% |
| UndoDancer | -0.4% | -0.4% | -0.1% | -0.2% | -0.2% | 100.1% | 102.0% | 97.7% |
| 1024x768 | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 99.6% | 99.0% | 96.3% |
| 1920x1088 | -0.3% | -0.1% | -0.1% | -0.1% | -0.1% | 98.6% | 98.7% | 100.9% |
| average | -0.3% | -0.1% | -0.1% | -0.1% | -0.1% | 99.0% | 98.8% | 98.9% |

Table 4 Results of Test3

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.4% | -0.5% | -0.2% | -0.2% | -0.2% | 99.3% | 100.1% | 97.9% |
| Kendo | -0.5% | -0.4% | -0.3% | -0.2% | -0.2% | 100.0% | 100.0% | 99.1% |
| Newspapercc | -0.4% | -0.4% | -0.2% | -0.1% | -0.1% | 100.2% | 100.1% | 99.3% |
| GhostTownFly | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 100.0% | 100.2% | 98.5% |
| PoznanHall2 | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | 100.5% | 101.0% | 101.4% |
| PoznanStreet | -0.5% | -0.4% | -0.2% | -0.2% | -0.2% | 100.0% | 100.2% | 103.6% |
| UndoDancer | -0.5% | -0.5% | -0.2% | -0.2% | -0.2% | 94.7% | 99.9% | 99.4% |
| 1024x768 | -0.4% | -0.5% | -0.2% | -0.2% | -0.2% | 99.8% | 100.1% | 98.8% |
| 1920x1088 | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 98.8% | 100.3% | 100.7% |
| average | -0.3% | -0.4% | -0.2% | -0.1% | -0.2% | 99.2% | 100.2% | 99.9% |

Table 5 Results of Test4

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 99.8% | 99.6% |
| Kendo | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.0% | 100.3% | 101.0% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.2% | 102.0% | 101.6% |
| GhostTownFly | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 100.0% | 100.5% | 102.3% |
| PoznanHall2 | 0.4% | 0.4% | 0.2% | 0.1% | 0.1% | 99.7% | 99.2% | 97.1% |
| PoznanStreet | -0.3% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 101.3% | 100.4% |
| UndoDancer | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.8% | 100.1% | 97.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 100.7% | 100.7% |
| 1920x1088 | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 100.1% | 100.3% | 99.2% |
| average | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.9% | 100.4% | 99.8% |

Table 6 Results of Test5

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.2% | -0.4% | -0.2% | -0.2% | -0.2% | 103.8% | 114.1% | 92.4% |
| Kendo | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 101.3% | 101.9% | 98.4% |
| Newspapercc | -0.4% | -0.3% | -0.2% | -0.1% | -0.1% | 95.7% | 95.6% | 101.6% |
| GhostTownFly | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 95.8% | 85.6% | 99.9% |
| PoznanHall2 | -0.1% | -0.5% | -0.2% | -0.2% | -0.2% | 100.4% | 101.8% | 105.8% |
| PoznanStreet | -0.6% | -0.2% | -0.2% | -0.1% | -0.2% | 107.5% | 116.2% | 99.4% |
| UndoDancer | -0.4% | -0.4% | -0.1% | -0.2% | -0.2% | 95.4% | 86.6% | 99.3% |
| 1024x768 | -0.3% | -0.3% | -0.2% | -0.1% | -0.1% | 100.2% | 103.6% | 97.4% |
| 1920x1088 | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 99.7% | 96.8% | 101.1% |
| average | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 99.9% | 99.6% | 99.5% |

Table 7 Results of Test6

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | 98.2% | 101.2% | 87.9% |
| Kendo | -0.2% | -0.1% | -0.1% | 0.0% | -0.1% | 102.7% | 101.4% | 96.4% |
| Newspapercc | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 97.2% | 99.0% | 108.0% |
| GhostTownFly | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 99.1% | 102.6% |
| PoznanHall2 | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% | 101.4% | 97.2% | 99.6% |
| PoznanStreet | -0.7% | -0.4% | -0.2% | -0.2% | -0.2% | 96.3% | 98.3% | 103.1% |
| UndoDancer | -0.3% | -0.3% | -0.1% | -0.2% | -0.1% | 98.6% | 98.1% | 104.0% |
| 1024x768 | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 99.3% | 100.5% | 97.1% |
| 1920x1088 | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 99.0% | 98.2% | 102.3% |
| average | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 99.1% | 99.2% | 100.0% |

Table 8 Results of Test7

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.3% | -0.5% | -0.2% | -0.2% | -0.2% | 100.8% | 100.5% | 103.3% |
| Kendo | -0.5% | -0.4% | -0.2% | -0.2% | -0.2% | 100.4% | 101.0% | 102.2% |
| Newspapercc | -0.4% | -0.5% | -0.2% | -0.1% | -0.1% | 100.4% | 100.7% | 102.7% |
| GhostTownFly | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% | 98.4% | 101.4% | 97.7% |
| PoznanHall2 | 0.1% | -0.5% | -0.2% | -0.1% | -0.1% | 101.7% | 101.7% | 100.2% |
| PoznanStreet | -0.4% | -0.6% | -0.2% | -0.2% | -0.2% | 100.0% | 99.8% | 99.9% |
| UndoDancer | -0.4% | -0.5% | -0.2% | -0.2% | -0.2% | 96.8% | 91.3% | 93.7% |
| 1024x768 | -0.4% | -0.5% | -0.2% | -0.1% | -0.2% | 100.5% | 100.7% | 102.7% |
| 1920x1088 | -0.2% | -0.4% | -0.2% | -0.1% | -0.1% | 99.2% | 98.4% | 97.9% |
| average | -0.3% | -0.4% | -0.2% | -0.1% | -0.2% | 99.8% | 99.4% | 99.9% |

# Conclusion

This contribution has presented the results of CE5.h experiments related to the improvement of the pruning process. In Test6, the number of required comparisons in the merge and skip modes is reduced from 5 to 4 and the number of required comparisons in the inter mode is also reduced from 3 to 2, and all comparisons can be performed in parallel. This could help to reduce the cycles of the AMVP and merging candidate lists reconstructions. The results also show 0.3% and 0.2% BD-rate saving for view 1 and view 2, respectively, and 0.1% BD-rate saving for the coded and synthesized views.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] HTM-4.0.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-4.0.1/

[2] Y. Chang, S. Yea, “Description of Core Experiment 5 (CE5) on Motion/Mode Parameter Prediction”, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-A1105, July 2012, Stockholm.

[3] Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, and Shawmin Lei, “3D-CE5.h related: Pruning process for inter-view candidate”, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-A0048, July 2012, Stockholm.

[4] Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments”, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-A1100, July 2012, Stockholm.