| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, SE, 13–19 Oct 2012 | Document: JCT3V-B0093 |
| --- | --- |

| Title: | Removal of the parsing dependency of inter-view residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Kai Zhang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, kai.zhang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, two methods are proposed to improve the inter-view residual prediction in HTM. Firstly, an approach to solve the parsing problem related to the inter-view residual prediction is proposed. The experimental results reportedly show 5% encoding time saving and 0.1-0.2% BD-rate reduction for video1 and video2, while the parsing dependency of the inter-view residual prediction is removed. Secondly, we further remove the interpolation in inter-view residual prediction to reduce the complexity, and the disparity vector is directly rounding to a nearest integer pixel. When the two proposed methods are combined, the experimental results reportedly showed no BD-rate loss.

# Introduction

In the inter-view residual prediction (IVRP) in HTM [1], a disparity vector is firstly derived for a current prediction unit (PU). As illustrated in Figure 1, the residual block in the reference view referenced by the disparity vector is used for predicting the residual of the current PU. If the disparity vector points to a sub-sample position, the residual prediction signal is obtained by interpolating the residual samples of the reference view using a bi-linear filter.

Figure 1.  Basic concept for inter-view residual prediction

To control the usage of the IVRP, an IVRP enable flag is conditionally signaled in the coding unit (CU) level when all the following conditions are true:

- The current CU is a texture CU in dependent view, and the current slice is not an I-slice.

- The current CU has at least one PU using motion-compensated prediction [2].

- One or more transform units (TUs) covered or partially covered by the reference block in the reference view are non-intra coded and contain at least one nonzero coded block flag (CBF). [3]

In order to decide whether the IVRP enable flag should be decoded or not in the current CU, the decoder has to access the prediction type (motion-compensated prediction or inter-view compensated prediction) for every PU in the current CU and also needs to access the CBF flag of every TU and the coding type (intra or non-intra) of every CU covered or partially covered by the reference block. This causes the parsing throughput and parsing dependency issue.

For a CU where the inter-view residual flag is true, each PU in the CU has to determine whether to enable the IVRP or not according to the prediction type as follows [2]:

- If a PU only uses motion-compensated prediction, the IVRP is enabled in the PU.

- If a PU only uses disparity-compensated prediction, the IVRP is disabled in the PU.

- If the current PU is bi-prediction and one direction is motion compensated prediction and the other direction is disparity-compensated prediction, the IVRP is enabled, but the reference residual used in IVRP is multiplied by 1/2.

# 2. Problems of the current IVRP

As presented above, there are some drawbacks of the IVRP in current HTM 4.0.1 [4] as shown in the followings.

- The parsing of IVRP enable flag depends on the information (coding type, and CBF) of the reference view. If these pieces of information in the reference view are not decoded correctly, the entire picture of the current view cannot be parsed correctly, which is clearly undesirable.

- The parsing of IVRP enable flag requires checking the prediction type (motion-compensated prediction or disparity-compensated prediction) of every PU in the current CU, which will impact the parsing throughput and is undesirable for parsing.

- When the disparity vector points to a sub-sample location, the residual prediction signal is obtained by interpolating the residual samples of the reference view using a bi-linear filter. This will increase the complexity at both encoders and decoders.

These problems are addressed in this proposal. The first two parsing issues are solved by the proposed method in Section 3.1, and the third issue is solved by the proposed method in Section 3.2. The experimental results are shown in Section 4.

# Proposed methods

## Removal of parsing dependency of inter-view residual prediction

The IVRP is proposed to exploit the correlation of the residual signal between the reference view and the current view. When a PU is coded using the skip or merge mode with the inter-view merging candidate in temporal direction (i.e., the inter-view candidate is derived from the motion vectors in the reference block not from the disparity vector), the residual block of the current PU can be expected to have a high correlation with the residual of the reference block.

To avoid the parsing dependency and parsing throughput issues, we remove the on/off control flag for the IVRP. We also restrict the IVRP to the CU which is coded by the skip and merge modes with 2Nx2N partition for simplification. Instead of using rate-distortion optimization on the encoder side to decide the IVRP on/off flag and conditionally transmitting that flag to the decoder, in our approach the IVRP is applied when a CU satisfies the following condition:

- The current CU is a texture CU in dependent view, and the current slice is not an I-slice.

- The current CU is coded by skip or merge mode with 2Nx2N partition.

- The inter-view merging candidate in the temporal direction is selected (i.e., merging candidate index equal to 0 and it is motion vector candidate).

In HTM, the first merging candidate (index=0) is always the inter-view merging candidate. The inter-view merging candidate is derived from the motion vector in the reference block (motion vector candidate). Only when the motion vector of the reference block is unavailable (for example, reference block is intra block), the inter-view merging candidate is then derived as the disparity vector [5]. In this approach, the IVRP is always applied to skip or merge mode with 2Nx2N partition when the merging candidate index is equal to 0 and it is derived from a motion vector, not a disparity vector. With this approach, the parsing problem of the inter-view residual prediction could be totally removed.

## Removal of residual interpolation

To further simplify the IVRP, the residual interpolation in IVRP is also proposed to be removed. If the disparity vector of the current PU points to a sub-sample location, the residual prediction signal is obtained by rounding the sub-sample location to the nearest integer location of the reference view. This approach removes the bi-linear interpolation at both encoders and decoders to reduce complexity.

# Experimental results

The proposed methods are integrated on HTM-4.0.1. Three tests are conducted under common test condition [6]. Test1 is the proposed method to remove parsing dependency of inter-view residual prediction as presented in Section 3.1, and test2 is the proposed method to remove residual interpolation as presented in Section 3.2. The combination is also tested as test3. The results of test1, test2, and test3 are listed in Table 1, Table 2 and Table 3, respectively.

Table 1.  Results of removing parsing dependency of inter-view residual prediction (test1)

Table 2.  Results of removing residual interpolation (test2)

Table 3.  Results of the combination of test1 and test2 as test3

# Conclusion

Approaches to solve the parsing problem and to reduce the complexity in IVRP are proposed. The conditional parsing flag for the IVRP is removed by applying IVRP to a CU only when the CU is coded by skip or merge mode with 2Nx2N partition and the inter-view merging candidate in temporal direction is selected. The experimental results show this approach could also achieve an average 0.1% and 0.2% BD-rate reduction for video1 and video2, respectively, with 5% encoding time saving. In the second proposed method, the residual interpolation in IVRP is removed to reduce the complexity, and the disparity vector is directly rounded to the nearest integer position. The experimental results show an average 0.1% BD-rate loss for video coding while the bi-linear interpolation filter is removed.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- H. Schwarz, C. Bartnik, S. Bosse, H. Brust, T. Hinz, H. Lakshman, D. Marpe, P. Merkle, K. Muller, H. Rhee, G. Tech, M. Winken, T. Wiegand, “Description of 3D video technology proposal by Fraunhofer HHI (HEVC compatible; configuration B),” Document of ISO/IEC JTC 1/SC 29/WG 11, m22571, November, 2011.

- H. Liu, J. Jia, J. Heo, “Restricted inter-view residual prediction,” Document of ISO/IEC JTC 1/SC 29/WG 11, m24766, April, 2012.

- X. Li, Y. Chen, M. Karczewicz, “3DV-HEVC: Simplified inter-view residual prediction,” Document of ISO/IEC JTC 1/SC 29/WG 11, m24938, April, 2012.

- HTM-4.0.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-4.0.1/

- J. An, Y.-Wen Chen, J.-L. Lin, Y.-W. Huang, S. Lei, “3D-CE5.h related: Inter-view motion prediction for HEVC-based 3D video coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-A0049, July, 2012.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-A1100, July, 2012.