| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0094 |
| --- | --- |

| Title: | 3D-CE1.a: Inter-view Skip/Direct mode with sub-partition scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chi-Ling Wu, Yu-Lin Chang, Yu-Pao Tsai, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chiling.wu, yulin.chang, yupao.tsai, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, an inter-view Skip/Direct mode is reported. In AVC-based 3D video coding, View Synthesis Prediction (VSP) is a tool to use the reference from the synthetic frame to improve coding efficiency. A B-VSP [4] is proposed to use backward mapping VSP to build the synthetic frame. However, either VSP or B-VSP is costly in terms of number of computations and amount of required storage to perform image warping. The proposed inter-view Skip/Direct mode uses existing motion compensation module to generate the references from another view. This reduces the computational complexity, memory bandwidth, and buffer but still keeps similar coding gain comparing to B-VSP. Even when replacing the VSP scheme and the VSP skip mode with the proposed inter-view Skip/Direct mode in 3D-ATM v5.0, the experimental results reportedly show an average of 8% decoding time saving with 0.05% BD-rate reduction for synthesized pictures over the total bit-rate. Moreover, the proposed method can be applied to other modes to support VSP scheme without offline VSP reference frame generation in the current implementation.

# Introduction

In 3D video coding, View Synthesis Prediction (VSP) provides an inter-view reference from the synthetic frames. A forward mapping VSP was original proposed to provide a synthetic reference as the following method: provided that the texture and depth pair of a first view is coded and decoded, a second view can be predicted by warping the first view to the second view position [1]. Also, a VSP skip/direct mode [2] and a context-based adaptive skip flag positioning method [3] were proposed to use a skip_type flag to adaptively select a synthetic reference or a non-synthetic reference according to the skip status of neighbouring blocks. In 3D-ATM version 5.0, B-VSP [4] is implemented to replace the original forward mapping VSP. A backward mapping view synthesis scheme is used by B-VSP: provided that the texture of a first view and the depth of a second view are coded and decoded, the texture of the second view can be predicted by warping the texture of the first view to the second view position through the converted disparity vector from the depth of the second view.

However, in these implementations, VSP is still quite expensive due to the warping process of every pixel to generate a synthetic picture. In the MPEG 100th meeting, some contributions were proposed to reduce the complexity, such as [4], [6], and [7], which were integrated in 3D-ATM v4.0. In the MPEG 101st meeting, Generalized View Synthesis Prediction (GVSP) mode [10] and B-VSP [4] are proposed. GVSP provided a signalling scheme of applying the VSP pictures to the modes with the partitions greater than or equal to 8x8, which is adding a flag at the MB partition level to indicate whether a MB partition was predicted from a view synthesized picture. The VSP picture was not put in the reference list and the motion vector to the other reference pictures are not used to predict the motion to a VSP picture. Although the results of GVSP achieved 0.37% total (coded+synthesized) BD-rate saving, the encoding time increased around 11% as a penalty. On the other hand, B-VSP was proposed to reduce the complexity of VSP and memory access. The software was integrated in 3D-ATM v5.0 for CE investigation. The upsampling of the depth was disabled in the B-VSP proposal, and the process of B-VSP with backward warping was conceptually equivalent to collecting 2x2 interview corresponding blocks, which is obtaining the corresponding references from another view with the block size 2x2. The same as the goal of reducing the complexity and memory requirements for VSP, the proposed scheme achieves further decoding time saving and some total coding gain in 3D-ATM v5.0. It applies the concept of B-VSP with backward warping and further improves the backward warping process as utilizing the original motion compensation module in AVC codec to online generate the reference.

The rest of this contribution is organized as follows. The proposed method is described in Section 2, and the simulation results are shown in Section 3. Finally, Section 4 gives the conclusions.

# Algorithm description

In this contribution, an inter-view Skip/Direct mode without full frame view synthesis is proposed. Macroblocks with the inter-view Skip/Direct mode provides a good inter-view reference without warping the base view to the dependent views. In the inter-view Skip/Direct mode, the macroblock is partitioned into blocks, and their corresponding blocks are obtained in the other view as the reference with warping the other view to the current view block by block.

Figure 1 shows the details of the inter-view Skip/Direct mode. Note that View 0 is coded or decoded before View 1 and the texture (Texture 1) is coded after the depth (Depth 1) in View 1. When coding or decoding a macroblock in Texture 1, the macroblock is first partitioned into 16 4x4 blocks. For each 4x4 block, the corresponding block in Texture 0 is obtained from the disparity converted from the depth value. In 3D-ATM v5.0, the depth value of the block is the maximum depth value among the samples in the collocated block in Depth 1. The inter-view reference of the current block in Texture 1 is obtained by collecting 16 4x4 corresponding blocks in Texture 0 without warping Texture 0 to View 1. One flag is needed to indicate the inter-view Skip/Direct mode, which is the same as the VSP Skip/Direct mode in [2]. In the contribution the block size 4x4 is chosen because it is the smallest unit for motion compensation in AVC-based coding and the original motion compensation module can be utilized.

Figure 1 Proposed inter-view Skip and Direct mode

# Test results

The proposed method is implemented on the test model for AVC-based 3D video coding (3D-ATM v5.0), and all the simulations are done following the common test conditions [8]. Since the proposed scheme does not affect HP condidtions, the tests are only carried out for EHP conditions. The tables below show BD-rate results of the proposed scheme, using 3D-ATM v5.0 CTC as the anchor. In the tests, the generation of VSP synthetic frame of B-VSP is disabled by code definition (setting EnableVSP as zero) and the VSP Skip/Direct mode is modified to the proposed inter-view Skip/Direct mode. Since VSP synthetic frame is not available in our tests, there is no VSP reference frame in the reference list and the motion information can be derived without coded in the bitstream.

Table 1 shows the summary of the experimental results. Compared with 3DV-ATM v5.0, the proposed method reduces about 8% decoding time with small texture and total (synthesized+coded) gain, which are 0.1% gain and 0.05% gain in BD rates, respectively. Note that the decoding time saving mainly comes from disabling the generation of VSP synthetic frame. The experiment results show that it achieves more coding gain and further reduce the decoder time than the current B-VSP scheme.

Table 1 Test results of the proposed scheme without generation of VSP synthetic frame

|  | Texture Coding |  | Depth Coding |  | Total (Coded + Synthesed) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.05 | 0.00 | 0.00 | 0.00 | -0.02 | 0.00 | 99.34 | 88.03 | 98.74 |
| S02 | 0.00 | 0.00 | 0.00 | 0.00 | 0.03 | 0.00 | 99.81 | 96.91 | 100.37 |
| S03 | -0.24 | 0.01 | 0.00 | 0.00 | -0.12 | 0.00 | 97.75 | 89.31 | 98.38 |
| S04 | -0.28 | 0.01 | 0.00 | 0.00 | -0.22 | 0.01 | 99.62 | 98.64 | 99.96 |
| S05 | -0.10 | 0.00 | 0.00 | 0.00 | -0.08 | 0.00 | 99.27 | 95.06 | 97.04 |
| S06 | 0.04 | 0.00 | 0.00 | 0.00 | 0.08 | 0.00 | 99.17 | 86.52 | 98.73 |
| S08 | -0.05 | 0.00 | 0.00 | 0.00 | 0.02 | 0.00 | 98.64 | 90.50 | 97.37 |
| Average | -0.10 | 0.00 | 0.00 | 0.00 | -0.05 | 0.00 | 99.09 | 92.14 | 98.65 |

# Conclusion

An inter-view Skip/Direct mode is proposed to avoid the high complexity and huge memory bandwidth caused by full frame view synthesis. Besides, it generates the reference online with the existing motion compensation module in AVC. The inter-view Skip/Direct mode is tested on 3D-ATM v5.0 and applied on the texture coding without generation of VSP synthetic frame. When disabling the generation of VSP synthetic frame and modifying the VSP Skip/Direct mode to the proposed inter-view Skip/Direct mode, the results show that it achieves an average of 8% decoding time saving with small BD-rate gain 0.10% for texture coding and 0.05% for synthesized pictures over the total bit-rate. Moreover, the proposed method can be applied to other modes to support VSP scheme without offline VSP reference frame generation in the current implementation.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Tian, D. Graziosi, Y. Wang, N. Cheung, A. Vetro, “Mitsubishi Response to MPEG Call for Proposal on 3D Video Coding Technology”, ISO/IEC JTC1/SC29/WG11 document M22663, Geneva, Switzerland, November 2011.

- ISO/IEC JTC1/SC29/WG11 “3D-AVC-CE1 results on VSP skip/direct mode by Mitsubishi,” M23915, Feb. 2012.

- ISO/IEC JTC1/SC29/WG11 “3D-CE1.a results on a context-based adaptive skip flag positioning method by Samsung,” M24819, May 2012.

- Wenyi Su et al, “3DV-CE1.a: Block-based View Synthesis Prediction for 3DV-ATM, ”, JCT3V-A0107, Stockholm, Sweden, July 2012

- ISO/IEC JTC1/SC29/WG11 “3D-CE1.a related: Simplification of in-loop view synthesis with fractional-pel disparity,” M24796, May 2012.

- ISO/IEC JTC1/SC29/WG11 “3D-CE1.a related: 3DV-CE1.a-related: Low complexity block-based View Synthesis Prediction,” M24915, May 2012.

- ISO/IEC JTC1/SC29/WG11 “3D-CE1.a related: View synthesis based on asymmetric texture and depth resolutions,” M24941, May 2012.

- Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, ”Common Test Conditions of 3DV Core Experiments”, ISO/IEC JTC1/SC29/WG11 document m26349, Stockholm, Sweden, July 2012.

- Heiko Schwarz, Dmytro Rusanovskyy, ” Common Test Conditions for AVC and HEVC-based 3DV”, ISO/IEC JTC1/SC29/WG11 document N12560, San Jose, USA, February 2012.

- Xin Zhao et al, “3D-CE1.a: Generalized view synthesis prediction (GVSP) mode, ”, JCT3V-A0103, Stockholm, Sweden, July 2012