| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0095 |
| --- | --- |

| Title: | CE5.h related: Adaptive inter-view MVP candidate position for merging candidate list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin, Jianhua Zheng, Xiaozhen Zheng No. 156, Rd. Beiqing Haidian District, Beijing 100095 China | Tel: Email: | +8610-60612372 yblin@huawei.com |
| Source: | Hisilicon Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes to construct MERGE candidate list by adaptively inserting an inter-view MVP candidate. In 3D-HTM, an inter-view MVP candidate for dependent view is obtained based on its reference view. It is observed that the derived inter-view MVP may be either temporal motion vector (MV) or inter-view disparity vector (DV). It is proposed that the inter-view MVP candidate’s position in the MERGE candidate list is not fixed, i.e., the first position is taken when the inter-view MVP is a kind of MV while a different position close to the end of the list is assigned when the inter-view MVP is a kind of DV. It is reportedly shown that the proposed method archives average -0.23%, -0.21% BD-rate reduction for texture view1 and view2, respectively, with comparison to HTM4.0.1.

# Introduction

In 3D-HTM, an inter-view MVP candidate (IVMVP) for dependent views is obtained based on its reference view, then the IVMVP is inserted into the first position of HEVC-like MERGE candidate list.

It is observed that the IVMVP may be either temporal motion vector (MV) or inter-view disparity vector (DV), and the two kinds of candidates have different performance improvement to inter-view MV prediction. It is proposed to adaptively insert the IVMPV into the MERGE candidate list. That is to say, different position of IVMVP candidate in the MERGE candidate list is assigned according to whether the IVMVP is MV or DV. Specifically, the first position in the candidate list is taken when the IVMVP is a kind of MV while a different position in the list is assigned when the IVMVP is a kind of DV.

# Proposed algorithm

Fig. 1 MGER candidate list construction

The existing MERGE candidate list in 3D-HTM is as follows.

IVMVP -> A1 -> B1 -> B0 -> A1 -> B2 -> TMVP

Here, A0, A1, B0, B1 and B2 are spatial neighbour reference blocks of current PU, TMVP is temporal MVP candidate for the current block, and IVMVP is inter-view MVP candidate, as shown in figure 1. The IVMVP takes the first position in 3D-HTM.

In the proposed method, he following MERGE list is shown as an example.

IVMVP -> A1 -> B1 -> B0 -> A1 -> B2 -> TMVP   (in case IVMVP is a kind of MV)

A1 -> B1 -> B0 -> A1 -> IVMVP -> B2 -> TMVP   (in case IVMVP is a kind of DV)

In case IVMVP is a kind of MV, the IVMVP takes the 1st position in the list, as same as the existing one.

In case IVMVP is a kind of DV, the IVMVP takes the 5th position in the list. It is noted that other position is also possible.

It might be beneficial that different position in the MERGE candidate list is allowed for the IVMVP candidate

# Experimental results

The proposed method has been integrated in HTM4.0.1. The proposed method with the inserted 5th position in MERGE candidate list is tested. Table 1 shows BD-rate reduction of -0.23% for texture view1 and -0.21% for texture view2. The encoding and decoding running time is almost not changed.

Table 1 test results of the proposed method (the 5th position in the list)

# Conclusions

The proposed method enables adaptive position of inter-view MVP candidate in the MERGE candidate lists for purpose of performance improvement. It is reportedly shown that the proposed method archives average -0.23% and -0.21% bit rate reduction for texture view1 and view2 with comparison to HTM4.0.1. It is recommended to inclusion of the proposed method into 3D-HTM.

# Patent rights declaration(s)

Hisilicon Technologies, Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).