| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0096 |
| --- | --- |

| Title: | CE5.h related: removal of dependency between multiple PUs in a CU for disparity vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin, Jianhua Zheng, Xiaozhen Zheng No. 156, Rd. Beiqing Haidian District, Beijing 100095 China | Tel: Email: | +8610-60612372 yblin@huawei.com |
| Source: | Hisilicon Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes to remove the dependency among multiple PUs in a CU for purpose of enabling PU-based parallel disparity derivation. In 3D-HTM implementation, it is not possible for two PUs within a CU, e.g. with Nx2N partition, to perform disparity vector derivation process in a parallel way. This is because one or more of the neighbour blocks involved in disparity vector derivation for 2nd PU is located in the 1st PU. The dependency between the two PUs disables parallelism of PU-based disparity vector derivation. Such dependency is removed in this contribution. It is reportedly shown the coding efficiency is -0.1% and 0.0% BD-rate saving for texture view1 and view2, respectively, with comparison to HTM4.0.1.

# Introduction

In the existing 3D-HTM, a disparity vector (DV) for current PU is firstly derived based on its neighbour reference blocks, then the derived DV is utilized in process of inter-view MV prediction for dependent view.

In the PU-based DV derivation process, there exists dependency between multiple PUs within a CU, e.g., a CU with Nx2N partition type includes left and right PUs. The dependency makes it not possible for two PUs to perform DV derivation process in a parallel way. This is because one or more of the neighbour reference blocks involved in the DV derivation for 2nd PU (i.e., right PU) may be located in the 1st PU (i.e., left PU).

Fig. 1  Neighbour blocks involved in DV derivation for 2nd PU of a CU with Nx2N partition type

For example, as shown in Figure 1, the current CU with Nx2N partition includes two PUs, and the 5 neighbour blocks involved in the DV derivation for 2nd PU is also shown. It is noticed that the neighbour block A1 is located in the 1st PU, which means the DV derivation for 2nd PU can’t be processed until the encoding of the 1st PU is finished. That is to say, the PU-based DV derivations for both 1st and 2nd PUs are not able to be carried out in parallel.

This contribution proposes to remove the dependency for purpose of enabling parallelism of PU-based DV derivation in case of a CU including 2 PUs.

# Proposed algorithm

In the existing DV derivation in 3D-HTM, a DV is derived by checking neighbour blocks according to the following order.

A1 -> B1 -> B0 -> A0 -> B2 -> TDVP

Here, A0, A1, B0, B1 and B2 are spatial neighbour blocks, TDVP is temporal DV candidate for the current PU, also shown in figure 1.

It is proposed that the DV derivation for the 2nd PU skip to check the neighbour block A1 which is located in the 1st PU when current CU partition type is Nx2N or nLx2N or nRx2N, and skip to check the neighbour block B1 when current CU partition type is 2NxN or 2NxnU or 2NxnD.

# Experimental results

The proposed method has been integrated in HTM4.0.1. The test results are shown in Table1. It shows average coding efficiency of -0.1% and 0.0% BD-rate saving for texture view1 and view2, respectively. The encoding/decoding running time is almost not changed.

Table 1 test results of the proposed DV derivation

# Conclusions

The proposed method enables parallel DV derivation for a CU with 2 PUs by removing dependency among the PUs. More importantly, the process of merging candidate list construction for multiple PUs in a CU can be carried out in a parallel way. It is reportedly shown average -0.1%, 0.0% BD-rate saving for texture view1 and view2 with comparison to HTM4.0.1. It is recommended to inclusion of the proposed method into 3D-HTM.

# Patent rights declaration(s)

Hisilicon Technologies, Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text change

- G.8.5.2.1.13	Derivation process for a disparity vector

Inputs to this process are:

–	a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

–	variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit

–	a view identifier refViewIdx specifying a reference view.

Outputs of this process are:

–	a flag availableDV specifying whether the disparity vector is available,

–	a disparity vector mvDisp.

For N being A0, A1, B0, B1, or B2, and X being 0 or 1, flags availableFlagIvpMvNX and the components of disparity vectors ivpMvDispNX are set equal to 0. availableDV is set equal to 0.

The motion vector mvDisp and the availability flag availableDV are derived in the following ordered steps and the whole decoding process of this sub-clause terminates once availableDV is set to 1.

- For each N being A1, B1, A0, B0, or B2, and ( xN, yN ) being ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW − 1,  yP − 1 ), ( xP + nPSW,  yP − 1 ), ( xP − 1,  yP + nPSH  ), or ( xP − 1,  yP − 1 ), the following applies.

- When yP−1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), the following applies.

xB0 = ((xB0>>3)<<3) + ((xB0>>3)&1)*7		(G-1)
xB1 = ((xB1>>3)<<3) + ((xB1>>3)&1)*7		(G-2)
xB2 = ((xB2>>3)<<3) + ((xB2>>3)&1)*7		(G-3)

- If N is equal to A1, PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and partIdx is equal to 1, availableA1 is set equal to 0.

- If N is equal to B1, PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and partIdx is equal to 1, availableB1 is set equal to 0.

- For each X from 0 to 1, the following applies, if availableN is equal to 1 and PredFlagLX[ xN ][ yN ] is equal to 1

[Ed.(GT): Derivation of availableN is missing.]

  - If RefPicListX[ RefIdxLX[ xN ][ yN ] ] is an inter-view reference picture with ViewOrderIndex equal to refViewIdx, the following apply:

mvDisp = mvLXN[ xN ][ yN ]		(G-4)
availableDV = 1		(G-5)

  - Otherwise, if PredMode[ xN ][ yN ] is equal to MODE_SKIP and IvpMvFlagLX[ xN, yN ] is equal to 1, the following apply:

ivpMvDispNX = IvpMvDispLX[ xN, yN ]	(G-6)
availableFlagIvpMvNX = 1		(G-7)

- The derivation process for a disparity vector from temporal neighbour block as specified in Error: Reference source not found is invoked with a luma location ( xP, yP ), variables nPSW and nPSH as inputs, and flag availableDV, disparity vector mvDisp, availableFlagIvpMvColX, and ivpMvDispColX as outputs

- For each N being A1, B1, A0, B0, B2, and Col, the following applies.

- For each X from 0 to 1, the following applies.

  - If availableFlagIvpMvNX is equal to 1, the following apply:

mvDisp = ivpMvDispNX		(G-8)
availableDV = 1		(G-9)