| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0097 |
| --- | --- |

| Title: | CE 5.h related: Cross-check of JCT3V-B0095 on Adaptive inter-view MVP candidate position for merging candidate list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document JCT3V |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Thomas Guionnet (INRIA) Laurent Guillo (IRISA/CNRS) INRIA/IRISA Campus de Beaulieu 35042 Rennes Cedex France | Tel: Email: | +33 299847100 thomas.guionnet@inria.fr laurent.guillo@inria.fr |
| Source: | INRIA |  |  |

_____________________________

# Abstract

This document reports the results of the cross-check of a new construction of the MERGE candidate list proposed by Hisilicon Technologies and Huawei Technologies and described in JCT3V-B0095.

The idea of the proposal is to move the interview motion vector predictor candidate from the first position to the fourth position in the list when it is a disparity vector.

This proposal is not part of the current CE5.h but is related to.

# Code verification

Hisilicon Technologies and Huawei Technologies provided INRIA with the source code to be tested.

Three macros defined in Typedef.h are used to implement the proposal:

- #define ADAPTIVE_POS_MERGE		1  // different positions for pdmMERGE candidate

- #define pdmMERGE_POS_MV		0  //0

- #define pdmMERGE_POS_DV		4  //4

The first macro “ADAPTIVE_POS_MERGE” is set to 1 when the code specific to the proposal must be activated. When it is set to 0, the default behaviour is the one of HTM 4.0.1.

The two following macros specify the position in list of the interview motion vector predictor candidate depending on whether it is a temporal motion vector based candidate or an interview disparity vector based candidate.

Only the method “Void TComDataCU::getInterMergeCandidates” in TComDataCU.cpp is modified and only fourteen short lines of source code are added.

The source codes have been successfully compiled using Linux (Fedora Core 13 64 bits) with gcc 4.4.5.

Tests were run on a cluster grid with the following characteristics:

•	CPU: 2 x Intel(R) Xeon(R) E5640 @ 2.67GHz, 4 cores.

•	RAM: 12 Go per core.

No memory leaks were found.

# Results

The proposal was implemented in the HTM 4.0.1 and tests were run with all the JCT3C video corpus according to the common test conditions [1].

The table below gathers the obtained results:

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | -0,2% | -0,2% | -0,1% | -0,1% | -0,1% | 99,1% | 100,2% | 96,7% |
| Kendo | 0,0% | 0,0% | 0,3% | 0,0% | 0,0% | 0,0% | 99,5% | 100,1% | 100,2% |
| Newspapercc | 0,0% | -0,5% | -0,5% | -0,2% | -0,2% | -0,2% | 98,9% | 98,3% | 98,2% |
| GhostTownFly | 0,0% | 0,1% | -0,1% | 0,0% | 0,0% | 0,0% | 104,7% | 99,9% | 99,9% |
| PoznanHall2 | 0,0% | 0,0% | -0,2% | -0,1% | -0,1% | -0,1% | 101,7% | 99,3% | 96,3% |
| PoznanStreet | 0,0% | -0,9% | -0,9% | -0,3% | -0,3% | -0,3% | 98,8% | 99,8% | 100,0% |
| UndoDancer | 0,0% | -0,1% | 0,1% | 0,0% | 0,0% | 0,0% | 101,4% | 100,2% | 101,1% |
| 1024x768 | 0,0% | -0,2% | -0,1% | -0,1% | -0,1% | -0,1% | 99,2% | 99,6% | 98,3% |
| 1920x1088 | 0,0% | -0,2% | -0,3% | -0,1% | -0,1% | -0,1% | 101,6% | 99,8% | 99,3% |
| average | 0,0% | -0,2% | -0,2% | -0,1% | -0,1% | -0,1% | 100,6% | 99,7% | 98,9% |

Comments about the results:

- The obtained results perfectly match those provided by HiSilicon Technologies and Huawei Technologies

- The added complexity is negligible.

- There are gains on average for both side views (0.2% bit rate reduction) and the overall bit rate (0.1% bit rate reduction) however:

  - There is no gain for the overall bit rate of synthetic video such as “Ghost Town Fly” and “Undo Dancer”: it seems that modifying, this way, the order of the merge candidate list is not relevant when depth maps are accurate.

  - Gains are relevant with video with natural content such as “Poznan Street” (up to 0.9% bit rate reduction for both side views and 0.3% reduction for the overall bit rate).

Additional comments:

- The size of the candidate merge has not been modified and is still set to 6 candidates.

- To have a better understanding of this proposal, it would have been very interesting:

  - To know what the results are when the interview motion vector predictor candidate is moved elsewhere in the list and not only to the fourth position as it is implemented.

  - To have statistics about when the interview motion vector predictor candidate is moved.

- It will be interesting to compare this approach with possible similar solutions that will be presented during the meeting.

# Conclusion

The JCT3V-B0095 proposes a re-ordering of the merge candidate list, that can possibly move the interview motion vector predictor candidate to the fourth position of the list depending on whether it is a temporal motion vector based candidate or an interview disparity vector based candidate.

The bit rate gains for both side views are 0.2% and 0.1% for the overall bit rate with no added complexity.

There are no gains for videos with synthetic content but gains are relevant for videos with natural contents (up to 0.3% bit rate reduction for the overall bit rate).

It would be very interesting to know whether these gains might be added to those obtained with proposals that change the order of the merge candidate list or modify it.

# References

- Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, « Common test conditions for 3DV Core Experiments », JCT3V-A1100, July 2012, Stockholm, Sweden.

# Patent rights declaration(s)

INRIA does not have any current or pending patent rights relating to the technology described in this contribution.