| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0102 |
| --- | --- |

| Title: | CE1.h: Results on View Synthesis Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou, Dong Tian*, Anthony Vetro (MERL) Shinya Shimizu*, Shiori Sugimoto, Hideaki Kimata (NTT) | Tel: Email: | *+1-617-621-7556 tian@merl.com *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation and Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

View synthesis prediction (VSP) is a technique to remove interview redundancies among video signal from different viewpoints, in which synthetic pictures are first generated and then used as references to predict a current picture. A Core Experiment on VSP for HEVC based solution was set up at the 1st JCT-3V meeting at Stockholm. In this document, we made a more complete study on the proposed VSP techniques, including the basic VSP concept in HTM, VSP signalled using merging candidate list and zero/non-zero motion vector for VSP prediction. The overall bitrate savings on the coded and synthesized views are -0.9% for XGA sequences and -2.3% for HD sequences. The maximum bitrate savings on the dependent left and right views are -11.8% and -10.2%, respectively.

# Introduction

View synthesis prediction (VSP) for HEVC compatible 3D extension (HTM) has been initially studied in [1] and [2]. Although the implementations were not completed yet by then, the initial studies showed that VSP prediction has a good potential to improve the coding efficiency in the context of HTM.

In this document, the recent progress on the study of using VSP for HTM is reported, where improved performance is observed.

# View Synthesis Prediction

In this section, the proposed VSP techniques [1, 2] are described.

## Basic VSP concepts

Within a basic implementation of VSP, a synthetic picture is firstly generated using a decoded texture image and a decoded depth image. The rendering module in HTM is re-used with the simplification proposed in [2] to generate the synthetic picture in our current implementation. For a synthetic picture, POC and view_id values are set to equal to the picture being coded.

For the second step, the synthetic picture is proposed to be appended to the reference picture lists (LIST_0 and LIST_1). In principle, the synthetic picture can be inserted at the end of the reference picture lists or configured at any position in the lists. In such a way, the synthetic pictures could be selected to predict the current picture in the same manner as temporal and inter-view reference pictures.

In the current implementation, the synthetic picture is placed at the end of the reference picture list for both LIST_0 and LIST_1. For bi-predictive pictures, the same synthetic picture is added to the two lists to allow a flexible combination. More advanced placing synthetic pictures is subject to future work.

The above VSP prediction can to be applied to both texture and depth, which is configurable in our implementation.

It should be noted that the synthetic picture is treated as a different type of picture other than either temporal or inter-view reference picture. As a result, the motion/disparity vector scaling is skipped if they refer to a synthetic reference. Furthermore, any motion vectors associated with the synthetic pictures are ignored on the derivation process of disparity vector during inter-view motion prediction. The motion vectors associated with synthetic pictures are counted during the construction of merging candidate list and its further modifications.

In addition to sequence level signalling of VSP tool, slice level signalling is proposed on whether VSP is enabled using a one bit flag, use_vsp_flag. The semantics can be, e.g., value 1 indicates that VSP prediction is enabled and value 0 indicates that VSP prediction is disabled. With the slice level signalling, only when the VSP is turned ON for a slice, a synthetic picture is generated and inserted to the reference picture list. Otherwise, all VSP related syntax or process are to be skipped.

In the experiments, two set-ups are tested: 1) VSP is enabled for all pictures, and 2) VSP is enabled for reference pictures and disabled for non-reference pictures. More advanced VSP usage at picture level based on RD performance is subject to future work.

## VSP skip using the merging candidate list

VSP skip/direct [1] was proposed to enable the use of synthetic picture for skip/direct mode even when the synthetic picture is not placed at the beginning of the reference picture list. With VSP skip/direct mode, the co-located block from the synthetic picture is used for image prediction.

An adaptive signalling method was proposed in the context of ATM by re-defining the meaning of the skip flags [3] and introduced and evaluated within HTM in [2].

An alternative way to signal VSP skip using merging candidate list was also proposed in [2] at the 1st JCT-3V meeting, but its implementation was not finished then. In this document, we completed the related implementation.

During the construction of merging candidate list, an additional candidate corresponding to the synthetic reference picture is added. In the current implementation, the synthetic candidate is inserted after the spatial candidates if there is at least one of the spatial merging candidates either unavailable or duplicate with the other candidates. TABLE 1 shows the proposed order of the merging candidates in the list.

TABLE 1: The proposed merging candidate list

Inter-view (IVMP)

Left

Above

Above-right

Bottom-left

Above-left

VSP

Temporal

The construction process of merging candidate list is composed of four steps as described below.

- The inter-view candidate derived from inter-view motion prediction, if available, is added to the merging candidate list.

- Spatial merging candidates derived from neighboring prediction unit partitions, if available, are added to the merging candidate list. Similar to the spatial merging candidate handling in HEVC, the redundant merging candidates are eliminated from the list by inspecting the motion information of each candidate.

- Variable previousVSPFlag is set equal to 0 in the initialization.

- The following steps are to be performed to exclude certain candidate from the merging candidate list:

4.1) Left: If the neighbor from left refers to VSP reference with mv(0,0) and previousVSPFlag==1, the left candidate is excluded from the merging candidate list;

4.2) Above: If the neighbor from above has the same motion information as the neighbor from left, or it refers to VSP reference with mv(0,0) and previousVSPFlag==1, the above candidate is excluded from the merging candidate list;

4.3) Above-Right: If the neighbor from above-right has the same motion information as the neighbor from above, or it refers to VSP reference with mv(0,0) and previousVSPFlag==1, the above-right candidate is excluded from the merging candidate list;

4.4) Left-Bottom: If the neighbor from left-bottom has the same motion information as the neighbor from left, or it refers to VSP reference with mv(0,0) and previousVSPFlag==1, the left-bottom candidate is excluded from the merging candidate list;

4.5) Left-Above: If the neighbor from left-above has the same motion information as the neighbor from left, or it has the same motion information as the neighbor from above, or it refers to VSP reference with mv(0,0) and previousVSPFlag==1, the left-above candidate is excluded from the merging candidate list;

Note that similar to the current working draft of HTM, Left-Above is considered only if any of the five candidates from Inter-view, Left, Above, Above-Right and Left-Bottom is excluded from the merging list. However, if more than one candidates among the six candidates mentioned above are excluded from the merging candidate list, VSP candidate is inserted into the merging candidate list.

- For each available candidate, if it is VSP coded, previousVSPFlag is set equal to 1.

- The temporal merging candidate, if available, is added to the merging candidate list. (The same as current HTM design.)

- When the current slice is B, the number of valid candidates from above steps is larger than 1 and less than 6, the derivation process of combined bi-predictive merging candidates is invoked. (The same as current HTM design.)

- Zero motion vector merging candidates are added to the merging candidate list if there are less than 6 candidates derived from above steps. (The same as current HTM design.)

In Appendix A, the draft text corresponding to the proposed merging candidate list construction is provided.

## Motion vector and AMVP

In general, a block may be coded in three modes. A first is skip mode and a second is merge mode, where synthetic reference may be used if merging candidate index points to a synthetic reference. No motion information is to be transmitted for the first two modes. For a third mode, a block is predicted from a reference block using motion/disparity compensation. A motion vector difference (MVD) and a residual block would be coded.

When coding a block using normal prediction (not skip or merge), it is worth to study whether motion vectors are worthy to be transmitted if a synthetic picture is being referenced. The potential advantage is to save some bits for motion vectors, but it may degrade the efficiency of motion compensation due to the constraint on the motion vector selection. Another cost for no motion vector transmission is that the parsing process becomes a little more complex, because whether MVD syntaxes appear in the bitstream would depend on the reference picture type.

For each design, the fixes to the WD are described as follows.

Design 1: Non-zero motion vector may be transmitted even when a synthetic picture is being referenced. In this design, it is proposed that the MVP is always set to zero MV and AMVP list construction is kept untouched.

Design 2: No motion vector is to be transmitted if the reference picture is a synthetic picture. If the refrence picture being used is a non-synthetic picture, MVD and MVPIdx are coded. During the AMVP list construction process, if a neighboring block refers to a synthetic picture, the corresponding MVP is set unavailable. On the other hand, if the reference picture being used is a synthetic picture, neither MVD nor MVPIdx is coded. Zero motion vector is always assumed.

Evaluation results on both designs would be provided.

# View Synthesis Prediction

## CTC

The proposed methods are built on the top of the reference software 3DV-HTM v4.0.1.

The experiments are first conducted according to the common test condition [4] in TABLE 2.

At this moment, I’ve finished only one case for texture-only, but I think it will work with any combinations. If you encountered the problem, please inform us. We’ll try to fix them as quickly as possible. When the simulation results are available, we’ll update the results.

TABLE 2: Test cases summary

|  | VSP including VSP skip | VSP used | MV info for VSP | VSP for depth coding | VSO setting |
| --- | --- | --- | --- | --- | --- |
| Anchor | N/A | N/A | N/A | N/A | CTC |
| Case a | X | ref only | No motion info | X | CTC |
| Case b | X | ref and non-ref | No motion info | X | CTC |
| Case c | X | ref only | With motion info | X | CTC |
| case d | X | ref and non-ref | With motion info | X | CTC |
| case h | X | ref only | With motion info |  | CTC |

TABLE 3: Experimental Results

- Case a (Except for non-reference frames, no vector info., both texture and depth, CTC VSO)

- Case b (All frames, no vector info., both texture and depth, CTC VSO)

- Case c (Except for non-reference frames, coding vector info., both texture and depth, CTC VSO)

- Case d (All frames, coding vector info., both texture and depth, CTC VSO)

- Case h (Except for non-reference frames, coding vector info., texture only, CTC VSO)

Evaluation

The coding performance is evaluated for the 5 test cases and the results are shown in TABLE 3. The details are available in the attached EXCEL files. Our observations are summarized as below.

- When enable all VSP related tools in our implementation for all views (case b), -1.7% coding gain is found for the coded and synthesized views. The most gains are achieved for FHD sequences: -2.3% for the coded and synthesized views, -5.7% and -4.9% for the coded views.

- When disabling VSP for non-reference frames (case a), 0.1% loss is found on average relative to the full package although -0.3% gain is observed for XGA sequences. Besides, this provides the benefit of decoding time about 44%.

- When allowing transmission of vector information for VSP (case c and d), there is almost no change on the overall performance although FHD sequences tend to have small benefits on the transmission of motion vector information. Encoding and decoding times are slightly increased due to motion estimation and parsing motion information.

- When VSP is limited for texture coding (case h), 0.8% loss is found on the synthesized views relative to the case when VSP is allowed for both texture and depth coding (case d). Encoding and decoding time are reduced since the generation of virtual depth map is skipped.

These results show that the coding gains are promising for all sequences. When we have more advanced decision process on whether VSP is turned on or off, the gain and complexity could be more optimized.

Regarding the transmission of motion information for VSP, no motion vector is slightly better. However, it requires some changes on the low-level syntax and the decoding process from the original HEVC. Therefore, the benefit should be carefully evaluated.

Almost half of the overall gains for the synthesized views are provided by VSP for depth coding. In the current HTM design, inconsistent reference picture buffers between texture and depth coding may penalize the performance of the motion vector inheritance (MVI).

## non-CTC

In order to study the effects of VSO upon the VSP, we further performed more tests using different VSO configuration. The tested VSO configuration is set as below.

[cx0 B(cc1) I(s0.25 s0.5 s0.75)][cx1 B(oo0) B(oo2) I(s0.25 s0.5 s0.75 s1.25 s1.5 s1.75) E(o0 o2)][cx2 B(cc1) I(s1.25 s1.5 s1.75)].

In this configuration, additional optimization targets, original non-base views, are added when depth map of the base view is coded.

The coding performance is evaluated for 2 test cases, including no VSP (ted for texture coding (case eable, we'other cases, I'n e the perfomance h map is skipped. re inconsistent. ger-pel accuacy 00, as shown in TABLE 4. The results are shown in TABLE 5. Our observations are summarized as below.

- Test case i and j are designed as counter parts for anchor and case d, respectively by modifying the VSO configuration. Compared to the VSO setting in the CTC, there are large losses for the synthesized view.

The result for case j is not available yet. We’ll update the results during the meeting. We think that VSP can achieve more efficient coding for the coded views if VSP performance, i.e. synthesis quality on non-base view, is taken into account on depth map coding. And we also think the current VSO is highly optimized for the final synthesized views; it is difficult to conclude the proposed VSP performance.

TABLE 4: Test cases summary

|  | VSP including VSP skip | VSP used | MV info for VSP | VSP for depth coding | VSO setting |
| --- | --- | --- | --- | --- | --- |
| Case i (Anchor) | N/A | N/A | N/A | N/A | non-CTC |
| Case j | X | ref and non-ref | With motion info | X | non-CTC |

TABLE 5: Experimental Results

- Case j (no VSP, non-CTC VSO)

- Case j (All frames, coding vector info., both texture and depth, non-CTC VSO)

# Conclusions

In this document, the use of VSP prediction in the context of HTM is studied. With the best performing setting, the average bitrate saving of -1.6% for all sequences and coded & synthesized views is presented. Up to about -12% bitrate saving is also observed for the coded view. We therefore recommend 1) adopt to include VSP reference picture in the reference picture list; 2) adopt the VSP prediction using merging candidate list; 3) continue to study whether non-zero motion vector for VSP need be transmitted in the context of CE; 4) further study picture-level VSP usage based on RD optimization; 5) further study the impact of depth map coding tools on VSP (e.g. VSO tools).

# Patent rights declaration(s)

NTT Corporation and Mitsubishi Electric Research Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Danillo Graziosi, Dong Tian, and Anthony Vetro, "3D-CE1.h related proposal for view synthesis prediction", ISO/IEC JTC1/SC29/WG11 MPEG2012/M24933, April 2012, Geneva, Switzerland.

- Shinya Shimizu, Dong Tian, Shiori Sugimoto, Feng Zou, Hideaki Kimata, and Anthony Vetro, "3D-CE1.h Results on View Synthesis Prediction", JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT2-A0018, July 2012, Stockholm, Sweden.

- Jin Young Lee, Jaejoon Lee, and Du Sik Park, "3D-CE1.a results on a context-based adaptive skip flag positioning method by Samsung", ISO/IEC JTC1/SC29/WG11 MPEG2012/M24819, April 2012, Geneva, Switzerland

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT2-A1100, July 2012, Stockholm, Sweden.

# Appendix A: Modified merging candidate list construction process

Below is the modified merging candidate list constructio process with VSP candidate being inserted. The original text is copied from WD and the modifications are yellow highlighted. The neighboring blocks are defined as shown in Figure 1.

Figure 1. Neighboring block definition

Section 8.5.2.1.1. Derivation process for luma motion vectors for merge mode

- Depending on multi_view_mv_pred_flag, the following applies.

–	If multi_view_mv_pred_flag is equal to 0, the variable availableFlagInterView is set equal to 0.

–	Otherwise (multi_view_mv_pred_flag is equal to 1), the derivation process for the inter-view merge candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, and the reference view identifier refViewIdx set equal to 0 as the inputs and the outputs are the flag availableFlagInterView, the prediction utilization flags predFlagL0InterView and predFlagL1InterView, the reference indices refIdxL0InterView and refIdxL1InterView, and the motion vectors mvL0InterView and mvL1InterView.

- The merge candidate list, mergeCandList, is constructed as specified by the following ordered steps:.

  - The variable numMergeCand is set equal to 0.

  - When availableFlagInterView is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to InterView and the variable numMergeCand is increased by 1.

  - When availableFlagA1 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to A1 and the variable numMergeCand is increased by 1.

  - When availableFlagB1 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to B1 and the variable numMergeCand is increased by 1.

  - When availableFlagB0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to B0 and the variable numMergeCand is increased by 1.

  - When availableFlagA0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to A0 and the variable numMergeCand is increased by 1.

  - When availableFlagB2 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to B2 and the variable numMergeCand is increased by 1.

  - When availableFlagVSP is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to VSP and the variable numMergeCand is increased by 1.

  - When availableFlagCol is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to Col and the variable numMergeCand is increased by 1.

Section 8.5.2.1.2. Derivation process for spatial merging candidates

The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xN, yN ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableN.

–	If one or more of the following conditions are true with X being replaced by 0 and 1, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN is set equal to −1 and predFlagLXN is set equal to 0.

- (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xN >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2)).

- N is equal to B2 and availableFlagInterView+ availableFlagA0+ availableFlagA1+ availableFlagB0+ availableFlagB1 is equal to (4+multi_view_mv_pred_flag).

- N is equal to VSP and availableFlagInterView+ availableFlagA0+ availableFlagA1+ availableFlagB0+ availableFlagB1+ availableFlagB2 is smaller than (4+multi_view_mv_pred_flag).

- availableN is equal to FALSE

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and partIdx is equal to 1 and N is equal to B1

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and partIdx is equal to 1 and N is equal to A1

- N is equal to B1 and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to B1 and luma location ( xN, yN ) has the motion vectors (0,0) and the reference index corresponding to a VSP picture, and previousVSPFlag is equal to 1..

- N is equal to B0 and the prediction units covering luma location ( xB1, yB1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to B0 and luma location ( xN, yN ) has the motion vectors (0,0) and the reference index corresponding to a VSP picture, and previousVSPFlag is equal to 1..

- N is equal to A0 and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to A0 and luma location ( xN, yN ) has the motion vectors (0,0) and the reference index corresponding to a VSP picture, , and previousVSPFlag is equal to 1..

- N is equal to B2 and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to B2 and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to B2 and luma location ( xN, yN ) has the motion vectors (0,0) and the reference index corresponding to a VSP picture, and previousVSPFlag is equal to 1.

- N is equal to VSP and previousVSPFlag is equal to 1

–	Otherwise, availableFlagN is set equal to 1 and the following assignments are made.

- mvLXN = MvLX[ xN ][ yN ]		(8-1)
refIdxLXN = RefIdxLX[ xN][ yN ]		(8-2)
predFlagLXN = PredFlagLX[ xN ][ yN ]		(8-3)

- when luma location ( xN, yN ) has the motion vectors (0,0) and the reference index corresponding to a VSP picture, previousVSPFlag is set equal to 1.

# Appendix B: Modified AMVP list construction process

Below is the modified AMVP list construction process when no motion vector is transmitted if the reference picture is a synthetic picture. The original text is copied from WD and the modifications are yellow highlighted.

Section 8.5.2.1.6. Derivation process for motion vector predictor candidates

When the reference index refIdxLX (with X being equal to 0 or 1) corresponds to a VSP picture, the following AMVP construction is skipped, and set reconstructed mvLX equal to (0,0); otherwise, the following AMVP is invoked.

The reference view identifier refViewIdx is set equal to 0.

The motion vector predictor mvpLX is derived as specified by the following ordered steps:

- The variable maxNumMVPCand is set equal to 2 + multi_view_mv_pred_flag.

- The derivation process for motion vector predictor candidates from neighbouring prediction unit partitions as specified in subclause 8.5.2.1.6 is invoked with the luma location ( xC, yC ), the width and the height of the luma prediction block nPbW and nPbH, the reference index refIdxLX (with X being 0 or 1, respectively), and the partition index partIdx as the inputs and the outputs are the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A and B.

- Depending on multi_view_mv_pred_flag, the following applies:

–	If multi_view_mv_pred_flag is equal to 0, the variable availableFlagLXInterView is set equal to 0.

–	Otherwise (multi_view_mv_pred_flag is equal to 1), the derivation process for the inter-view motion vector predictor candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), the variables nPbW and nPbH, the reference index refIdxLX and the reference view identifier refViewIdx as the inputs and the outputs are the flag availableFlagLXInterView and the motion vector candidate mvLXInterView.

- The variable numAvail is set equal to availableFlagLXA + availableFlagLXB + availableFlagLXInterView.

- Depending on the value of numAvail and the motion vectors mvLXA  and mvLXB, the following applies:

–	If numAvail is equal to maxNumMVPCand and any two of available mvLXA, mvLXB and mvLXInterView, are different, availableFlagLXCol is set equal to 0.

–	Otherwise (numAvail is less than maxNumMVPCand or mvLXA is equal to mvLXB), the derivation process for temporal luma motion vector prediction as specified in subclause 8.5.2.1.7 is invoked with the luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and the reference index refIdxLX (with X being 0 or 1, respectively) as the inputs and the outputs are the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The motion vector predictor candidate list, mvpListLX, is constructed as specified by the following ordered steps:

  - The variable numMVPCandLX is set equal to 0. When availableFlagLXA + availableFlagLXB + availableFlagLXInterView is larger than 1.

        - When both availableFlagLXA and availableFlagLXB are both equal to 1 and mvLXA is equal to mvLXB, availableFlagLXB is set equal to 0.

        - When both availableFlagLXB and availableFlagLXInterView are both equal to 1 and mvLXInterView is equal to mvLXB, availableFlagLXInterView is set equal to 0.

        - When both availableFlagLXA and availableFlagLXInterView are both equal to 1 and mvLXInterView is equal to mvLXA, availableFlagLXInterView is set equal to 0.

  - When availableFlagLXA is equal to 1, mvpListLX[ numMVPCandLX ] is set equal to mvLXA and numMVPCandLX is incremented by 1.

  - When availableFlagLXB is equal to 1, mvpListLX[ numMVPCandLX ] is set equal to mvLXB and numMVPCandLX is incremented by 1.

  - When availableFlagLXInterView is equal to 1, mvpListLX[ numMVPCandLX ] is set equal to mvLXInterView and numMVPCandLX is incremented by 1.

  - When availableFlagLXCol is equal to 1, mvpListLX[ numMVPCandLX ] is set equal to mvLXCol and numMVPCandLX is incremented by 1.

- The motion vector predictor mvpaLX is derived as follows

    - If mvp_lX_flag[ xP, yP ] is less than numMVPCandLX, the motion vector predictor mvpLX is set equal to mvpListLX[ mvp_lX_idx[ xP, yP ] ]

    - Otherwise (mvp_lX_idx[ xP, yP ] is greater than or equal to numMVPCandLX), both components of mvpLX are set equal to 0.

- The inter-view predicted motion vector flag IvpMvFlagLX[ xC, yC ] is set equal to 0.