| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0103r1 |
| --- | --- |

| Title: | CE1.h related: Block-based VSP using synthesized depth map |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu*, Shiori Sugimoto, Hideaki Kimata (NTT) | Tel: Email: | *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

# Abstract

One of the major issues on view synthesis prediction (VSP) is quite high decoder complexity. This contribution proposes a block-based VSP scheme in order to reduce the decoder runtime. In the proposed scheme, virtual depth map on coding target view is firstly synthesized from reconstructed depth map on reference view, and then prediction image for VSP is generated only for the pixels where VSP is used. Relative to the forward-warping VSP, 28% decoding time reduction is reported without negligible loss of coding performance on average.

# Introduction

View synthesis prediction (VSP) has a potential to reduce texture bitrate significantly. However, the decoder complexity is one of major issues. At the 1st JCT-3V meeting, a block-based VSP using a backward warping was proposed for 3D-ATM [1], and shows the significant reduction of the decoder runtime. In the current CTC for 3D-HTM, the same technique cannot be applied since 3D-HTM adopts texture-first coding order. In this contribution, we propose a block-based VSP using synthesized depth map.

# Block-based VSP using synthesized depth map

The proposed scheme is also based on two-step backward (or reverse) warping.

The first step is a forward warping of depth map. On this step, virtual depth map corresponds to the target picture is generated from already decoded depth map on reference picture. Since actual spatial resolution of depth map is not so high, the forward warping is performed at integer-pixel precision although depth precision is kept. Different from the conventional depth warping, we propose to perform hole-filling with considering picture quality on the final synthesized texture. The proposed hole-filling for depth warping is described in 2.1. The generated virtual depth map is utilized only for block-based VSP on texture coding. The virtual depth map for VSP on depth map coding is generated in the conventional method. Note that most of the processes are the same in both cases, so that such processes can be shared in order to minimize overall computational cost.

The second step is generating VSP picture by a backward warping which utilizes the virtual depth map generated at the first step. As proposed in [1], VSP picture is generated only for the blocks where VSP is used.

## Proposed hole-filling for virtual depth map

In the normal hole-hilling process, the value corresponds to the background is filled to disocclusion pixels as in Figure 1.

Figure 1: Conventional hole-filling

This operation might be correct for the generation of high quality virtual depth map. However, this type of depth map offers low quality synthesized view if pixel correspondences are identified from coding target view to reference view. Figure 2 shows the obtained pixel-correspondence with virtual depth map in Figure 1: some filled pixels are corresponds to the foreground pixels in reference view although such pixels have depth value for background.

Figure 2: Pixel correspondence by the conventional virtual depth map

It is possible to avoid such mismatch by checking the depth consistency. However, it requires additional cost and other method to decide pixel value if depth values are inconsistent.

Therefore, we propose to grow foreground pixel in one pixel to the dis-occlusion region, and then to interpolate remaining holes. Figure 3 shows the proposed hole-filling for virtual depth map.

Figure 3: Proposed hole-filling for virtual depth map

The obtained virtual depth map is not correct in terms of depth map quality. However, this gives better synthesized picture by a backward warping without checking depth consistency. Figure 4 shows the synthesized view with the proposed virtual depth map. As it can be seen, the hole-filling is correctly operated by a simple backward warping.

Figure 4: Synthesized view with the proposed virtual depth map

# Experiments

The proposed methods are built on the top of [2], which based on 3DV-HTM v4.0.1. The experiments are first conducted according to the common test condition [3].

The test cases are summarized in TABLE 1. The experimental results are summarized in TABLE 2. The details are available in the attached EXCEL files. TABLE 3 shows the performance of the proposed block-based VSP relative to the frame-based VSP proposed in [2].

TABLE 1: Test cases summary

|  | VSP including VSP skip | VSP used. | MV info for VSP | Block-based VSP |
| --- | --- | --- | --- | --- |
| Anchor | N/A | N/A | N/A | N/A |
| Case I | X | VSP for all pic. | No motion info |  |
| Case II | X | VSP for all pic. | No motion info | X |
| Case III | X | VSP for ref. pic. | No motion info |  |
| Case IV | X | VSP for ref. pic. | No motion info | X |

TABLE 2: Experimental Results

- Case I (Frame-based VSP for all pic., same as case b in [2])

- Case II (Block-based VSP for all pic.)

- Case III (Frame-based VSP only for ref. pic., same as case a in [2])

- Case IV (Block-based VSP only for ref. pic.)

TABLE 3: Comparison between the frame-based and the block-based VSP

- Case I vs. Case II (VSP for all pic.)

- Case III vs. Case IV (VSP only for ref. pic.)

The results show that there is no loss on the overall coding performance for coded & synthesized. Moreover, the proposed block-based VSP scheme performs slightly better in term of texture coding. About 28% and 20% decoder runtime reduction are also achieved on average when VSP is used for all pictures and reference pictures, respectively. Note that, in the worst case scenario, where VSP is used for all blocks, the decoder complexity is increased since virtual depth map is additionally generated.

# Conclusions

In this document, a block-based VSP scheme using synthesized virtual depth map is proposed. Decoder runtime can be reduced significantly with no coding performance loss. We therefore recommend adopt the proposed technique into the 3DV-HTM software, and further study the block-based VSP in the context of CE.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Wenyi Su, Dmytro Rusanovskyy, and Miska M. Hannuksela, " 3DV-CE1.a: Block-based View Synthesis Prediction for 3DV-ATM", JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT2-A0107, July 2012, Stockholm, Sweden.

- Fen Zou, Shinya Shimizu, Dong Tian, Shiori Sugimoto, Anthony Verto, and Hideaki Kimata, "3D- CE1.h: Results on View Synthesis Prediction ", JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-B0102, October 2012, Shanghai, China.

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT2-A1100, July 2012, Stockholm, Sweden.