| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0105 |
| --- | --- |

| Title: | 3D-HEVC: Reduced resolution update mode for depth map coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shiori Sugimoto, Shinya Shimizu, Hideaki Kimata | Tel: Email: | +81-46-859-2866 sugimoto.shiori@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a consideration of Reduced Resolution Update (RRU) techniques as methods for 3D video compression. The RRU coding mode is done by encoding residuals at reduced resolution, while performing prediction using full resolution references, which allows also the final image to be reconstructed at full resolution. Such a reduced resolution residuals need to be up-sampled before used for the reconstruction of the final image. Our proposal is an enhancement of up-sampling and reconstruction performance based on the high correlation between the texture and the depth map signal. Here we investigate their use in depth map coding. In this proposal, a primitive coding design using weighted mode filtering with its own prediction information and corresponding texture signal is reported. BDR gains of 0.4% on the average and 0.8% on the maximum are observed for synthesized only and coded + synthesized cases respectively under common test conditions.

# Introduction

The Reduced Resolution Update mode already exists within H.263 as a mechanism that can enable an increase in coding picture rate while maintaining sufficient subjective quality. This is achieved by encoding prediction residuals at reduced resolution, which was achieved by performing prediction using full resolution references. In an essence at the encoder, the residuals need to be down-sampled, while transform and quantization of the residuals is performed at a lower resolution. For image reconstruction at a full resolution, the residuals need to be up-sampled after inverse quantization and inverse transform. The up-sampler needs to be fully specified at both encoder and decoder. It is considered a normative part of the implementation. Ordinary RRU techniques for 2D Video are achieved by basic linear up-sampling.

This contribution proposes an advancement of RRU by exploiting the high correlation between the texture and the depth map signal. Here we investigate their use in depth map coding. The proposed RRU for depth map is achieved by non-linear up-sampling directed by the corresponding texture signal. Reduced resolution depth coding which be utilized by 3DV-ATM, is achieved by down-sampling as pre-processing and up-sampling as post-processing. This contribution proposes a depth map coding for 3DV-HTM using RRU instead of these processing.

# Reduced Resolution Update Mode for Depth map in 3DV-HTM

RRU for 3DV-HTM is achieved by extending RRU to CU level with would allow prediction to be performed in full resolution whereas residual is performed in a lower resolution. The overview of proposed coder is shown in Figure 1 and Figure 2

Figure 1. Encoder for depth map

Figure 2. Decoder for depth map

.

This contribution also proposes a weighted mode filter as a non-linear up-sampler. It computes the localized histogram of depth map residual based on prediction error similarity. In our current design, the similarity of prediction error between two pixels is modeled by the similarity of texture and the similarity of the predictive depth map. Each similarity measure computed using a predictive depth map which need to be encoded and a decoded corresponding texture.

This filter computes R(p), which is up-sampled residual value at each position p in range of the current unit, using down-sampled residual r(q), at each position q in sampling range N(p) on the reduced resolution signal, based on following arithmetic:

R(p)=argmaxdH(p,d)

H(p,d)=q∈N(p)w(p,q)Grange(d−r(q))

w(p,q)=Gtex(tex(p)−tex(q))Gpred(pred(p)−pred(q))Glength(p−q)

Where Grange, Gtex, Gpred and Glength are Gaussian kernels directed by a parameter σrange, σtex, σpred and σlength.

They means following :

Grange : a spreading function

Gtex : the similarity of texture

Gpred : the similarity of the predictive depth map

Glength : the geometric closeness between two pixels

tex : the decoded texture corresponding to current CU

pred : the predictive depth map in current CU

The down-sampling method is in fact a non-normative tool, so it could be designed adaptively with applications. Our current implementation is designed for minimization of total sampling error; therefore down-sampling is achieved by to take the mean value in each RRU-unit which is subdivided sub-units of CU or PU into (RRUwidth x RRUheight) size. RRUwidth and RRUheight defined by rate of reductions, e.g. in the case of half for both dimensions, RRUwidth = RRUheight = 2. And the corresponding up-sampling method needs an error diffusion process shown in Figure 3 after filtering. It keeps average residual value in a unit.

Figure 3. Error diffusion in the case of RRUwidth = RRUheight = 2

Specifically, it is necessary to introduce additional syntax elements within either unit that signal that RRU shall be used. It impacts the TU that can be used given the required down-sampling of residual. Prediction could be done while retaining the supported prediction modes in 3DV-HTM.

# Experimental Results

Table 2 and Table 3 report the experimental results of the proposed method under common test conditions and fixed parameters shown in Table 1, in HTM4.0.1.

| Parameters | Value |
| --- | --- |
| Rate of reduction | 1/2 for both dimensions |
| Tap length | 6 |
| σrange | 7.0 |
| σtex | 6.0 |
| σpred | 6.0 |
| σlength | 6.0 |

Table 1. Fixed parameters

|  | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- |
| Balloons | -0.6% | -0.6% | 102.8% | 123.4% | 99.2% |
| Kendo | -0.7% | -0.7% | 102.8% | 121.9% | 102.4% |
| Newspaper | -0.8% | -0.8% | 102.6% | 118.3% | 120.0% |
| GT_Fly | -0.3% | -0.3% | 101.7% | 123.5% | 102.7% |
| Poznan_Hall2 | 0.0% | -0.1% | 101.7% | 123.0% | 100.9% |
| Poznan_Street | -0.3% | -0.3% | 102.4% | 118.3% | 120.1% |
| Undo_Dancer | 0.1% | 0.0% | 102.0% | 123.7% | 98.3% |
| 1024x768 | -0.7% | -0.7% | 102.7% | 121.2% | 106.8% |
| 1920x1088 | -0.1% | -0.2% | 102.0% | 122.1% | 105.2% |
| average | -0.4% | -0.4% | 102.3% | 121.7% | 105.9% |

Table 2. Experimental results – 3 view test case

|  | synthesized only | coded & synthesized |
| --- | --- | --- |
| Balloons | -0.6% | -0.6% |
| Kendo | -0.8% | -0.8% |
| Newspaper | -0.8% | -1.0% |
| GT_Fly | -0.1% | -0.1% |
| Poznan_Hall2 | -0.2% | -0.2% |
| Poznan_Street | -0.3% | -0.4% |
| Undo_Dancer | 0.1% | 0.0% |
| 1024x768 | -0.8% | -0.8% |
| 1920x1088 | -0.1% | -0.2% |
| average | -0.4% | -0.4% |

Table 3. Experimental results – 2 view test case

# Conclusions

The contribution proposed a consideration of Reduced Resolution Update techniques as methods for 3D video compression and an advancement of RRU by exploiting the high correlation between the texture and the depth map signal. Our preliminary test indicates that for reduction of depth map residual, our advanced RRU design seems to provide an advantage against the anchor. We recommend adopting either of them to the current HTM.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

[1] ISO/IEC JTC1/SC29/WG11, “3D-HEVC Test Model 1”, Doc. JCT3V-A1005, Stockholm, SE, July 2012.

[2] ISO/IEC JTC1/SC29/WG11, “Common Test Conditions of 3DV Core Experiments”, Doc. JCT3V-A1100, Stockholm, SE, July 2012.

[3] A.M. Tourapis, J. Boyce, “Reduced Resolution Update Mode for Advanced Video Coding”, ITU-T Q6/SG16, document VCEG-V05, Munich, March 2004