| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 October 2012 | Document: JCT3V-B0111 |
| --- | --- |

| Title: | 3D-CE5.h: Decoupling inter-view candidate for AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshiya Yamamoto Tomohiro Ikai Tadashi Uchiumi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.yoshiya@sharp.co.jp ikai.tomohiro@sharp.co.jp uchiumi.tadashi@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- At the last meeting, a simplified motion vector candidate derivation for AMVP (JCT3V-A0014) has been proposed. The method removes the dependency between the 3D extension part (inter-view candidate) and HEVC part (regular spatial and temporal candidates) to minimize the extension part. It removes the case that the inter-view candidate is not available and removes the pruning process between the inter-view and regular ones. This decoupling is also beneficial for decoder complexity reduction because it can avoid the unnecessary candidate derivation process. It is reported that average BD-rate of the proposal is not changed from that of the anchor (HTM4.0.1).

# Background

- In 3D-HEVC, an extended candidate, named inter-view candidate, is additionally used for non-base view picture decoding. The extended candidate doesn’t exist in HEVC. In the current HTM, the derivation of the inter-view candidate is always needed due to having pruning process between inter-view candidate and other candidates and the case that the inter-view candidate may be unavailable. The proposal addresses this problem and achieves decoupling the inter-view candidate and regular candidate for AMVP.

# Proposed Method

To simplify the motion vector candidate derivation, we propose the following changes.

|  | HTM4.0.1 | Proposal |
| --- | --- | --- |
| Change1 | When the inter-view candidate is not available, it is not added to the candidate list. | When the inter-view candidate is not available, Zero vector is added to the candidate list. |
| Change2 | Pruning is executed between Inter-view candidate and spatial candidates. | Pruning is not executed between Inter-view candidate and spatial candidates. |
| Change3 | The inter-view candidate is added at the third order in the candidate list. The candidate order: Spatial Left Spatial Top Inter-view Temporal | The inter-view candidate is added at the first order in the candidate list. The candidate order: Inter-view Spatial Left Spatial Top Temporal |

# Performance Discussion

The proposed method is implemented on HTM4.0.1. The result is shown in Table 1.

Table 1: Experiment results (proposal)

Additionally we tested the case the inter-view candidate is used as the first position instead of the third position for HTM4.0.1 (i.e. only Change 3 is tested). The result is shown in Table 2.

Table 2: Experiment results (only the inter-view candidate position is changed)

# Conclusions

To minimize the extension part from HEVC version 1, the proposing scheme which decouples the inter-view candidate and regular HEVC candidates has been proposed. In this contribution, it is reported that the BD-rate on common test condition is not changed compared to the anchor HTM4.0.1. It is recommended to adopt the proposed scheme in the next HTM.

# References

- Y. Yamamoto, T. Ikai, T. Uchiumi, “3D-CE5.h related: Simplification of AMVP”, JCT3V-A0014, JCT2 1st meeting, Stockholm, SE, 16-20 July, 2012.

- “Common Test Conditions for 3DV experimentation”, JCT3V-A1100, JCT2 1st meeting, Stockholm, SE, 16-20 July, 2012.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).