| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0112r1 |
| --- | --- |

| Title: | 3D-CE5.h related: Disparity vector restrictions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Tadashi Uchiumi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution presents disparity vector restrictions for HTM4.0.1. In practical usage, base view and other views need to be decoded simultaneously. In the current scheme, the other view depends on the data of base view and the reference area of base view (the range of disparity vector) is not restricted.  In this contribution, the restriction of disparity vectors range is proposed. The proposal consists of two parts.  The first one clips the disparity vector to restrict the reference image area. The second one clips to the internal disparity vector to restrict the area of base layer’s motion information, at inter-view vector candidate derivation process. It is reported both restrictions doesn’t affect RD-performance.  And the second part is similar to the restriction proposed in JCT3V-B0087.

# Introduction

In practical usage, base view and other views need to be decoded in 16 ms (e.g 60 fps case). Normally  one decoder is designed to meet only one picture decoding in 16 ms, it is difficult that this designed decoder meets two view decoding in time. Therefore the simultaneous decode with two decoders is needed in that case as shown Figure 1.

In the current scheme, the simultaneous decoding is impossible because the other view depends on the data of base view and the reference area of base view, in other words the range of disparity vector is not restricted.

Figure 1: Practical case

In our proposals, we restrict the reference area of base-view to enable N x CTB lines latency decoding. We think 1 CTB line latency (N = 1) is desirable. Therefore we propose the restriction of the disparity vector.

- In HTM decoder, the disparity vector is used in the two retrieval process of base view information. The first one is to get base-layer reference image and the second one is to get the associated base-layer motion vectors. These are illustrated in Figure 2.

- Figure 2. Reference area with disparity vector

# Discussion on Disparity area range

For parallel decoding

To enable simulateously decoding, we needs to restrict the range of vertical range within CTB size – 4 (DF+SAO area) – 4 (8 tap filtering). Therefore if the case CTB size is 64, the dvRangeY shall be less than 56 (= 64 - 8). The available area and non-available area is shown in Figure 1.

- Therefore we set the restriction range to 56 pixels, which is reasonable in regards to our observations described below.

- dvRangeY = 56

- Figure 2. Available area in the parallel decoding case

- As for horizontal range, we set restriction range to 1/8 or 12.5 % of resolution, which is reasonable in regards to observations and concept.

- dvRangeX = pic_width_in_luma_samples / 8

Observation and requirement

- We’ve studied the range of disparity vector in both experimental and conceptual.

- Experimentally, the distribution of motion vectors and disparity vectors of the common condition sequence ( on average ) was measured as Figure 1.  The vertical range of disparity-vector is very narrow, which means 90 % of disparity vectors are within 0 to 1 (4 in quarter pel). Horizontal range of disparity vector is wide but almost of all vectors are within 128 (512 in quarter pel)

- Figure 2. Distribution of motion vector and disparity vector
(common test condition)

- Conceptually the disparity vector range depends on interocular distance, depth and resolution and in the past MPEG contribution [1], we have proposed that the range of disparity vector should cover 10 % of resolution.

# Proposal

We proposed two restrictions: (1) base-view image retrieval restriction and (2) base-view motion vector retrieval restriction. Figure 3 shows the concept of the proposed restrictions.

- Figure 4. Concept of proposed restrictions

(1) Image retrieval restriction

Both x and y component of disparity vector are clipped as below,

disparity_vector_X = clip3 (-dvRangeX, dvRangeX-1, disparity vector X)

disparity_vector_Y = clip3 (-dvRangeY, dvRangeY-1, disparity vector Y)

where dvRangeX and dvRangeY are the range of restriction.

(2) Motion vector retrieval restriction

Only x component is clipped. y component of disparity vector is set to zero, which means we don’t need to use y component of disparity vector.

disparity_vector_X = clip3 (-dvRangeX, dvRangeX-1, disparity vector X)

disparity_vector_Y = 0

where dvRangeX is the range of restriction

# Text modification

See the attached revised text.

-
					-
					-
					-

-
	-

# Experimental results

The experimental results of our proposal are shown in Table 1.

Table 1: Both restrictions (proposal 1+2)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 98.1% | 100.2% | 100.0% |
| Kendo | 0.0% | -0.3% | 0.2% | 0.0% | 0.0% | 0.0% | 96.9% | 101.3% | 100.5% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 96.3% | 100.8% | 100.3% |
| GhostTownFly | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 98.6% | 101.3% | 101.8% |
| PoznanHall2 | 0.0% | 0.4% | 0.5% | 0.2% | 0.2% | 0.2% | 98.2% | 101.5% | 101.9% |
| PoznanStreet | 0.0% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 98.4% | 101.0% | 99.8% |
| UndoDancer | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 99.1% | 102.2% | 100.2% |
| 1024x768 | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 97.1% | 100.8% | 100.3% |
| 1920x1088 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 98.6% | 101.5% | 100.9% |
| average | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 97.9% | 101.2% | 100.6% |

Additionally we conducted the image retrieval restriction only and the motion vector retrieval restriction only as shown in Table 2 and Table 3.

Table 2: Image retrieval restriction (proposal 1)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 97.8% |  | 100.4% |
| Kendo | 0.0% | -0.2% | 0.0% | 0.0% | 0.1% | 0.0% | 97.1% |  | 101.2% |
| Newspapercc | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | 0.0% | 96.0% |  | 99.9% |
| GhostTownFly | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 98.4% |  | 102.6% |
| PoznanHall2 | 0.0% | 0.5% | 0.2% | 0.1% | 0.2% | 0.1% | 98.4% |  | 100.9% |
| PoznanStreet | 0.0% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 98.3% |  | 99.3% |
| UndoDancer | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 99.7% |  | 102.0% |
| 1024x768 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 97.0% |  | 100.5% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 98.7% |  | 101.2% |
| average | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 98.0% |  | 100.9% |

Table 3: Motion vector retrieval restriction (proposal 2)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 102.3% | 100.1% |
| Kendo | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 99.9% | 100.5% | 100.6% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 101.6% | 99.8% |
| GhostTownFly | 0.0% | 0.2% | -0.2% | 0.0% | 0.0% | 0.0% | 100.2% | 97.1% | 100.1% |
| PoznanHall2 | 0.0% | 0.5% | 0.6% | 0.2% | 0.2% | 0.2% | 101.6% | 101.4% | 101.7% |
| PoznanStreet | 0.0% | -0.5% | 0.0% | -0.1% | -0.1% | -0.1% | 100.4% | 98.2% | 99.6% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.7% | 98.7% | 101.4% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.0% | 101.5% | 100.2% |
| 1920x1088 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.7% | 98.8% | 100.7% |
| average | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.4% | 100.0% | 100.5% |

# Conclusion

This contribution presents disparity vector restrictions for HTM4.0.1. Considering the practical usage where base view and other views are required to decode simultaneously,  we restrict vertical range of disparity vector within 56. The horizontal range of disparity vector is also restricted within 1/8 of image width. In this contribution, the restriction of disparity vectors range is proposed. The proposal consists of two restrictions.  The first one clips the disparity vector to restrict the reference image area. The second one clips to the internal disparity vector to restrict the area of base layer’s motion information, at inter-view vector candidate derivation process. It is reported both restrictions doesn’t affect RD-performance.

- It is recommended to adopt the two restrictions in HTM5.

# References

[1] T. Uchiumi, S. Sato, K. Iwauchi, S. Tanaka, K. Tokui, Y. Wakabayashi, M. Ohtsu, J. Sato, Y. Yamamoto, “Proposals on Applications and Requirements on 3D Video Coding”, ISO/IEC JTC1/SC29/WG11 N19355, Daegu, KR, Jan. 2011.

[2] Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei, 3D-CE5.h related: Constrained DV for inter-view data access, JCT3V-B0087, JCT3V 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).