| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0113 |
| --- | --- |

| Title: | 3D-CE5.h related: Restricted motion vector coding for inter-view prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tadashi Uchiumi Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 uchiumi.tadashi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution presents a motion vector coding tool for inter-view prediction for HEVC-3V, which is related to CE 5. The proposal introduces a restriction on inter-view vectors and discards the vertical motion vector difference (mvd) if the restriction is applied, considering sequences which are produced with 1-D camera arrangement and the vertical disparity is always zero. The simulation result reports this restriction decrease encoding complexity and there are coding gains about 0.4 % compared to encoder only restriction in sequences generated with CG.

# Introduction

One of applications for 3D Video Coding is stereoscopic displays. For that purpose, many of the stereo video sequences are considered to be produced with 1-D camera arrangement. At those sequences, there must be no vertical disparity between each view, which means the vertical component of the motion vector difference is always zero. Therefore the sending of the difference is redundant, if the sequence has been produced with accurate 1-D arrangement.

# Proposed Method

This proposal enables the codec to restrict the vertical component of motion vector to zero if the vector is associated with inter-view prediction, say disparity vector. This method is switchable by a flag in SPS named disp_mvdy_zero_flag.  If disp_mvdy_zero_flag is enabled, the vertical component of motion vector difference is omitted and infered to be zero.

(1) Motion vector coding and syntax change (normative)

The modifications are highlighted with yellow.

(2-1) mvd_coding( )

The vertical component of motion vector difference abs_mvd_greater0_flag[ 1 ] is omitted when motion vector is used for inter-view prediction (inter_view_pred is true) and a new SPS flag disp_mvdy_zero_flag is 1.

| mvd_coding( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| if(!inter_view_pred ¦¦ !disp_mvdy_zero_flag) |  |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) { |  |
| if(abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if(abs_mvd_greater0_flag[ 1 ] ) { |  |
| if(abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

abs_mvd_greater0_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 0. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1. When abs_mvd_greater0_flag is not present, it is inferred to be equal to 0.

The variable inter_view_pred , which specifies whether the prediction unit associated with the motion vector difference is used as inter-view prediction, is derived as follows When the ViewId of the RefPicListLX[ refIdxLX ] is not equal to ViewId, inter_view_pred is set to be 1, otherwise inter_view_pred is set to be 0.

(2-2) disp_mvdy_zero_flag

| seq_parameter_set_extension_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| base_view_flag | u(1) |
| if (!base_view_flag){ |  |
| disp_mvdy_zero_flag | u(1) |
| depth_flag | u(1) |
| … |  |
| } |  |
| … |  |
| } |  |

disp_mvdy_zero_flag equals to 1 specifies that the vertical component of motion vector associated with inter-view prediction shall be zero and its difference is not present. disp_mvdy_zero_flag equals to 0 specifies that that vertical components of motion vector differences is present. When disp_mvdy_zero_flag is not present, it is inferred to be equal to 0.

(2) ME restriction at encoder (non-normative)

When 1-D camera arrangement is used, the encoder sends disp_mvdy_zero_flag as 1 and restricts vertical motion search range for inter-view prediction to zero (Figure 1). As a result, the vertical component of disparity vector (mvY) and its difference (mvdY) become zero.

Reference Picture

(eg. view 0)

Disparity vector

mv = (x,0)

Search range

(vertical direction = 0)

Coding Picture

(eg. view 1)

Figure 1: Restriction on disparity vector search

# Simulation Results

The experimental results are shown in Table 1 and Table 2. Table 1 shows the results of proposed method, while Table 2 shows the encoder only results. The both of software were implemented and evaluated on HTM-4.0.1. The Simulation was performed based on the common test conditions [1].

As shown in Table 1, there are coding gains for dependent views in the sequences of Computer Graphics images (i.e. GhostTownFly and UndoDancer) which were produced with accurate 1-D camera arrangement.

Compared to encoder only case, the proposed motion coding scheme achieves 0.4% coding gain on average, that is, the loss of encoder only case is 2.4 % while the loss of the proposal is 2.0 %. At the GhostTownFly and UndoDancer, the proposal restricts disparity vector with no loss.

The proposed restriction decrease encoding time by 10 % due to search range restriction for inter-view prediction. The decoding and rendering times are almost the same as the anchor.

Table 1: Experimental results (proposal)

| Sequences | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 4.4% | 2.1% | 1.5% | 1.0% | 1.1% | 90.7% | 94.5% | 100.4% |
| Kendo | 0.0% | 4.0% | 1.9% | 1.4% | 0.9% | 1.0% | 89.1% | 94.9% | 99.4% |
| Newspapercc | 0.0% | 3.6% | 10.0% | 3.0% | 1.9% | 2.1% | 87.3% | 93.7% | 99.7% |
| GhostTownFly | 0.0% | -0.9% | -0.5% | 0.0% | 0.0% | 0.0% | 91.6% | 93.6% | 100.6% |
| PoznanHall2 | 0.0% | 9.9% | 7.0% | 4.1% | 2.9% | 3.2% | 88.7% | 95.6% | 101.0% |
| PoznanStreet | 0.0% | 21.2% | 26.1% | 8.2% | 6.0% | 6.5% | 89.5% | 93.1% | 99.6% |
| UndoDancer | 0.0% | -0.5% | -0.6% | -0.1% | 0.2% | 0.1% | 91.9% | 94.2% | 100.7% |
| 1024x768 | 0.0% | 4.0% | 4.6% | 1.9% | 1.2% | 1.4% | 89.0% | 94.4% | 99.8% |
| 1920x1088 | 0.0% | 7.4% | 8.0% | 3.0% | 2.3% | 2.4% | 90.4% | 94.1% | 100.5% |
| average | 0.0% | 6.0% | 6.6% | 2.6% | 1.8% | 2.0% | 89.8% | 94.2% | 100.2% |

Table 2: Experimental results (encoder restriction only)

| Sequences | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 5.2% | 2.7% | 1.8% | 1.2% | 1.4% | 90.7% | 97.2% | 99.7% |
| Kendo | 0.0% | 4.8% | 2.8% | 1.7% | 1.2% | 1.3% | 88.9% | 96.4% | 100.6% |
| Newspapercc | 0.0% | 4.8% | 10.9% | 3.4% | 2.3% | 2.5% | 87.1% | 100.1% | 99.2% |
| GhostTownFly | 0.0% | 0.7% | 0.8% | 0.4% | 0.4% | 0.4% | 90.4% | 97.8% | 102.4% |
| PoznanHall2 | 0.0% | 11.4% | 8.2% | 4.7% | 3.4% | 3.7% | 88.7% | 95.7% | 99.0% |
| PoznanStreet | 0.0% | 22.8% | 28.0% | 8.8% | 6.5% | 7.0% | 90.2% | 95.7% | 97.6% |
| UndoDancer | 0.0% | 0.7% | 0.7% | 0.4% | 0.6% | 0.5% | 91.3% | 96.9% | 101.3% |
| 1024x768 | 0.0% | 4.9% | 5.5% | 2.3% | 1.6% | 1.7% | 88.9% | 97.9% | 99.8% |
| 1920x1088 | 0.0% | 8.9% | 9.4% | 3.6% | 2.7% | 2.9% | 90.1% | 96.5% | 100.1% |
| average | 0.0% | 7.2% | 7.7% | 3.0% | 2.2% | 2.4% | 89.6% | 97.1% | 100.0% |

# Conclusion

- This contribution presents a motion vector coding tool for inter-view prediction for HEVC-3V. The proposal introduces a restriction on inter-view vectors and discards the vertical motion vector difference (mvd) if the restriction is applied. In a practical encoding case, it is considered to be appropriate to restrict a vertical component of disparity vector to be zero when the source video has been generated with accurate 1-D camera arrangement. This restriction decrease encoding time by 10 % at HTM4.0.1 simulation. Compared to the encoder only restriction, the proposed motion coding method achieves 0.4 % gain by omitting the vertical vector difference for disparity vectors. It is recommended to adopt the restriction into new HTM.

# References

[1] Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT2-A1100, Stockholm, SE, 16–20 July 2012.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).