| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0115 |
| --- | --- |

| Title: | 3D-CE5.h related cross-check of results on disparity vector derivation from blocks coded by inter-view motion parameter prediction (JCT3V-B0157) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Tadashi Uchiumi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check result of DV-MCP improvement proposal (JCT3V-B0157). The experiment result perfectly matches with those provided by the proponent. The software are also studied and confirmed that it follows the description. The average BD-rate (gain) is 0.1 % in total (coded & synthesized). The BD-rate of video only and synthesized only is also 0.1 %. Enc time, Dec time, and Ren time is almost the same as the anchor (HTM4.0.1).

# Introduction

The proposal is about improved DV-MCP, which was adopted at the last meeting (JCT2-A0126). Usage of the current DV-MCP is restricted to the only skip mode. In JCT3V-B0157, applied the concept of DV-MCP into the inter modes.

The proposal consists of two changes.

|  | HTM4.0.1 | Test (JCT3V-B0157) |
| --- | --- | --- |
| Change1 | use only SKIP neighboring blocks at DV-MCP vector derivation | use Inter neighboring blocks at DV-MCP vector derivation |
| Change2 | the order of DV-MCP neighboring blocks is the same as AMVP candidate’ order LEFTBELOW LEFT ABOVERIGHT ABOVE ABOVELEFT COL | the order of DV-MCP neighboring blocks is the same as merge candidate’ order except that the collocated block is used as the first one. COL LEFT ABOVE ABOVERIGHT LEFTBELOW ABOVELEFT |

Change 1:

// TComDataCU.cpp

#if SAIT_IMPROV_DVMCP

bTmpIsInter   = !pcTmpCU->isIntra( uiIdx );

#else

bTmpIsSkipped = pcTmpCU->isSkipped( uiIdx );

#endif

Change 2:

// TypeDef.h

#define LGE_DVMCP                         1   //  JCT2-A0126

#if LGE_DVMCP

#define SAIT_IMPROV_DVMCP                 1

#if SAIT_IMPROV_DVMCP

#define DVFROM_COL                        1

#define DVFROM_LEFT                       2

#define DVFROM_ABOVE                      3

#define DVFROM_ABOVERIGHT                 4

#define DVFROM_LEFTBELOW                  5

#define DVFROM_ABOVELEFT                  6

#else

#define DVFROM_LEFTBELOW                  1

#define DVFROM_LEFT                       2

#define DVFROM_ABOVERIGHT                 3

#define DVFROM_ABOVE                      4

#define DVFROM_ABOVELEFT                  5

#define DVFROM_COL                        6

#endif

#endif

# Experimental results

The experiment by cross-checker result is shown in Table 1.

Table 1: Experiment result

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.4% | -0.7% | -0.2% | -0.2% | -0.2% | 99.4% | 100.1% | 100.0% |
| Kendo | 0.0% | -0.4% | -0.3% | -0.2% | -0.1% | -0.1% | 99.7% | 100.2% | 100.6% |
| Newspapercc | 0.0% | -0.6% | -0.4% | -0.2% | -0.1% | -0.1% | 99.3% | 99.8% | 99.8% |
| GhostTownFly | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 99.6% | 100.3% |
| PoznanHall2 | 0.0% | 0.1% | -0.7% | -0.2% | -0.1% | -0.1% | 100.7% | 100.5% | 102.0% |
| PoznanStreet | 0.0% | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 99.4% | 99.5% | 101.1% |
| UndoDancer | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 100.5% | 101.7% |
| 1024x768 | 0.0% | -0.4% | -0.5% | -0.2% | -0.1% | -0.1% | 99.5% | 100.0% | 100.1% |
| 1920x1088 | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | 100.1% | 100.1% | 101.3% |
| average | 0.0% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | 99.8% | 100.0% | 100.8% |

# Conclusion

This contribution reports a cross-check result of DV-MCP improvement proposal (JCT3V-Bxxxx). The experiment result perfectly matches with those provided by the proponent. The coding gain is 0.1 % in total (coded & synthesis). The derivation order is the same as the merge mode derivation order except that the collocated block is used as the first one.

# References

[1] J. Young Lee, J. Lee, D.-S. Park, J. Sung, M. Koo, S. Yea, “3D-CE5.h related results on disparity vector derivation from blocks coded by inter-view motion parameter prediction, “JCT3V-B0157, JCT3V  2nd Meeting: Shanghai, CN, 13–19 Oct. 2012

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).