| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0117 |
| --- | --- |

| Title: | 3D-CE5.h related: cross-check of Unified temporal collocated picture for DV derivation (JCT3V-B0088) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Tadashi Uchiumi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reports a cross-check result of the explicit signaling of a colPic for disparity vector derivation (JCT3V-B0088). The scheme also includes remove the restriction of SKIP block in the DV-MCP derivation process. The experiment results perfectly match with those provided by the proponent. The provided text has been also reviewed and confirmed that the text is appropriate and the software and the text match except for a small discrepancy which can be easily fixed.  The proposal prevents arbitrary memory access at PU level and is beneficial for memory access reduction. No coding loss was observed on average.

# Verification

// Sheme 1

|  | HTM4.0.1 | Test (JCT3V-B0088) |
| --- | --- | --- |
| Change1 (main) | may use the different colPics at each PU for disparity vector derivation colPic is decided on PU basis by - refIdx (AMVP) - left block’s reference (merge) | send explicit colPic for disparity vector using dv_collocated_from_l0_flag dv_collocated_ref_idx |
| Change2 (additional) | use only SKIP neighboring blocks at DV-MCP vector derivation | use Inter neighboring blocks at DV-MCP vector derivation |

At the encoder, the number of DV blocks is counted and colPic is decided based on maximum count picture in the reference picture list.

Note:

There are a minimum difference between the provided software and the draft, which should be fixed at integration phase if adopted. Specifically, in the released software, dv_collocated_from_l0_flag and dv_collocated_ref_idx is coded before collocated_from_l0_flag and collocated_ref_idx, while in the draft those are interleaved in collocated_from_l0_flag and collocated_ref_idx.

# Experimental results

The experiment by cross-checker results are shown in Table 1.

Table 1: Experiment result (scheme 1)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | -0.3% | -0.1% | 0.0% | 0.0% | 99.4% | 100.7% | 100.8% |
| Kendo | 0.0% | 0.3% | 0.6% | 0.2% | 0.1% | 0.1% | 99.5% | 100.3% | 100.4% |
| Newspapercc | 0.0% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 98.9% | 100.6% | 100.4% |
| GhostTownFly | 0.0% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% | 100.3% | 99.2% | 102.4% |
| PoznanHall2 | 0.0% | 0.8% | 0.0% | 0.1% | 0.1% | 0.1% | 101.1% | 103.6% | 102.7% |
| PoznanStreet | 0.0% | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 99.2% | 98.4% | 100.7% |
| UndoDancer | 0.0% | 0.2% | 0.3% | 0.1% | 0.0% | 0.0% | 99.8% | 99.3% | 101.8% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.3% | 100.6% | 100.5% |
| 1920x1088 | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 100.1% | 100.1% | 101.9% |
| average | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 100.3% | 101.3% |

# Conclusion

This contribution reports a cross-check result of the explicit signaling of colPic for disparity vector derivation (JCT3V-B0088). The experiment results perfectly match with those provided by the proponent. The proposal prevents arbitrary memory access at PU level and is beneficial for memory access reduction. No coding loss was observed on average.

# References

[1] Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei, N. Zhang, J. An, K. Zhang, S. Ma, D. Zhao, W. Gao, “

3D-CE5.h related: Unified temporal collocated picture for DV derivation, “ JCT3V-B0088, JCT3V, 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).