| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0118 |
| --- | --- |

| Title: | 3D-CE5.h related: cross-check of MV memory reduction on motion/mode parameter prediction (JCT3V-B0038) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tadashi Uchiumi Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 uchiumi.tadashi@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check result of MV memory reduction on motion/mode parameter prediction (JCT3V-B0038). The experiment results perfectly match with those provided by the proponent.

# Introduction

The proposal is for MV memory reduction on motion/mode parameter prediction in 3D-HTM. Two approaches with different compression ratios of MVs are proposed. Both of them have been tested and each of the results is compared to that of the anchor HTM4.0.1.

# Experimental results

The experimental result on Approach 1 is shown in Table 1, and that of Approach 2 is shown in Table 2.

Table 1: Experiment result on Approach 1

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 2.0% | 2.0% | 0.8% | 0.9% | 0.9% | 100.2% | 93.8% | 100.3% |
| Kendo | 0.0% | 1.4% | 1.9% | 0.7% | 0.8% | 0.8% | 99.9% | 92.4% | 98.9% |
| Newspapercc | 0.0% | 1.3% | 1.2% | 0.5% | 0.7% | 0.7% | 100.2% | 92.4% | 99.7% |
| GhostTownFly | 0.0% | 1.3% | 1.3% | 0.3% | 0.4% | 0.4% | 100.4% | 91.1% | 102.8% |
| PoznanHall2 | 0.0% | 0.7% | 0.6% | 0.3% | 0.2% | 0.2% | 101.1% | 97.1% | 102.4% |
| PoznanStreet | 0.0% | 0.4% | 0.6% | 0.2% | 0.4% | 0.4% | 99.2% | 90.4% | 99.4% |
| UndoDancer | 0.0% | 1.8% | 2.2% | 0.6% | 1.0% | 1.0% | 99.0% | 91.4% | 101.2% |
| 1024x768 | 0.0% | 1.6% | 1.7% | 0.7% | 0.8% | 0.8% | 100.1% | 92.9% | 99.6% |
| 1920x1088 | 0.0% | 1.1% | 1.2% | 0.4% | 0.5% | 0.5% | 99.9% | 92.5% | 101.5% |
| average | 0.0% | 1.3% | 1.4% | 0.5% | 0.6% | 0.6% | 100.0% | 92.6% | 100.7% |

Table 2: Experiment result on Approach 2

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.0% | 102.3% | 99.4% |
| Kendo | 0.0% | 0.0% | 0.2% | 0.0% | 0.2% | 0.2% | 100.4% | 101.5% | 99.8% |
| Newspapercc | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 99.9% | 101.2% | 100.5% |
| GhostTownFly | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% | 100.8% | 99.4% | 100.6% |
| PoznanHall2 | 0.0% | 0.1% | 0.8% | 0.2% | 0.1% | 0.1% | 101.9% | 103.5% | 101.7% |
| PoznanStreet | 0.0% | -0.2% | 0.3% | 0.0% | 0.1% | 0.1% | 100.8% | 101.4% | 100.7% |
| UndoDancer | 0.0% | 0.1% | 0.3% | 0.1% | 0.1% | 0.1% | 101.0% | 100.9% | 101.4% |
| 1024x768 | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 100.1% | 101.7% | 99.9% |
| 1920x1088 | 0.0% | 0.0% | 0.4% | 0.1% | 0.1% | 0.1% | 101.1% | 101.3% | 101.1% |
| average | 0.0% | 0.0% | 0.3% | 0.1% | 0.1% | 0.1% | 100.7% | 101.4% | 100.6% |

# Conclusion

This contribution reports a cross-check result of MV memory reduction on motion/mode parameter prediction (JCT3V-B0038). The experiment results perfectly match with those provided by the proponent.

# References

[1] Y. Takahashi, S. Hattori, O. Nakagami, T. Suzuki, “3D-CE5.h related: MV memory reduction on motion/mode parameter prediction”, JCT3V-B0038, Shanghai, CN, 13–19 Oct. 2012.