| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0119 |
| --- | --- |

| Title: | MV-HEVC: cross-check of Vertical length restriction of inter-view vector for HEVC simple 3D extension (JCT3V-B0037) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tadashi Uchiumi Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 uchiumi.tadashi@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check result of Vertical length restriction of inter-view vector for HEVC simple 3D extension (JCT3V-B0037). The experiment results perfectly match with those provided by the proponent.

# Introduction

The proposal is for restriction of vertical length of inter-view vector for HEVC simple 3D extension. Two test sets with different restriction are proposed. Both of them have been tested and each of the results is compared to that of the anchor HTM4.0.1.

# Experimental results

The experimental result with vertical motion search range of inter-view vector is set to 8 is shown in Table 1, and that with 16 as the search range is shown in Table 2.

Table 1: Experiment result 1 (search range = 8)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 95.1% | 96.2% | 100.6% |
| Kendo | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 93.3% | 95.1% | 101.1% |
| Newspapercc | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 92.2% | 95.0% | 100.0% |
| GhostTownFly | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 96.3% | 95.0% | 102.4% |
| PoznanHall2 | 0.0% | 0.9% | 0.8% | 0.4% | 0.3% | 0.3% | 94.0% | 95.5% | 102.0% |
| PoznanStreet | 0.0% | -0.3% | 0.1% | 0.0% | 0.0% | 0.0% | 96.1% | 93.5% | 100.8% |
| UndoDancer | 0.0% | 0.3% | 0.2% | 0.1% | 0.2% | 0.2% | 97.2% | 96.7% | 102.3% |
| 1024x768 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 93.5% | 95.4% | 100.6% |
| 1920x1088 | 0.0% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% | 95.9% | 95.2% | 101.9% |
| average | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 94.9% | 95.3% | 101.3% |

Table 2: Experiment result 2 (search range = 16)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 95.6% | 94.6% | 100.0% |
| Kendo | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 93.8% | 96.2% | 100.5% |
| Newspapercc | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 92.9% | 95.0% | 100.0% |
| GhostTownFly | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 96.3% | 94.9% | 102.3% |
| PoznanHall2 | 0.0% | 0.7% | 0.1% | 0.2% | 0.2% | 0.2% | 94.9% | 96.4% | 102.2% |
| PoznanStreet | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 96.4% | 94.5% | 101.7% |
| UndoDancer | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 97.1% | 93.3% | 100.2% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 94.1% | 95.3% | 100.1% |
| 1920x1088 | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 96.2% | 94.8% | 101.6% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 95.3% | 95.0% | 101.0% |

# Comment

The cross-checker found that there is a case that inter-view vector is over 8 of its value of the vertical component in the Experiment 1 (search range is set to 8). Actually a value of 8.75 [pixel] was observed in the test. All the values of the vertical components of inter-view vector by the proposal must be in range of 8 [pixel] according to the proponent’s intention. Therefore it seems that the provided source code may have any mistakes or misunderstanding against its intention of the proposal.

# Conclusion

This contribution reports a cross-check result of Vertical length restriction of inter-view vector for HEVC simple 3D extension (JCT3V-B0037). The experiment results perfectly match with those provided by the proponent,

# References

[1] Y. Takahashi, S. Hattori, O. Nakagami, T. Suzuki, “MV-HEVC: Vertical length restriction of inter-view vector for HEVC simple 3D extension”, JCT3V-B0037, Shanghai, CN, 13–19 Oct. 2012.