| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0120 |
| --- | --- |

| Title: | 3D-CE5.h related: cross-check of support of parallel merge in disparity vector derivation (JCT3V-B0136) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Tadashi Uchiumi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reports a cross-check result of the parallel merge support in disparity vector derivation (JCT3V-B0136). The experiment results perfectly match with those provided by the proponent. The software was reviewed and confirmed that parallel merge for 3V extension in disparity vector is appropriately implemented. The cross-checker also confirmed the existing PARALLEL_MERGE scheme of HTM4.0.1 (HM6.1), which is enabled in the test, is the same as DIS HEVC (HM8).

Compared to non parallel merge anchor, overall coding loss of the proposal (parallel merge is employed at base view and dependent views) was observed by 0.1 % at the case LOG2_PARALLEL_MERGE_LEVEL_MINUS2 is 1, which means that merge candidates in PUs of 8x8 CU can be derived parallel. It is recommended to include this feature in 3D-HEVC. The appropriate text of disparity vector derivation process might be needed.

# Verification

|  | HTM4.0.1 | Test (JCT3V-B0136) |
| --- | --- | --- |
| Change1 | not support of parallel merge in disparity vector | introduced to support with a macro LGE_IVMP_PARALLEL_MERGE |

#if LGE_DVMCP

#if LGE_IVMP_PARALLEL_MERGE

getDisMvpCand2ParMerg(uiPUIdx, uiAbsPartIdx, &cDisInfo, true );

#else

getDisMvpCand2(uiPUIdx, uiAbsPartIdx, &cDisInfo, true );

#endif

#if LGE_IVMP_PARALLEL_MERGE

if (pcTmpCU)

{

if (!pcTmpCU->isDiffMER(xP+nPSW-1, yP-1, xP, yP))

{

pcTmpCU = NULL;

}

}

#endif

Note:

In the provided software getDisMvpCand2 is replaced with a new function getDisMvpCand2ParMerg but the two functions are so similar that we should keep getDisMvpCand2.

# Experimental results

The experiment by cross-checker results are shown in Table 1.

Table 1: Experiment result

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 100.4% | 101.2% | 100.1% |
| Kendo | 0.1% | 0.0% | 0.2% | 0.1% | 0.2% | 0.2% | 100.2% | 101.0% | 100.5% |
| Newspapercc | 0.0% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 100.2% | 98.8% | 99.7% |
| GhostTownFly | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% | 100.7% | 100.2% | 102.2% |
| PoznanHall2 | 0.0% | 0.3% | 0.2% | 0.1% | 0.1% | 0.1% | 101.8% | 101.7% | 102.3% |
| PoznanStreet | -0.1% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% | 100.8% | 99.5% | 101.5% |
| UndoDancer | 0.1% | 0.2% | 0.3% | 0.2% | 0.2% | 0.2% | 101.5% | 98.9% | 102.4% |
| 1024x768 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 100.2% | 100.3% | 100.1% |
| 1920x1088 | 0.0% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 101.2% | 100.1% | 102.1% |

# Conclusion

The parallel merge support in disparity vector derivation (JCT3V-B0136) was cross-checked. The experiment results perfectly match with those provided by the proponent. Overall coding loss was observed by 0.1 % with LOG2_PARALLEL_MERGE_LEVEL_MINUS2 is 1, which means that merge candidates in PUs of 8x8 CU can be derived parallely. It is recommended to include this feature in 3D HEVC. The appropriate text of disparity vector derivation process might be needed.

# References

[1] M. Koo, J. Sung, S. Yea, “3D-CE5.h Related: Support of parallel merge in disparity vector derivation, “ JCT3V-B0136, JCT3V, 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).