| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0121 |
| --- | --- |

| Title: | CE1.a related results on forward warping block-based VSP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yichen Zhang, Yin Zhao, Lu Yu Zhejiang University | Tel: Email: | +86-571-87953107, felixzyc@gmail.com zhaoyin@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This contribution proposes a low-complexity block-based VSP (BBVSP) algorithm for texture coding which is based on forward warping, namely Forward warping Block-based VSP (FBVSP). Compared with the backward warping block-based VSP algorithms in ATM5, the FBVSP only use the texture and depth data of an already coded view (called a reference view), and is independent of the depth map of the current coding view (called a target view). For each macroblock using VSP in the target view (called a target block), the synthesis samples are obtained by forward warping of pixels in a proper window in the reference view. The window, i.e., a reference block, is decided by a three-step method. It is reported that over 75% macroblocks are synthesized from a reference block which is equal to or narrower than 17 pixels in width. The RD performance and complexity of the FBVSP is competitive with the BBVSP in ATM5, while the FBVSP also supports the flexible coding order of the non-base view (i.e., both texture-first or depth-first coding order) due to the independency of the non-base view depth.

# Introduction

In JCT3V-A0107 [1], Nokia proposed a block-based VSP (BBVSP) which is based on backward warping method as shown in Figure 1. The disparity of each pixel in Cb is calculated from its corresponding depth value in target view. This technique has been adopted and be a part of current anchor ATM5.1r2.

Figure 1. Block-based VSP based on backward warping in JCT3V-A0107

The BBVSP is dependent on depth map of the target view. Under current common test conditions, the coding order of ATM is “T0D0D1D2T1T2” (i.e. depth-first coding order). In other words, depth map of each side view (i.e. target view) is decoded prior to texture picture. With this coding order, depth map of target view is available for VSP module. However, if the texture of the target view is coded prior to the depth map, using the texture-first coding order of “T0D0T1D1T2D2”, the VSP module is not applicable in this case, since the depth map of the target view is not available.

This contribution proposes a block-based VSP (BBVSP) algorithm for texture coding which is based on forward warping, namely Forward warping Block-based VSP (FBVSP). The FBVSP only use the texture and depth data of an already coded view, and is independent of the depth map of the current coding view, and thus it supports both the texture-first and depth-first coding order of the non-base view.

Besides, the synthesized picture by the BBVSP in ATM5 present strong synthesis distortions at one side of object boundaries, as shown in Figure 2. The artefacts are owing to the fact that the BBVSP use foreground pixels to fill the disocclusion region, which is also a problem identified in [2]. The FBVSP uses directional hole filling to complete the disocclusion region with background color, and has better image quality. However, in current test conditions, VSP modes are generally less efficient than other modes (e.g., disparity compensated prediction) at object boundaries. This is because the synthesized image quality greatly depends on the corresponding reconstructed depth data, but unfortunately depth around object boundaries usually presents low accuracy owing to distortions from heavy lossy coding and erroneous depth values. For high-quality depth data with less quantization, the synthesized samples by forward warping are more likely to win in mode decision than those by backward warping.

(a) synthesis picture with backward warping		(b) synthesis picture with forward warping

Figure 2. VSP picture synthesized at encoder by (a) backward warping and (b) forward warping. Strong artefacts appear at the dis-occlusion side (left side) of objects when using backward warping, while the synthesis picture by forward warping does not have this problem.

# Algorithm Description

The synthesis of a target block is based on two parts: deciding the reference block and warping the reference block.

First, the reference block of a proper size is determined by the method described below:

Figure 3. Example of the largest reference block (in which the reference view is at the right side of the target view)

The most straightforward method to determine a reference block in the reference view is subtracting MaxDisparity and MinDisparity from the left and right boundaries of the target block, respectively, as shown in Figure 3:

MaxLeftBoundaryref = LeftBoundarytar – MaxDisparity

MaxRightBoundaryref = RightBoundarytar – MinDisparity

where MaxDisparity and MinDisparity are the (signed) maximum and minimum disparity values between the reference view and the target view (corresponding to depth values 255 and 0), respectively. Theoretically, pixels outside of the reference block cannot be warped into the target block. In other words, such a reference block contains all necessary pixels to synthesize the target block, and the block-based synthesis samples are identical to those by frame-based synthesis. Such a reference block is called a Theoretical Necessary Window (TNW).

However, there are often many redundant pixels in a TNW which will not be warp to a position within the target block, since the depth values in the target block may not range from 0 to 255. Therefore, such pixels do not contribute to the synthesis results in the target block. To improve the efficiency of the warping, the proposed FBVSP employs a three-step technique to exclude as many pixels that are probably not warped to the target block as possible (i.e., to reduce the reference block from the TNW to a proper smaller size).

Four corner pixels (top-left, top-right, bottom-left, bottom-right) of the target block are utilized to refine the left and right boundaries of the reference block. For simplicity, we have the following notations.

the top-left corner –> refLeft1

the bottom-left corner –> refLeft2

the top-right corner –> refRight1

the bottom-right corner –> refRight2

Figure 4. Illustration of reference block boundary derivation process

The three-step reference block derivation method is described as follows:

Step 1: Initialization of refLeft1, refLeft2, refRight1, refRight2 to be the TNW:

refLeft1 = refLeft2 = MaxLeftBoundaryref;

refRight1 = refRight2 = MaxRightboundaryref.

Step 2: Boundary refinement:

For every corner pixel, four boundary candidates, Cand0~Cand3, are derived in the following order. Here, we mainly take refLeft1 as an example. The process of refLeft2 is identical, and the process of the two right corners is similar to that of the left corners, and the slight difference is explained in italic.

i) Cand0 = (MaxLeftBoundaryref, Yp) (or Cand0 = (MaxRightBoundaryref, Yp), for the right corners), and Cand0 is marked as “available”; where the corner pixel p is denoted as (Xp, Yp).

ii) The depth of p is assumed to be 128 (no matter whether the depth is available), then the corresponding position of p in the reference view Cr1 (Xcr1, Ycr1) is determined, and Cr1 is considered as Cand1, i.e.,

Cand1 = Cr1 = (Xcr1, Ycr1) = (Xp – D128, Yp), as illustrated as ① in Figure 4(a).

If the warping position of pixel Cr1 in the target view, namely Ct1 (Xct1, Yct1) (② in Figure 4(a)), is on the left (or on the right, for the right corners) of the target block or on the left edge (or right edge, for the right corners) of the target block, Cand1 is marked as “available”; otherwise, Cand1 is marked as “unavailable”.

More explanation: assume that depth values of several horizontally adjacent pixels are similar/identical, the pixels around Cand1 will be warped to adjacent positions in the target view. In the “available” case, some pixels at the right side of Cand1 can still be warped to positions outside of the target block (i.e., warped into the gap between Ct1 and p). Thus, the reference block is available to be reduced further. In the “unavailable” case, some pixels at the left side of Cand1 will be warped into the target block; in other words, if the left boundary of the reference block is determined by Cand1, the reference block excludes some useful reference pixels which are warped to the target block, and the window should be enlarged to adopt the useful pixels.

iii) The distance between Ct1 and p is derived, denoted as Dist (i.e., ③ in Figure 4(a)). A new candidate Cand2 is determined at Cr2 which is shifted by Dist from Cr1 (i.e., ④ in Figure 4(b)):

Dist = Xp – Xct1;

Cand2 = Cr2 = (Xcr2, Ycr2) = (Xcr1 + Dist, Yp) = (Xcr1 + Xp – Xct1, Yp).

(or Cand2 = Cr2 = (Xcr2, Ycr2) = (Xcr1– Dist, Yp) = (Xcr1– Xp + Xct1, Yp), for the right corners).

Note: if Xp < Xct1 in the unavailable case, the Dist value is negative, then Cand2 is on the left of Cand 1; such as the case in Figure 4(c).

Similar to ii), if the warping position of pixel Cr2 in the target view, namely Ct2 (Xct2, Yct2) is on the left (or on the right, for the right corners) of the target block or on the left edge (or right edge, for the right corners) of target block, Cand2 is marked as “available”; otherwise, Cand2 is marked as “unavailable”;

iv) Repeat iii): the distance between Ct2 and p is derived, denoted as Dist’ (i.e., ③ in Figure 4(b)). A new candidate Cand3 is determined at Cr3 which is shifted by Dist from Cr2 (i.e., ④ in Figure 4(c)):

Dist’ = Xp – Xct2;

Cand3 = Cr3 = (Xcr3, Ycr3) = (Xcr2 + Dist’, Yp) = (Xcr1 + Xp – Xct1 + Xp – Xct2 , Yp).

(or Cand3 = Cr3 = (Xcr3, Ycr3) = (Xcr2– Dist’, Yp) = (Xcr2– Xp + Xct1 – Xp + Xct2, Yp), for the right corners).

Similar to ii), if the warping position of pixel Cr3 in the target view, namely Ct3 (Xct3, Yct3) is on the left (or on the right, for the right corners) of the target block or on the left edge (or right edge, for the right corners) of target block, Cand3 is marked as “available”; otherwise, Cand3 is marked as “unavailable”;

v) the best corner position in the reference view for p (i.e., refLeft1 for the top-left corner) is set to be one among the available candidates which has the maximum (or minimum, for the right corners) x coordinate (i.e., the right-most available candidate).

Step 3: Final determination of the reference block:

The left and right boundaries of the reference block are:

LeftBoundaryref = Minimum (refLeft1, refLeft2)

RightBoundaryref = Maximum (refRight1, refRight2)

After the reference block is determined, the reference block is warped to target view by the forward warping method in JCT3V-A0023 (synthesis without z-buffering) [3] and M24796 [4] (synthesis with fractional disparity) which are adopted in previous meetings.

Similar to [1], motion vectors for VSP blocks are set as zero, and in the current simulation the target block is always a macroblock that uses synthesis samples as reference pixels.

# Experimental Results

The proposed algorithm is implemented on ATM5.1r2, and all the simulations are done under the common test conditions [5]. Since VSP is only used in EHP conditions, HP conditions are not tested.

Simulation 1: ATM5.1r2 in depth-first coding order (FBVSP is turned on) vs.  ATM5.1r2 in depth-first coding order (Anchor).

Table 1. Coding performance of simulation 1

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity Estimate |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % |
| S01 | -0.44 | 0.01 | 0.00 | 0.00 | -0.39 | 0.01 | -0.32 | 0.01 | 99.88 | 101.60 |
| S02 | -0.23 | 0.01 | 0.00 | 0.00 | -0.21 | 0.01 | -0.06 | 0.00 | 100.64 | 102.45 |
| S03 | -0.33 | 0.01 | 0.00 | 0.00 | -0.31 | 0.01 | -0.05 | 0.00 | 87.26 | 104.27 |
| S04 | -0.12 | 0.00 | 0.00 | 0.00 | -0.11 | 0.00 | 0.03 | 0.00 | 85.73 | 103.52 |
| S05 | 0.40 | -0.02 | 0.00 | 0.00 | 0.30 | -0.01 | 0.42 | -0.02 | 111.76 | 101.39 |
| S06 | 0.55 | -0.03 | 0.00 | 0.00 | 0.44 | -0.02 | 0.80 | -0.03 | 118.37 | 103.51 |
| S08 | -0.10 | 0.00 | 0.00 | 0.00 | -0.10 | 0.00 | 0.02 | 0.00 | 107.68 | 98.30 |
| Average | -0.04 | 0.00 | 0.00 | 0.00 | -0.05 | 0.00 | 0.12 | -0.01 | 101.62 | 102.15 |

Simulation 2: ATM5.1r2 in texture-first coding order (FBVSP is turned on) vs. ATM5.1r2 in texture-first coding order (VSP is turned off).

Table 2. Coding performance of simulation 2

|  | Texture Coding |  | Depth Coding |  | Depth Coding |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -15.90 | 0.53 | 1.09 | -0.06 | -14.70 | 0.50 | -14.21 | 0.47 |
| S02 | -7.78 | 0.24 | 4.86 | -0.21 | -6.86 | 0.21 | -5.95 | 0.19 |
| S03 | -12.50 | 0.47 | 7.74 | -0.61 | -11.41 | 0.44 | -9.46 | 0.32 |
| S04 | -16.69 | 0.67 | 8.03 | -0.44 | -15.66 | 0.63 | -13.33 | 0.49 |
| S05 | -7.80 | 0.37 | 1.66 | -0.08 | -5.93 | 0.27 | -7.39 | 0.31 |
| S06 | -9.90 | 0.50 | 2.69 | -0.12 | -8.65 | 0.43 | -9.06 | 0.41 |
| S08 | -3.65 | 0.15 | 1.66 | -0.07 | -2.76 | 0.11 | -2.81 | 0.10 |
| Average | -10.60 | 0.42 | 3.96 | -0.23 | -9.42 | 0.37 | -8.89 | 0.33 |

For the common test conditions (simulation 1), FBVSP achieves similar coding performance to the anchor and little increase in encoding and decoding time.

With the texture-first coding order, the simulation results of ATM5.1 with the proposed FBVSP shows 10.60% BD-rate saving against ATM5.1 (with VSP turned off, simulation 4), which demonstrates the great benefits of VSP in the texture-first coding order.

It is observed that the texture-first coding order is less efficient than depth-first coding order in current ATM5, as shown in Table III, due to switching off some depth-aided texture coding tools like VSP, DMVP and A0045 (motion vector competition-based Skip/Direct mode with explicit signalling) [6] in the texture-first coding order. However, when the FBVSP is turned on, the loss of texture-first coding order is reduced to 11% and 8% in terms of coded texture and synthesis view, respectively, as shown in Table 4.

Simulation 3: ATM5.1r2 in texture-first coding order (VSP is turned off) vs. ATM5.1r2 in depth-first coding order (Anchor).

Table 3. Coding performance of simulation 3

|  | Texture Coding |  | Depth Coding |  | Depth Coding |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 37.43 | -1.01 | -2.78 | 0.18 | 33.81 | -0.94 | 29.32 | -0.82 |
| S02 | 12.48 | -0.34 | -6.01 | 0.28 | 10.83 | -0.30 | 9.03 | -0.26 |
| S03 | 21.77 | -0.68 | -6.02 | 0.50 | 19.95 | -0.65 | 15.07 | -0.44 |
| S04 | 27.94 | -0.90 | -12.43 | 0.75 | 25.67 | -0.84 | 20.30 | -0.64 |
| S05 | 25.71 | -1.09 | -2.76 | 0.13 | 20.78 | -0.86 | 19.61 | -0.74 |
| S06 | 32.58 | -1.41 | -6.94 | 0.33 | 27.99 | -1.20 | 25.11 | -1.01 |
| S08 | 14.36 | -0.56 | -3.05 | 0.13 | 11.43 | -0.45 | 10.32 | -0.35 |
| Average | 24.61 | -0.86 | -5.71 | 0.33 | 21.50 | -0.75 | 18.40 | -0.61 |

Simulation 4: ATM5.1r2 in texture-first coding order (FBVSP is turned on) vs. ATM5.1r2 in depth-first coding order (Anchor).

Table 4. Coding performance of simulation 4

|  | Texture Coding |  | Depth Coding |  | Depth Coding |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 16.27 | -0.48 | -1.72 | 0.12 | 14.78 | -0.45 | 11.24 | -0.35 |
| S02 | 3.81 | -0.11 | -1.45 | 0.07 | 3.31 | -0.10 | 2.59 | -0.08 |
| S03 | 6.67 | -0.22 | 1.25 | -0.10 | 6.37 | -0.21 | 4.21 | -0.13 |
| S04 | 6.80 | -0.23 | -5.40 | 0.31 | 6.19 | -0.22 | 4.37 | -0.14 |
| S05 | 16.29 | -0.72 | -1.15 | 0.06 | 13.95 | -0.60 | 10.89 | -0.44 |
| S06 | 19.91 | -0.88 | -4.43 | 0.21 | 17.33 | -0.76 | 13.96 | -0.58 |
| S08 | 10.28 | -0.41 | -1.44 | 0.06 | 8.44 | -0.33 | 7.25 | -0.25 |
| Average | 11.43 | -0.44 | -2.05 | 0.10 | 10.05 | -0.38 | 7.79 | -0.28 |

# Conclusion

The forward warping block-based VSP (FBVSP) method achieves competitive RD performance and computational complexity with the backward warping block-based VSP in ATM5. FBVSP does not rely on the availability of the dependent-view depth map, thus supporting both texture-first and depth-first coding order for the dependent view, and achieves around 10% BD-rate saving in the texture-first coding order (in which the BBVSP cannot be applied).

We recommend adopting the FBVSP method.

# References

[1] Wenyi Su, Dmytro Rusanovskyy and Miska M. Hannuksela, “3DV-CE1.a: Block-based View Synthesis Prediction for 3DV-ATM”, JCT-3V document, JCT3V-A0107, Stockholm, Sweden, July 2012.

[2] K. N. Iyer, K. Maiti, B. Navathe, H. Kannan, and A. Sharma, "Multiview video coding using depth based 3D warping", in Proc. IEEE Int. Conf. Multimedia & Expo (ICME), 2010, pp. 1108-1113.

[3] Shinya Shimizu, Shiori Sugimoto and Hideaki Kimata, “CE1.a related results on the modification of in-loop view synthesis”, JCT-3V document, JCT3V-A0023, Stockhom, Sweden, July 2012.

[4] Shinya Shimizu and Hideaki Kimata, “3D-CE1.a related: Simplification of in-loop view synthesis with fractional-pel disparity”, ISO/IEC JTC1/SC29/WG11/ M24796, Geneva, Switzerland, April 2012.

[5] Dmytro Rusanovskyy, Karsten Müller and Anthony Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT-3V document, JCT3V-A1100, Stockholm, Sweden, July 2012.

[6] Jian-Liang Lin, Yi-Wen Chen, Yu-Lin Chang, Yu-Pao Tsai, Yu-Wen Huang, and Shawmin Lei, “3D-CE5.a results on the motion vector competition-based Skip/Direct mode with explicit signaling”, JCT-3V document, JCT3V-A0045, Stockholm, Sweden, July 2012.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).