| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 15–19 Oct 2012 | Document: JCT3V-B0126 |
| --- | --- |

| Title: | CE6.h: Results on edge intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo, Eunyong Son, Sehoon Yea, | Email: | jin78.heo@lge.com, eunyong.son@lge.com, sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents an adaptive context modeling method for region boundary chain coding described in JCT3V-A0070. Under common test conditions this method provides 0.041% BD-rate gain and 0.013% BD-rate gain under all-intra configuration.

# Introduction

In the last meeting, the region boudnary chain coding described in JCT3V-A0070 [1] was adopted and it was integrated into the HTM-4.0.1. The region boundary chain coding partitions the block into two regions by signaling the region boundaries with chain codes. Each bin of the chain codes is encoded using a context model which has the same probability of zero and one. However, since the frequency of each chain code is different, various context models which have the different statistical distribution are designed in this proposal. Therefore, this contribution proposes an adaptive context model selection method for encoding a chain code.

# Proposed method

The traverse type which represents the edge direction is signalled with edge_code. The edge_code is constructed to have a different codeword length based on the frequency of traverse types [2]. In this contribution, the edge_code is rearranged based on the frequency of traverse types in depth map coding.

Table 1 edge_code for each traverse type

The proposed method proposes five context models (ctx) which have different kinds of statistics adaptively to encode each bin of the edge_code. Each context model is designed according to the occurrence probability between previously encoded and current traverse types and is selected based on the previously encoded traverse type. Table 2 shows the assignment of context model to bin index (binIdx).

Table 2 Context model for each binIdx

All bins of the edge_code for the first edge starting from the block boundaries are encoded with ctx2 as there is no previously encoded edge information. Moreover, the occurrence probability of four traverse types: 90, -90, 135, and -135 is quite low compared to other three traverse types: 0, 45, and -45. Therefore, all bins of them are also encoded with only one context model, ctx4. Table 3 shows the feature of each context model.

Table 3 Feature of context model

Figure 1 shows the example of the proposed method.

Figure 1 Example of edge_code coding

# Experimental Results

According to the CE6. description [3] and the common test conditions [4], the proposed method is evaluated for random access (CTC) and intra-only configuration.

Table 4 Result for 3-view scenario under CTC

Table 5 Result for 3-view scenario under intra-only configuration

# Working Draft Modifications

## General Coding Unit Syntax (G.7.4.9.1)

- G.7.4.9.1  General coding unit semantics

…

edge_code[ i ] shall be one of the values shown in Table_G-. edge_code[ i ] is used to derive the edge direction of the ith edge when edge_intra_flag[ x0 ][ y0 ] is equal to 1.

Table_G- – Interpretation of edge_code

| Bin string of edge_code[i] | edge direction |
| --- | --- |
| 0 | 0° |
| 10 | 45° |
| 110 | -45° |
| 10 | -45° |
| 110 | 45° |
| 1110 | 90° |
| 11110 | -90° |
| 111110 | 135° |
| 111111 | -135° |

# Conclusion

This contribution proposes an adaptive context modeling decision method for region boundary chain coding. The proposed method achieves 0.041% and 0.013% gains under common test conditions and all-intra configuration, respectively. It is recommended that the proposed method be adopted into 3DV-HEVC.

# Reference

- Jin Heo, Eunyong Son, Sehoon Yea, “CE6.h Simple Region-based Intra Prediction for Depth-map,” JCT3V-A0068, Stockholm, Sweden, July 2012.

- Yong Kui Liu and Borut Zalik, “An efficient chain code with Huffman coding,” Pattern Recognition, vol. 38, issue 4, pp. 553-557, April 2005.

- Dmytro Rusanovskyy, Karsten Müller, and Anthony Vetro, “Common Test Conditions for 3DV Core Experiments,” JCT3V-A1100, Stockholm, Sweden, July 2012.

- Philipp Merkle, “Description of Core Experiment 6 (CE6) on Depth Intra Coding,” JCT3V-A1106, Stockholm, July 2012.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).