| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 15–19 Oct 2012 | Document: JCT3V-B0127 |
| --- | --- |

| Title: | 3D-CE6.h: Region-based intra prediction for DMM mode 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo, Eunyong Son, Sehoon Yea, Philipp Merkle | Email: | jin78.heo@lge.com, eunyong.son@lge.com, sehoon.yea@lge.com, philipp.merkle@hhi.fraunhofer.de |
| Source: | LG Electronics, Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution is a joint proposal and presents a depth intra coding method for Region-based DMM mode 2. The proposed method substitutes the intra-predicted Wedgelet partitioning method of DMM mode 2 described in JCT3V-A1005 with the region-based intra prediction method described in JCT3V-A0068. Under common test conditions this method provides 0.1% BD-rate gain and 0.2% BD-rate gain under all-intra configuration.

# Introduction

The region-based intra prediction method proposed in JCT3V-A0068 [1] and the DMM mode 2 described in JCT3V-A1005 [2] are both based on the approach to use the information of neighboring blocks for predicting a depth block by partitioning it into two disjoint regions separated by a straight line, i.e. a Wedgelet. The main difference between the methods is that the region-based method is not limited to direction information of neighboring blocks, but builds upon the intra direction scheme for mode and most probable mode selection. Therefore, this contribution proposes a method that harmonizes DMM mode 2 with region-based intra prediction.

# Proposed method

The proposed method uses the mode signaling and the deltaDC method of DMM [2] without any changes. In contrast to the current DMM mode 2 the proposed method applies the prediction process described in in JCT3V-A0068 [1] for separating the depth block into two constant-valued regions using the edge starting point and the intra prediction mode that implies the direction of edges inside the block. Consequently, the corresponding intra direction identifier has to be stored and transmitted for the region-based intra prediction instead of the line end offset of the current DMM mode 2. For efficient processing and signaling the most probable mode approach of the regular intra modes is applied. However, the deviation process of most probable modes is specially adapted to region-based intra prediction, preferring directional modes over DC and planar mode.

# Experimental Results

According to the CE6.H description [3] and the common test conditions [4], the proposed method is evaluated for random access (CTC) and intra-only configuration.

Table 1 Result for 3-view scenario under CTC

Table 2 Result for 3-view scenario under intra-only configuration

# Working draft modifications

## General Coding unit syntax (G.7.3.9.1)

- G.7.3.9.1    General Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I && TextureModeDepth[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] = = -1) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| if( TextureModeDepth[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] = = 1 ) { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| if ( enable_DMM_flag && log2CbSize <= Log2DMMMaxSize ) { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| dmm_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if (dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| dmm_mode[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULL ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULLDELTA ) ) |  |
| wedge_full_tab_idx[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| dmm_delta_end_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_abs_minus1[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_sign_flag[ x0 ][ y0 ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_REGION ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_REGIONDELTA ) ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0 + i ] ) |  |
| dmm_dc_1_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_1_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_1_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] ) |  |
| dmm_dc_2_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_2_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_2_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| } else { |  |
| … |  |
| } |  |
| } |  |
| if ( !depth_flag && ViewId && multi_view_residual_pred_flag && ResidualCbfNonZero && PredMode != MODE_INTRA) |  |
| res_pred_flag | ae(v) |
| if ( TextureModeDepth[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] = = ctDepth && split_coding_unit_flag[ x0 ][ y0 ] ) |  |
| coding_tree( x0, y0, log2CbSize, ctDepth ) |  |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

## General coding unit semantics (G.7.4.9.1)

- G.7.4.9.1   General coding unit semantics

The specification in subclause F.7.4.9.1 apply with the following modifications and additions.

dmm_flag[ x0 ][ y0 ] equal to 0 specifies that depth map model modes are not used. dmm_flag[ x0 ][ y0 ] equal to 1 specifies that depth map model modes are used that the dmm_mode syntax element is present. If the dmm_flag[ x0 ][ y0 ] is not present in the bitstream, its value should be inferred to be equal to 0.

dmm_mode[ x0 ][ y0 ] shall be one of the values shown in Error: Reference source not found. dmm_mode is used to derive the intra prediction mode when the dmm_flag[ x0 ][ y0 ] is equal to 1.

Table G- – Interpretation of dmm_mode

| dmm_mode | Method identifier |
| --- | --- |
| 0 | MODE_DMM_WFULL |
| 1 | MODE_DMM_WFULLDELTA |
| 2 | MODE_REGION |
| 3 | MODE_REGIONDELTA |
| 2 4 | MODE_DMM_WPREDTEX |
| 3 5 | MODE_DMM_WPREDTEXDELTA |
| 4 6 | MODE_DMM_CPREDTEX |
| 5 7 | MODE_DMM_CPREDTEXDELTA |
| 6 | MODE_DMM_WPREDDIR |
| 7 | MODE_DMM_WPREDDIRDELTA |

wedge_full_tab_idx[ x0 ][ y0 ] specifies the index of the wedgelet pattern in the corresponding pattern list.

dmm_delta_end_flag[ x0 ][ y0 ] equal to 0 specifies that dmm_delta_end_abs_minus1[ x0 ][ y0 ] and dmm_delta_end_sign[ x0 ][ y0 ] syntax elements are not present. dmm_delta_end_flag equal to 1 specifies that dmm_delta_end_abs_minus1[ x0 ][ y0 ] and dmm_delta_end_sign[ x0 ][ y0 ] syntax elements are present.

dmm_delta_end_abs_minus1[ x0 ][ y0 ] and dmm_delta_end_sign_flag[ x0 ][ y0 ] are used to derive DmmDeltaEnd[ x0 ][ y0 ] as follows:

DmmDeltaEnd[ x0 ][ y0 ] = ( 1 − 2 *dmm_delta_end_sign_flag[ x0 ][ y0 ] ) *
( dmm_delta_end_abs_minus1[ x0 ][ y0 ] + 1)	(G-1)

dmm_dc_1_abs[ x0 ][ y0 ], dmm_dc_1_sign_flag[ x0 ][ y0 ], dmm_dc_2_abs[ x0 ][ y0 ], dmm_dc_2_sign_flag[ x0 ][ y0 ] are used to derive DmmQuantOffsetDC1[ x0 ][ y0 ] and DmmQuantOffsetDC2[ x0 ][ y0 ] values as follows:

DmmQuantOffsetDC1[ x0 ][ y0 ]  = ( 1 − 2 *dmm_dc_1_sign_flag[ x0 ][ y0 ] ) * dmm_dc_1_abs[ x0 ][ y0 ]	(G-2)
DmmQuantOffsetDC2[ x0 ][ y0 ]  = ( 1 − 2 *dmm_dc_2_sign_flag[ x0 ][ y0 ] ) * dmm_dc_2_abs[ x0 ][ y0 ]	(G-3)

edge_intra_flag[ x0 ][ y0 ] equal to 0 specifies that region boundary chain coding is not used. edge_intra_flag[ x0 ][ y0 ] equal to 1 specifies that region boundary chain coding is used.

…

## Derivation process for luma intra prediction mode (G.8.4.2)

- G.8.4.2   Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PbSize specifying the size of the current luma prediction block.

…

IntraPredMode[ xB ][ yB ] labelled 0..34 represents directions of predictions as illustrated in Figure 8-1.

–	If dmm_flag[ xB ][ yB ] is equal to 1 and dmm_mode[ xB ][ yB ] is not equal to 2 or 3, IntraPredMode[ xB ][ yB ] is derived as follows:

IntraPredMode[ xB ][ yB ] = dmm_mode[ xB ][ yB ] + 35	(G-4)

–	Otherwise (dmm_flag[ xB ][ yB ] is equal to 0 or dmm_mode[ xB ][ yB ] is equal to 2 or 3), IntraPredMode[ xB ][ yB ] is derived as the following ordered steps.

- The neighbouring locations ( xBA, yBA ) and ( xBB, yBB ) are set equal to ( xB−1, yB ) and ( xB, yB−1 ), respectively.

- For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

- The availability derivation process for a block in z-scan order as specified in subclause 6.4.1 is invoked with the location ( xCurr, yCurr ) set equal to ( xB, yB ) and the neighbouring location ( xN, yN ) set equal to ( xBN, yBN ) as the input and the output is assigned to availableN.

- The candidate intra prediction mode candIntraPredModeN is derived as follows.

- If availableN is equal to FALSE, candIntraPredModeN is set equal to Intra_DC.

- Otherwise, if PredMode[ xBN ][ yBN ] is not equal to MODE_INTRA, candIntraPredModeN is set equal to Intra_DC,

- Otherwise, if N is equal to B and yB−1 is less than (( yB >> Log2CtbSizeY ) << Log2CtbSizeY), intraPredModeB is set equal to Intra_DC.

- Otherwise, candIntraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ].

- The candModeList[ x ] with x=0..2 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA and dmm_flag[ xB ][ yB ] is equal to 0 the following applies:

  - If candIntraPredModeA is less than 2 (either Intra_Planar or Intra_DC), candModeList[ x ] with x=0..2 is derived as:

candModeList[0] = Intra_Planar		(G-5)
candModeList[1] = Intra_DC		(G-6)
candModeList[2] = Intra_Angular (26)		(G-7)

  - Otherwise, candModeList[ x ] with x=0..2 is derived as:

candModeList[0] = candIntraPredModeA		(G-8)
candModeList[1] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(G-9)
candModeList[2] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(G-10)

- Otherwise, if candIntraPredModeB is equal to candIntraPredModeA and dmm_flag[ xB ][ yB ] is equal to 1 the following applies:

  - If candIntraPredModeA is less than 2 (either Intra_Planar or Intra_DC), candModeList[ x ] with x=0..2 is derived as:

candModeList[0] = Intra_Angular (26)		(G-11)
candModeList[1] = Intra_DC		(G-12)
candModeList[2] = Intra_Angular (10)		(G-13)

  - Otherwise, candModeList[ x ] with x=0..2 is derived as:

candModeList[0] = candIntraPredModeA		(G-14)
candModeList[1] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(G-15)
candModeList[2] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(G-16)

- Otherwise, if candIntraPredModeB is not equal to candIntraPredModeA and dmm_flag[ xB ][ yB ] is equal to 0, the following applies:

  - candModeList[0] and candModeList[1] are derived as follows:

candModeList[0] = candIntraPredModeA		(G-17)
candModeList[1] = candIntraPredModeB		(G-18)

  - If none of candModeList[0] and candModeList[1] is equal to Intra_Planar, candModeList[2] is set equal to Intra_Planar,

  - Otherwise, if none of candModeList[0] and candModeList[1] is equal to Intra_DC, candModeList[2] is set equal to Intra_DC,

  - Otherwise, candModeList[2] is set equal to Intra_Angular (26).

- Otherwise, (candIntraPredModeB is not equal to candIntraPredModeA and dmm_flag[ xB ][ yB ] is equal to 1) the following applies:

  - candModeList[0] and candModeList[1] and candModeList[2] are derived as follows:

  - If candIntraPredModeA and candIntraPredModeB are not equal to Intra_Planar,

candModeList[0] = candIntraPredModeA

candModeList[1] = candIntraPredModeB

  - If candIntraPredModeA and candIntraPredModeB are not equal to Intra_Angular(26),

candModeList[2] = Intra_Angular(26)

  - Otherwise, if candIntraPredModeA and candIntraPredModeB are not equal to Intra_DC,

candModeList[2] = Intra_DC

  - Otherwise,

candModeList[2] = Intra_Angular(10)

  - Otherwise, if candIntraPredModeA is equal to Intra_Planar and candIntraPredModeB is  not equal to Intra_Planar,

  - If candIntraPredModeB is not equal to Intra_Angular(26),

candModeList[0] = Intra_Angular(26)

  - Otherwise,

candModeList[0] = Intra_DC

candModeList[1] = candIntraPredModeB

  - If candModeList[0] and candModeList[1] are not equal to Intra_DC,

candModeList[2] = Intra_DC

  - Otherwise,

candModeList[2] = Intra_Angular(10)

  - Otherwise,

candModeList[0] = candIntraPredModeA

  - If candIntraPredModeA is not equal to Intra_Angular(26),

candModeList[1] = Intra_Angular(26)

  - Otherwise,

candModeList[1] = Intra_DC

  - If candModeList[0] and candModeList[1] are not equal to Intra_DC,

candModeList[2] = Intra_DC

  - Otherwise,

candModeList[2] = Intra_Angular(10)

…

## Decoding process for intra blocks (G.8.4.4.2)

- G.8.4.4.2.1    General intra sample prediction

The specification in subclause 8.4.4.2.1 with the following paragraphs added to the end of subclause apply:

  - Otherwise, if intraPredMode is equal to Intra_DepthPartition(35,36), the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the sample array p and the transform block size nT as the inputs and the output are the predicted sample array predSamples.

  - Otherwise, if intraPredMode is equal to Intra_DepthPartition(37,38), the corresponding intra prediction mode specified in subclause 8.4.4.2 of JCTVC-J1003_d7 is invoked with the sample array p and the transform block size nT as the inputs and the output are the predicted sample array predSamples.

  - Otherwise, if intraPredMode is equal to Intra_DepthPartition(37,3839,40), the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the sample array p and the transform block size nT as the inputs and the output are the predicted sample array predSamples.

  - Otherwise, if intraPredMode is equal to Intra_DepthPartition(39,4041,42), the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the sample array p and the transform block size nT as the inputs and the output are the predicted sample array predSamples.

  - Otherwise, if intraPredMode is equal to Intra_DepthPartition(41,42), the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the sample array p and the transform block size nT  as the inputs and the output are the predicted sample array predSamples.

  - Otherwise, if intraPredMode is equal to Intra_DepthPartition(43,44), the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the sample array p and the transform block size nT  as the inputs and the output are the predicted sample array predSamples.

…

- G.8.4.4.2.8    Specification of Intra_DepthPartition (37, 38 39, 40) prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = −1..2*nT−1,

–	a variable nT specifying the transform size,

…

- G.8.4.4.2.9    Specification of Intra_DepthPartition (39, 40 41,42) prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = −1..2*nT−1,

–	a variable nT specifying the transform size,

…

- G.8.4.4.2.10   Specification of Intra_DepthPartition (41, 42) prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = −1..2*nT−1,

–	a variable nT specifying the transform size,

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 41 or 42.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nT−1, are derived as the following ordered steps:

- A binary partition pattern dmmWedgeletPattern[ x, y ], with x, y =0..nT−1,  is derived by the continuous intra direction wedgelet partition prediction process, using DmmDeltaEnd[ x0 ][ y0 ].

[Ed. (GT): Add description of continuous intra direction wedgelet partition prediction process]

The depth partition value derivation and assignment process as specified in subclause Error: Reference source not found is invoked with p[ x, y ], dmmWedgeletPattern[ x, y ], and nT as inputs and the output is assigned to predSamples[ x, y ].

# Conclusion

This proposal harmonizes DMM mode 2 with the region-based intra prediction method, as the two approaches are based on the same concept and have several processing steps in common. The harmonized method achieves 0.1% and 0.2% gains under common test conditions and all-intra configuration, respectively. It is recommended that the proposed method be adopted into 3DV-HEVC.

# Reference

- Jin Heo, Eunyong Son, Sehoon Yea, “CE6.h Simple Region-based Intra Prediction for Depth-map,” JCT3V-A0068, Stockholm, Sweden, July 2012.

- Gerhard Tech, Krzysztof Wegner, Ying Chen, Sehoon Yea, “3D-HEVC Test Model 1,” JCT3V-A1005, Stockholm, Sweden, July 2012.

- Dmytro Rusanovskyy, Karsten Müller, and Anthony Vetro, “Common Test Conditions for 3DV Core Experiments,” JCT3V-A1100, Stockholm, Sweden, July 2012.

- Philipp Merkle, " Description of Core Experiment 6 (CE6) on Depth Intra Coding", JCT3V-A1106, Stockholm, July 2012.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).